3GPP TSG-RAN WG4 Meeting #103-e	R4-2209070

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.7.3

Source:	Ericsson

Title:	Simulation results of PDSCH demodulation requirements for CA with HST-SFN scenario

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the CR for the demodulation requirements for HST FR1 [1], but the requirements with CBW 35/45MHz in FDD CA is still TBD. This contribution provides our simulation results for CBW 35/45MHz in FDD CA to complete the WI.

# 2	Simulation results

Table 1 and Table 2 summarize our simulation results for PDSCH HST-SFN/DPS with FDD CBW=35/45MHz.

Table 1	SNR to achieve 70% of maximum throughput for PDSCH with HST-SFN/DPS with FDD SCS 15kHz CBW=35/45MH (Alignment results)

|  | HST-SFN 2Rx | HST-SFN 4Rx | HST-DPS 2Rx | HST-DPS 4Rx |
| --- | --- | --- | --- | --- |
| CBW=35MHz | 10.8 | 8.8 | 9.7 | 7.2 |
| CBW=45MHz | 10.9 | 8.8 | 9.7 | 7.2 |

Table 2	SNR to achieve 70% of maximum throughput for PDSCH with HST-SFN/DPS with FDD SCS 15kHz CBW=35/45MH (Impairment results)

|  | HST-SFN 2Rx | HST-SFN 4Rx | HST-DPS 2Rx | HST-DPS 4Rx |
| --- | --- | --- | --- | --- |
| CBW=35MHz | 12.8 | 10.8 | 11.7 | 9.2 |
| CBW=45MHz | 12.9 | 10.8 | 11.7 | 9.2 |

# 3	References

- R4-2207196, “Big CR to 38.101-4: Introduction of FR1 HST demodulation requirements”, CMCC.