3GPP TSG-RAN WG4 Meeting #103-e	R4-2209074

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	10.8.6.2.2

Source:	Ericsson

Title:	BS demodulation requirements for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the way forward on BS demodulation requirements for Rel-17 NB-IoT [1], where RAN4 agreed to define NPUSCH format 1 demodulation requirements with 16QAM.

# 2	Discussion

The remaining open issue is the number of test cases including the number of allocated subcarriers, number of RUs, and TBS. We present our initial simulation results and discuss the open issues.

Number of allocated subcarriers - Option 1: 3,6,12 tones - Option 2: 12 tones - Option 3: 3 tones Number of scheduled RUs and TBS - Option 1: (IRU, ITBS) = (0,15), TBS=280 bits - Option 2: (IRU, ITBS) = (0,14), TBS=256 bits

Figure 1 shows our initial simulation results according to the way forward. According to our simulation results, we don’t observe the performance difference between 3 tones and 6 tones with regard to the SNR to achieve 70% of the maximum throughput. On the other hand, 12 tones case requires more SNR (~1dB) to achieve 70% of the maximum throughput compared with 3 and 6 tones. If companies have the same observation, we may need to define at least two requirements: one for 12 tones and another one is 3 tones, and it could be a good compromise considering the options.

| (a) TBS=256bits | (b) TBS=280bits |
| --- | --- |

Figure 1	NPUSCH format 1 with 16QAM

Observation 1: Required SNR to achieve 70% of the maximum throughput for NPUSCH format 1 with 3 tones and 6 tones are same, although 12 tones case requires about 1dB higher SNR to achieve 70% of the maximum throughput.

Proposal 1: Define NPUSCH format 1 demodulation requirements with 3 tones and 12 tones for 16QAM depending on the companies’ observations.

Another open issue is TBS used for NPUSCH format 1 demodulation requirements. According to our simulation results in Figure 1, TBS=280bits requires higher SNR to achieve 70% of the maximum throughput compare with TBS=256bits, as expected, but both the required SNR values are feasible. We slight prefer to set (IRU, ITBS) = (0, 15) so that the target code rate is close to A16-5. However we are also fine with setting (IRU, ITBS) = (0, 14).

Proposal 2: Set (IRU, ITBS) = (0, 15) and TBS=280 bits for NPUSCH format 1 demodulation requirements with 16QAM.

# 3	Summary

Observation 1: Required SNR to achieve 70% of the maximum throughput for NPUSCH format 1 with 3 tones and 6 tones are same, although 12 tones case requires about 1dB higher SNR to achieve 70% of the maximum throughput.

Proposal 1: Define NPUSCH format 1 demodulation requirements with 3 tones and 12 tones for 16QAM depending on the companies’ observations.

Proposal 2: Set (IRU, ITBS) = (0, 15) and TBS=280 bits for NPUSCH format 1 demodulation requirements with 16QAM.

# References

- R4-2207200, “Way forward for performance requirements of Rel-17 NB-IOT and eMTC”, Huawei, HiSilicon.

# Appendix

|  | IRU |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ITBS | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | Code rate with 16QAM |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |  |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |  |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |  |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |  |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |  |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  | ~0.5 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  | ~0.54 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  | ~0.56 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  | ~0.62 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  | ~0.70 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  | ~0.75 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  | ~0.80 |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  | ~0.88 |