3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209092

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-1: Adding the missing additional spurious emission requirement for CA_NC_NS_04 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2022-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The additional spurious emission requirement for CA_NC_NS_04 is missing in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The subclause 6.5A.3.3.2.1 on defining the additional spurious emission requirement for CA_NC_NS_04 is added based on the agreed CR R4-2016815 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for CA_n41(2A) is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change>

6.5A.3.3.1	Additional spurious emissions for intra-band contiguous  CA

6.5A.3.3.1.1		Requirement for network signalling value "CA_ NS_04"

When "CA_NS04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.1-1: Additional requirements for "CA_ NS_04"

| Frequency range (MHz) | BWChannel_CA (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 to 190 MHz |  |
| 2495 ≤ f < 2496 | -13 | Max(1 % of BWChannel_CA, 1 MHz) |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

6.5A.3.3.1.2		Requirement for network signalling value "CA_ NS_27"

When "CA_NS 27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.2-1: Additional requirements for "CA_ NS_27"

| Frequency range (MHz) | Spectrum emission limit (dBm) for aggregated channel bandwidth of max 40 MHz | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

6.5A.3.3.1.3		Requirement for network signalling value "CA_ NS_46"

When "CA_NS 46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.3-1: Additional requirements for “CA_NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for carriers confined in 2500-2570 MHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

6.5A.3.3.2	Additional spurious emissions for intra-band non-contiguous CA

6.5A.3.3.2.1		Requirement for network signalling value "CA_NC_ NS_04"

When "CA_NC_NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.2.1-1.

Table 6.5A.3.3.2.1-1: Additional requirements for "CA_ NC_NS_04"

| Frequency range (MHz) | BWChannel_CA (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 to 190 MHz |  |
| 2495 ≤ f < 2496 | -13 | Max(1 % of BWChannel_CA, 1 MHz) |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

6.5A.4	Transmit intermodulation for CA

# <End of change>