3GPP TSG-RAN WG4 Meeting#103-e      	R4-2209105

E-meeting, 9 – 20 May, 2022

Title:	Release Independence aspects of high-power UE operation for fixed-wireless/vehicle-mounted use cases

Source:	Nokia

Agenda item:	8.23.1

Document for:	Approval

# 1	Introduction

In WID [1] of High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands release independence aspects are marked as TBD, see Table 1.

| Band | contact company (email address) | other supporters (min. 3) | Status | Release independent from |
| --- | --- | --- | --- | --- |
| 5 | sebastian.thalanany@uscellular.com | Nokia, Ericsson and Samsung | Ongoing | Rel-10...16 (tbd) |
| 12 | sebastian.thalanany@uscellular.com | Nokia, Ericsson and Samsung | Ongoing | Rel-10...16 (tbd) |
| 13 | zheng.zhao@VERIZONWIRELESS.COM | Nokia, Ericsson, Qualcomm and Samsung | Ongoing | Rel-10...16 (tbd) |
| n5 | sebastian.thalanany@uscellular.com | Nokia, Ericsson and Samsung | Ongoing | Rel-15...16 (tbd) |
| n13 | zheng.zhao@VERIZONWIRELESS.COM | Nokia,,Ericsson, Qualcomm and Samsung | Ongoing | Rel-15...16 (tbd) |
| n26 | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Ongoing | Rel-15...16 (tbd) |
| n71 | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Ongoing | Rel-15...16 (tbd) |
| n85 | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Ongoing | Rel-15...16 (tbd) |

Figure 1: Capture from WID

In this contribution we discuss which release is applicable.

# 2	Discussion

Firstly, we discuss NR release independence aspect and present relevant table from TS 38.307, see table 5.1-2.

Table 5.1-2: NR UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 1 | FDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 1.5 | TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 2 | FDD, TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 3 | FDD, TDD, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |

We have two observations, firstly Power Class 1 is already specified due to the fact that n14 already supports PC1. Applicable release is REL15 which is same for all power classes. Secondly we observe that there is no band dependence to any of the power classes. Therefore, we propose

Proposal 1: Power class 1 operation shall be release independent from REL15 for all NR bands. This information is captured into WID.

Then we turn our attention to LTE. From table 3A.1-1 we can see that LTE power class 3 situation is not as straight forward as it is for NR. Reason is that REL8 singling did not support band numbers greater than 64 and numbering was extended in REL9 PC3 together with band number > 64 is release independent from REL9 but this due to band number issue not power class.

Power class 1 is release independent from REL10 for bands which have band number up to 64. Therefore, we propose

Proposal 2: Power class 1 operation shall be release independent from REL10 for LTE bands 5, 12 and 13. This information is captured into WID.

Table 3A.1-1: E-UTRA operating bands and UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 36.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands, band number <= 64, Power Class 3 | FDD, TDD | Rel-8 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number > 64, Power Class 3 | FDD, TDD | Rel-9 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, NS-value > 32 | FDD, TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Asymmetric operating bands, Power Class 3 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number <= 64, Power Class 1 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, Power Class 2 | TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |

# 3	Conclusion

In this contribution we have made following proposals

Proposal 1: Power class 1 operation shall be release independent from REL15 for all NR bands. This information is captured into WID.

Proposal 2: Power class 1 operation shall be release independent from REL10 for LTE bands 5, 12 and 13. This information is captured into WID.

# 4	References

[1] RP-220767, Revised WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands, Nokia, RAN#95