3GPP TSG-RAN4 Meeting #103-e	R4-2209154

# Electronic meeting, 9th May – 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | - | Current version: | 15.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to add ‘Annex G Difference of relative phase and power errors’ for FR2 UL coherent MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The 6.4D.4 Requirements for coherent UL MIMO specify maximum difference of relative phase and power errors, parameters not yet tested in other sections of 38.101-2 or previous 3GPP RATs. It is then necessary to give further details to RAN5 and TE vendors in an annex as done for the EVM so that what is to be measured is made clear and can be implemented as intended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Annex G Difference of relative phase and power errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear clause 6.4D.4 leading to misinterpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G, G.0, G-1, G-2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

# Annex G (normative):

Difference of relative phase and power errors

# G.0	General

This annex gives further information needed for understanding and implementing 6.4D.4. The following terms should be understood as follow:

- Relative phase: refers to the phase shift/difference between different antenna connectors, which should be ideally 0 degree. It should be understood as for a slot i.e. (slot) relative phase. It is calculated based on DMRS symbols of that slot or on SRS symbols.

- Relative phase error: refers to the phase shift/difference between different antenna connectors, which if different from 0 degree is considered as an error. It should be understood as for a slot i.e. (slot) relative phase error. It is calculated based on DMRS symbols of that slot or on SRS symbols.

- Difference of relative phase error: refers to the difference between the relative phase error determined based on DMRS symbols and the relative phase error determined based on the SRS transmitted.

# G.1	Measurement Point

Figure G.1-1 shows the measurement point for the unwanted emission falling into non-allocated RB(s) and the EVM for the allocated RB(s).

Figure G.1-1 - Measurement point for difference of relative phase/power error for UL coherent MIMO

# G.2	Relative Phase Error Measurement

Here are listed the different aspects that may lead to different interpretations.

## G.2.1. Symbols used

It should be determined based on DRMS REs (3 DMRS symbols per slot) with the option to use data symbols.

## G.2.2. CFO correction

The TE should perform a CFO correction on a slot-by-slot basis using a common frequency correction at the two uplink antenna connectors, that common frequency correction should be calculated as the average of the signed frequency errors at the two uplink antenna connectors.

## G.2.3. Steps of the measurement method

Below are detailed the steps necessary to obtain the maximum difference of relative phase error during the 20ms time window.

- Determination of the SRS relative phase error based on the last SRS transmitted on Ant1 and Ant2, that relative phase error serves as a reference for the calculation of the difference of relative phase error for each slot of the 20ms time window.

- The output is the “SRS relative phase error” for the last SRS transmitted: 1×1.

- CFO correction on slot-by-slot basis using a common frequency correction for both antenna outputs. Each slot to be characterised has then its CFO corrected.

- Determination for each subcarrier and each antenna of the slot phase based on the 3 DMRS symbols of the slot using channel estimation based on the LSE technique.

- The output is one vector of dimension 1×number_of_subcarriers for each antenna.

- Calculation for a slot for each subcarrier of the relative phase error (relative so between Ant1 and Ant2).

- The output is subcarrier relative phase errors of a slot: 1×number_of_subcarriers.

- Calculation for a slot of the DMRS relative phase error based on the quadratic mean (RMS) of the subcarrier relative phase errors determined in step 4.

- The output is a “DMRS relative phase error” for a slot: 1×1.

- Calculation for a slot of the difference of relative phase error based on the “SRS relative phase error” (reference) determined in step 1 and the “DMRS relative phase error” determined in step 5.

- The output is a “difference of relative phase error” value for a slot: 1×1.

- Perform for each slot of the 20ms time window, steps 2 to 6.

- The output is a “difference of relative phase error” vector: 1×number_of_slots.

- Calculation of the maximum value of the “difference of relative phase error”.

- The output is the “difference of relative phase error” that should be verified as complying with the ±40° requirement: 1×1.

<<End of change>>