3GPP TSG-RAN4 Meeting #103-e	R4-2209160

# Electronic meeting, 9th May – 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for new additions to Annex F8 related to the FR2 DMRS bundling requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The 6.4.2.6 Requirements for Phase continuity requirements for DMRS bundling, contain parameters not yet tested in other sections of 38.101-2 or previous 3GPP RATs. It is then necessary to give further details to RAN5 and TE vendors in the annex F.8 as done for the EVM so that what is to be measured is made clear and can be implemented as intended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New additions to F.8 Phase offset measurement for DMRS bundling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear clause 6.4.2.6 leading to misinterpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G, G.0, G-1, G-2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

# F.8	Phase difference measurement for DMRS bundling

## F.8.1	Measurement point

The measurement point for phase offset measurement is defined in Figure F.8.1-1.

Figure F.8.1-1: Measurement point for phase offset for DMRS bundling

## F.8.2. Symbols used

It should be determined based on DRMS REs (3 DMRS symbols per slot) with the option to use data symbols.

## F.8.3. CFO correction

The TE should perform a CFO correction on a slot-by-slot basis using a common frequency correction, that common frequency correction should be calculated as the average of the signed frequency errors of two slots (“slot 0 and any slot p” or “any slot p-1 and slot p” as defined in clause 6.4.2.6).

## F.8.4. Steps of the measurement method

Below are detailed the steps necessary to obtain the maximum phase difference for DMRS bundling for the specified measurement interval.

### F.8.4.1. “Phase difference between slot 0 and any slot p” case

Let suppose X being the numbers of bundles and Y the number of slots per bundles, Z the number of subcarriers within the BWP. The following steps could be followed to determine the phase difference for DMRS bundling for the “Phase difference between slot 0 and any slot p” case:

- For the jth bundle, determine CFO estimations of the reference slot giving ferror_ref_j and all the other slots of the jth bundle ferror_i_j with i the slot number.

- Output format: one vector of dimension 1×Y for the jth bundle.

- Determine for each combination ferror_refferror_i of the common frequency correction fcorrection_i to apply.

- Output format: one vector of dimension [1×Y-1] for the jth bundle.

- Apply fcorrection_i and determine for each subcarrier, the reference slot phase based on the 3 DMRS symbols of the reference slot using channel estimation based on the LSE technique.

- Output format: one vector of dimension 1×Z for the reference slot of the jth bundle.

- Apply fcorrection_i and determine for each subcarrier, the ith slot phase based on the 3 DMRS symbols of the ith slot using channel estimation based on the LSE technique.

- Output format: one vector of dimension 1×Z for the ith slot of the jth bundle.

- For the jth bundle, calculation for the ith slot for each subcarrier of the relative phase error (relative to the reference slot of the jth bundle).

- Output format: one vector of dimension 1×Z, subcarrier relative phase errors of a slot.

- For the jth bundle, calculation for the ith slot of the relative phase error based on the quadratic mean (RMS) of the subcarrier relative phase errors determined in step 5 (previous step).

- Output format: one vector of dimension 1×1, “relative phase error” for a slot.

- For the jth bundle, repeat steps 3 to 6 for each slot of the jth bundle

- Output format: 1×Y, “relative phase error” for all slots of the jth bundle.

- For the jth bundle, calculate the maximum value among the “phase difference” (relative phase error) values of the slots of the jth bundle.

- Output format: 1×1, “maximum phase difference” among all slots of the jth bundle.

- For each other bundle of the measurement interval (composed of X bundles), repeat steps 1 to 8.

- Output format: 1×X, “maximum phase difference” among all slots for each bundle.

- Calculate the maximum value among the “phase difference” (relative phase error) values of the bundles of the measurement interval.

- Output format: 1×1, “maximum phase difference” among all slots and all bundles.

### F.8.4.2. “Phase difference between any slot p-1 and slot p” case

Let suppose X being the numbers of bundles and Y the number of slots per bundles, Z the number of subcarriers within the BWP. The following steps could be followed to determine the phase difference for DMRS bundling for the “Phase difference between any slot p-1 and slot p” case:

- For the jth bundle, determine CFO estimations of the slots of the jth bundle ferror_i_j with i the slot number.

- Output format: one vector of dimension 1×Y for the jth bundle.

- Determine for each combination ferror_(i-1)ferror_i of the common frequency correction fcorrection_i to apply, between 1 and Y-1.

- Output format: one vector of dimension [1×Y-1] for the jth bundle.

- Apply fcorrection_i and determine for each subcarrier, the reference slot phase based on the 3 DMRS symbols of the slot preceding the ith slot using channel estimation based on the LSE technique.

- Output format: one vector of dimension 1×Z for the reference slot of the jth bundle.

- Apply fcorrection_i and determine for each subcarrier, the ith slot phase based on the 3 DMRS symbols of the ith slot using channel estimation based on the LSE technique.

- Output format: one vector of dimension 1×Z for the ith slot of the jth bundle.

- For the jth bundle, calculation for the ith slot for each subcarrier of the relative phase error (relative to the reference slot of the jth bundle).

- Output format: one vector of dimension 1×Z, subcarrier relative phase errors of a slot.

- For the jth bundle, calculation for the ith slot of the relative phase error based on the quadratic mean (RMS) of the subcarrier relative phase errors determined in step 5 (previous step).

- Output format: one vector of dimension 1×1, “relative phase error” for a slot.

- For the jth bundle, repeat steps 3 to 6 for each slot of the jth bundle

- Output format: 1×Y, “relative phase error” for all slots of the jth bundle.

- For the jth bundle, calculate the maximum value among the “phase difference” (relative phase error) values of the slots of the jth bundle.

- Output format: 1×1, “maximum phase difference” among all slots of the jth bundle.

- For each other bundle of the measurement interval (composed of X bundles), repeat steps 1 to 8.

- Output format: 1×X, “maximum phase difference” among all slots for each bundle.

- Calculate the maximum value among the “phase difference” (relative phase error) values of the bundles of the measurement interval.

- Output format: 1×1, “maximum phase difference” among all slots and all bundles.

<<End of change>>