3GPP TSG-RAN4 Meeting #103-e	R4-2209225

# Electronic Meeting, 9 – 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2364 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on general requirements for PRS measurements in RRC Inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some issues with general requirements for PRS measurement in INACTIVE state: - The definition of PRS resource for defining collision is pending, and there is an editor note for this issue. - It is unclear what other DL signals/channels refer to. - The applicability condition on PRS resources being in windows is missing, which is negative to UE power consumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the general requirements ofr PRS measurement in INACTIVE state: - Clarify that for defining the collision, expected RSTD and uncertainty are accounted. - Clarify “other DL signals/channels” based on RAN1 LS R1-2110644. - Add applicability condition on measurement window. - Add an editor note that possible impacts to PDSCH is pending on RAN1 conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general requirements for PRS measurement in INACTIVE state are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.6.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements defined in TS 38.215 [4], including RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP, in RRC_INACTIVE state.

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 are applicable to PRS resources that are not overlapped with other DL signals/channels, including SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT.

-	If a PRS resource is within the intitial DL BWP, the PRS resource overlap with other DL signals/channels when any other DL signals/channel occurs within the PRS resource.

-	If a PRS resource is outside the intitial DL BWP, the PRS resource overlap with other DL signals/channels when any of the following conditions is met:

-	any other DL signals/channel occurs within the PRS resource, or

-	any other signals/channel occurs within X symbols before the PRS resource, or

-	any other signals/channel occurs within X symbols after the PRS resource.

where X is defined in Table 5.6.1-1. For defining the collision between a PRS resource and other DL signals/channels, nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD of the neighbour cell/TRP are accounted.

Table 5.6.1-1: Value of X number of symbols

| FR |  | NR Slot length (ms) | X symbols |
| --- | --- | --- | --- |
| FR1 | 0 | 1 | 7 |
|  | 1 | 0.5 | 14 |
|  | 2 | 0.25 | 28 |
| FR2 | 2 | 0.25 | 14 |
|  | 3 | 0.125 | 28 |
| Note 1: The FR1 value applies if one or both of the serving cell and the positioning frequency layer are in FR1. FR2 value applies both of the serving cell and the positioning frequency layer are in FR2. |  |  |  |

All measurement requirements specified in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 shall apply for any DRX configuration specified in TS 38.331 [2].

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 are applicable provided that the cell selection procedure for the selected PLMN defined in TS 38.304 [1] is not triggered during PRS measurement period.

The UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

When the UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 are applicable provided that all PRS resources on the same positioning frequency layer are configured within [M] separate windows within Tavailable_PRS,i, where each window is up to [L] ms.

Editor’s Note: FFS for M, L and the location of the windows.

Editor’s Note: FFS on possible impacts on PDSCH reception, pending on RAN1 conclusion.

<End of Change 1>