3GPP TSG-RAN4 Meeting #103-e	R4-2209229

# Electronic Meeting, 9 – 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2367 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on scheduling restriction for PRS-RSRPP measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some issues with scheduling restriction requirements for PRS-RSRPP: - For priority option 2 with priority state 2, UE is expected to receive PDSCH for URLLC and drop PRS measurement, but the current wording says UE does not receive PDSCH which includes both URLLC and non-URLLC PDSCH. - For FR2 case, even UE supports Cap. 1B or 2, it cannot still not receive on another FR2 band if UE does not support IBM between the two FR2 bands. - The definition of serving cell symbols for defining collision is unclear and there is an editor note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the scheduling restriction requirements for PRS-RSRPP: - Clarify that for priority option 2 with priority state 2, UE is not expected to receive non-URLLC-PDSCH, which implies that UE will receive PDSCH for URLLC and drop PRS measurement. - Add additional restriction for FR2 CBM case. - Clarify that the serving cell symbols for defining collision should accoutn for expected RSTD and uncertainty of neighbor cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scheduling restriction requirements for PRS-RSRPP are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.6.7	Scheduling Availability of UE during PRS-RSRPP Measurement

When the UE performs PRS-RSRPP outside measurement gap, the following restrictions apply due to the measurement.

-	If Cap. 1A UE capable of supporting priority options 1, 2 and 3 is configured with priority state 1 for PRS-RSRP measurement, then UE is not expected to receive PDCCH/PDSCH/CSI-RS inside PPW.

-	If Cap.1A UE capable of supporting priority option 2 is configured with priority state 2 for PRS-RSRP measurement, then UE is not expected to receive non-URLLC-PDSCH/CSI-RS inside PPW.

-	If Cap. 1B UE capable of supporting priority options 1, 2 and 3 is configured with priority state 1 for PRS-RSRP measurement, then UE is not expected to receive PDCCH/PDSCH/CSI-RS in the same band as DL PRS inside PPW. In addition, if the PPW is for an FR2 serving cell, UE is not expected to receive PDCCH/PDSCH/CSI-RS in a differnet FR2 band than the FR2 serving cell if UE is incapable of independent beam management for this FR2 band pair

-	If Cap. 1B UE capable of supporting priority option 2 is configured with priority state 2 for PRS-RSRP measurement, then UE is not expected to receive non-URLLC-PDSCH/CSI-RS in the same band as DL PRS inside PPW. In addition, if the PPW is for an FR2 serving cell, UE is not expected to receive non-URLLC-PDSCH/CSI-RS in a differnet FR2 band than the FR2 serving cell if UE is incapable of independent beam management for this FR2 band pair

-	If Cap. 2 UE capable of supporting priority options 1, 2 and 3 is configured with priority state 1 for PRS-RSRP measurement, then the UE is not expected to receive PDCCH/PDSCH/CSI-RS on overlapped symbols with DL PRS inside PPW. In addition, if the PPW is for an FR2 serving cell, UE is not expected to receive PDCCH/PDSCH/CSI-RS in a differnet FR2 band than the FR2 serving cell if UE is incapable of independent beam management for this FR2 band pair

-	If Cap. 2 UE capable of supporting priority option 2 is configured with priority state 2 for PRS-RSRP measurement, then UE is not expected to receive non-URLLC-PDSCH/CSI-RS on overlapped symbols with DL PRS inside PPW with DL PRS inside PPW. In addition, if the PPW is for an FR2 serving cell, UE is not expected to receive non-URLLC-PDSCH/CSI-RS in a differnet FR2 band than the FR2 serving cell if UE is incapable of independent beam management for this FR2 band pair

For UE supporting PRS processing type 2, the overlapped symbols include serving cell PRS symbols, and serving cell symbols mapped with non-serving cell PRS taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD of the neighbour cell/TRP.

Editor’s Note: FFS on whether there is scheduling restriction for DL signals/channels of higher priority.

<End of Change 1>