3GPP TSG-RAN WG4 Meeting #103-e	R4-2209234

Electronic Meeting, 9 – 20 May, 2022

Title: 	Discussion on accuracy requirements and TCs for PRS-RSRPP

Source: 	Huawei, HiSilicon

Agenda item:	9.20.2.6

Document for:	Approval

Introduction

RRM requirements for PRS-RSRPP measurement were discussed in RAN4#102-e, and the outcomes are captured in the WF [1]. According to the work plan, RAN4 should discuss the performance requirements and test cases for PRS-RSRPP. In our view, the following issues should be discussed.

- Accuracy requirements

- Test cases

In this paper we will provide our views on the above issues for PRS-RSRPP measurement.

Discussion

Accuracy requirements

PRS-RSRPP is a new measurement, and though it is a power measurement similar to PRS-RSRPP, the way how it is measured and the contributing factors to the estimation error can be quite different from PRS-RSRP. Therefore, the existing PRS-RSRPP accuracy requirements cannot be re-used for PRS-RSRPP, and RAN4 needs to conduct link level simulations to understand the accuracy performance of PRS-RRSPP.

Proposal 1: RAN4 to conduct link level simulations to evaluate the baseband performance of PRS-RSRPP measurement.

For the simulation, one issue to be discussed is the propagation model.

In Rel-16 the PRS-RSRP accuracy requirements are defined without specific propagation model, which means the requirements are supposed to be applicable in all propagation models, including LOS and NLOS. For PRS-RSRPP we think it may be difficult to define accuracy requirements under NLOS fading channels because the PRS-RSRPP depends on the determination of the first path which can be varying over time with NLOS fading channel.

From use case point of view, PRS-RSRPP was introduced for improving DL-AoD by providing the path level RSRP for the LOS path, so it is more meaningful to define the accuracy requirements based on LOS channel. However, the exact LOS channel model also needs to be discussed.

It is not meaningful to use AWGN channel because there is only a single path and PRS-RSRP and PRS-RSRPP would be same, so the accuracy of PRS-RSRPP cannot be verified. It is more meaningful to use LOS fading channel e.g. TDL-D.

However, if we look into the TDL-D channel mode in 38.901, it can be found that the first path is stronger than the next strongest path by around 18dB, which means the difference between PRS-RSRPP for the first path and the PRS-RSRP (which includes power from all paths) will be rather small. We suggest RAN4 to further discuss what propagation model to use for defining PRS-RSRPP accuracy requirements.

Table 7.7.2-4. TDL-D.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -13.5 | Rayleigh |
| 2 | 0.035 | -18.8 | Rayleigh |
| 3 | 0.612 | -21 | Rayleigh |
| 4 | 1.363 | -22.8 | Rayleigh |
| 5 | 1.405 | -17.9 | Rayleigh |
| 6 | 1.804 | -20.1 | Rayleigh |
| 7 | 2.596 | -21.9 | Rayleigh |
| 8 | 1.775 | -22.9 | Rayleigh |
| 9 | 4.042 | -27.8 | Rayleigh |
| 10 | 7.937 | -23.6 | Rayleigh |
| 11 | 9.424 | -24.8 | Rayleigh |
| 12 | 9.708 | -30.0 | Rayleigh |
| 13 | 12.525 | -27.7 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 13.3 dB and a mean power of 0dB. |  |  |  |

One possible way forward in our view is that we can use a modified version of TDL-D. For example, to make the power difference between the LOS path and the other paths smaller, we can change the K-factor from 13.3dB to 0dB. In this case, the ideal PRS-RSRPP would be 3dB smaller than PRS-RSRP, and they can be differentiated meaningfully. We are also open to other views on the channel model.

Proposal 2: RAN4 to consider using TDL-D with K1=0dB for defining PRS-RSRPP accuracy requirements.

Next, PRS-RSRPP is the power of the first path, and a path is characterized by its TOA. The ideal PRS-RSRPP should be based on an ideal TOA estimation, and for evaluating the PRS-RSRPP accuracy, realistic TOA estimation should be used because in real world UE does not know the TOA of the path but has to estimate the TOA also from PRS.

This means if there is an error in TOA estimation, the PRS-RSRPP derived based on the estimated TOA will be different from the ideal PRS-RSRPP even the PRS-RSRPP estimation is perfect, so the PRS-RSRPP error will not be contributed only by the PRS-RSRPP estimation (this error contribution is similar to that for PRS-RSRP estimation e.g. counting noise as useful power), but also from the possible error in TOA estimation.

Proposal 3: Use realistic TOA estimation in the simulation for PRS-RSRPP.

In Rel-16 accuracy requirements for PRS-RSRP are defined for following side conditions:

- Es/Iot >= -3dB, BW >= 24 RBs

- Es/Iot >= -13dB, BW >= 24/52/104 RBs

- The accuracy are based on 4-sample and all propagation conditions

In Rel-17 another set of accuracy requirements for PRS-RSRP will be defined for following side conditions:

- Es/Iot >= -6dB, BW >= 48 RBs

- The accuracy are based on 1-sample and LOS propagation conditions

As discussed above, the propagation condition for PRS-RSRPP is proposed to be LOS. If this is agreeable, only the following two sets of side conditions are needed. RAN4 can further check how to define the requirements e.g. by some BW bins can be grouped, based on the simulation results.

- Set 1: Es/Iot >= -13dB, BW >= 24/52/104 RBs, 4 sample

- Set 2: Es/Iot >= -6dB, BW >= 52/104 RBs, 1 sample

Proposal 4: Use the following Es/Iot and PRS BW conditions at least for the simulation of PRS-RSRPP accuracy requirements.

- Set 1: Es/Iot >= -13dB, BW >= 24/52/104 RBs, 4 sample

- Set 2: Es/Iot >= -6dB, BW >= 52/104 RBs, 1 sample

Finally, we think RAN4 should try to align the understanding on how ideal PRS-RSRPP is defined. Based on the definition of PRS-RSRPP in 38.215, PRS-RSRPP is the power of the channel response at the i-th path of the RE. As the channel response is in time domain, how it translates into a power on RE level in frequency domain needs to clarified, otherwise the delta PRS-RSRPP results from different companies are not comparable (as different companies may have different ideal values).

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

In our RAN1 contribution [2], we propose to define the ideal PRS-RSRPP for the p-th path as

RSRPPp∝kHkexpj2πDpkNIFFT2

where Hk is the channel response in frequency domain for the k-th RE, Dp is the delay of the p-th path.

The physical meaning of the calculation is that channel response at the p-th path delay on a resource element corresponds to the response on the resource element of a shifted version of the channel where the i-th path delay is shifted to 0. We think this definition can be used define the ideal PRS-RSRPP given the ideal channel Hk and ideal TOA of the path Dp, but we are also open to other view.

Proposal 5: RAN4 to align how ideal PRS-RSRPP is derived based on the definition.

The following can be considered

RSRPPp∝kHkexpj2πDpkNIFFT2

where Hk is the channel response in frequency domain for the k-th resource element, Dp is the delay of the p-th path.

Based on above Proposal 1 to 4, we summarized the simulation assumption in the Annex.

Proposal 6: Use the simulation assumption in Annex as starting point for link level simulations.

Test cases

For PRS-RSRP, the measurement period is same as PRS-RSRPP, so we do not see the need to define delay tests for PRS-RSRPP since a UE that can pass the delay tests for PRS-RSRP should also be able to meet the delay requirements for PRS-RSRPP.

RAN4 has first decided on the measurement period, so any UE would be able to generate a PRS-RSRPP result within the specified measurement period, but what should be taken care of is the accuracy for PRS-RSPRP based on the agreed measurement period. We have discussed in the accuracy previous section, and it is meaningful to verify whether the reported PRS-RSRPP meets the accuracy requirements, which are to be derived based on the simulations.

Therefore, it is meaningful to define accuracy TCs for PRS-RSRPP. This cannot be combined with the existing or newly defined accuracy TCs for PRS-RSRP since the propagation would be different. We suggest to define the following TCs for PRS-RSRPP measurement.

Proposal 7: Define the following 2 TCs for PRS-RSRPP measurement.

- PRS-RSRPP measurement accuracy for FR1 in SA

- PRS-RSRPP measurement accuracy for FR2 in SA

Conclusions

In this paper we provided our views on accuracy requirements and TCs for PRS-RSRPP measurement.

Proposal 1: RAN4 to conduct link level simulations to evaluate the baseband performance of PRS-RSRPP measurement.

Proposal 2: RAN4 to consider using TDL-D with K1=0dB for defining PRS-RSRPP accuracy requirements.

Proposal 3: Use realistic TOA estimation in the simulation for PRS-RSRPP.

Proposal 4: Use the following Es/Iot and PRS BW conditions at least for the simulation of PRS-RSRPP accuracy requirements.

- Set 1: Es/Iot >= -13dB, BW >= 24/52/104 RBs, 4 sample

- Set 2: Es/Iot >= -6dB, BW >= 52/104 RBs, 1 sample

Proposal 5: RAN4 to align how ideal PRS-RSRPP is derived based on the definition.

The following can be considered

RSRPPp∝kHkexpj2πDpkNIFFT2

where Hk is the channel response in frequency domain for the k-th resource element, Dp is the delay of the p-th path.

Proposal 6: Use the simulation assumption in Annex as starting point for link level simulations.

Proposal 7: Define the following 2 TCs for PRS-RSRPP measurement.

- PRS-RSRPP measurement accuracy for FR1 in SA

- PRS-RSRPP measurement accuracy for FR2 in SA

Reference

- R4-2206979, WF on NR Positioning Enhancements (Part 1), Ericsson

- R1-2203099, Maintenance of Rel-17 positioning accuracy improvements, Huawei, HiSilicon

Annex: simulation assumption for PRS-RSRPP

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the serving cell/reference cellNOTE 1 |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30, 60 kHz - FDD, TDD | - 40 GHz - 60, 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | TDL-D (30 ns delay spread, 5Hz) | TDL-D (30 ns delay spread, 5Hz) |
| Es/Iot for tw cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13), (-3, -6, -6) |  |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 2 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| NOTE 1: nr-DL-PRS-ReferenceInfo is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  |  | 132 | 1 | 32 |
|  | 60, FR1 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 1 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |
| Note: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. |  |  |  |  |

At least the following performance characteristics are to be provided for PRS-RSRPP:

- PRS-RSRPP error CDFs for 3 cells

- 5%-ile and 95-ile of the RSRP errors for each cell

In the above,

- PRS-RSRPP error = estimated PRS-RSRPP – ideal PRS-RSRPP (based on perfect channel knowledge and perfect knowledge of the first path TOA).