3GPP TSG-RAN WG4 Meeting #103-e                                      R4-2209246

Electronic Meeting, May 9 – 20, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on adding 25MHz channel bandwidth in Band n28

Agenda Item:	8.21.2.1

Document for:	Approval

Introduction

Regarding the WI [1] approved in RAN#85 meeting, it is tasked for RAN4 to add wider channel bandwidth in band n28. Based on the discussion for 30 MHz channel bandwidth, in this paper we would like to further provide our views on requirements for 25 MHz channel bandwidth.

Discussion

AMPR simulation assumption and result

According to the statements in TR [2], we see only additional spectrum emission requirements defined in Table 6.5.3.3.3-1 should be guaranteed if the channel bandwidth is larger than 10 MHz:

##### 6.5.3.3.3 Requirement for network signalling value "NS_18" When "NS_18" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3. 3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. Table 6.5.3.3.3-1: Additional requirements for "NS_18" Frequency range (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 5, 10, 15, 20, 30. 692-698; -26.2; 6 MHz.

Consequently, we reuse most of the 30 MHz AMPR simulation assumption for 25 MHz:

- PA calibrated to deliver -30 dBc ACLR for a fully allocated QPSK DFT-S-OFDM waveform at 1 dB MPR - -28 dBc image and carrier rejection - 25 MHz channel bandwidth located at the lowermost channel (703~728) (Centre frequency is 715.5 MHz) - CIM3: 60 dBc

Accordingly, simulation results are provided as in Annex. We propose the NS_18 AMPR requirements for 25MHz channel bandwidth as below:

Table 1. A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~728 | > 1.25*LCRB*12*SCS-2.925 | ≥ 12*SCS*NRB – 1.8 – RBstart*12*SCS | A6 |
|  |  | ≤ 1.25*LCRB*12*SCS-2.925 | ≥ 6.66 | A7 |
|  |  | ≤ 5.4 | < 6.66 | A8 |

Table 2. A-MPR for NS_18

| Modulation/Waveform |  | A6 | A7 | A8 |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | [3] | [6] | [4] |
|  | QPSK | [3] | [6] | [4] |
|  | 16 QAM | [3] | [7] | [4] |
|  | 64 QAM | [4] | [7] | [5] |
|  | 256 QAM | [4] | [7] | [5] |
| CP-OFDM | QPSK | [5] | [8.5] | [6.5] |
|  | 16 QAM | [5] | [8.5] | [6.5] |
|  | 64 QAM | [5] | [8.5] | [6.5] |
|  | 256 QAM | [5] | [9] | [7.5] |

Figure 1. NS_18 A-MPR regions

Proposal: Specify the NS_18 A-MPR requirements in Table 1 and Table 2 for band n28 25MHz channel bandwidth.

Conclusion

In this contribution we provide the simulation results for NS_18 A-MPR requirements for band n28 25MHz channel bandwidth. Based on the results in Annex, we have the following proposal:

Proposal: Specify the NS_18 A-MPR requirements in Table 1 and Table 2 for band n28 25MHz channel bandwidth.

- References

- RP-192261, New WI proposal: Addition of wider channel bandwidth in NR band n28, CBN, Huawei, HiSilicon, RAN#85, Newport Beach, USA, September 16 – 20, 2019.

- 3GPP TR 38.888, v16.0.0, March, 2020.

- Annex (NS_18 AMPR simulation results for 25MHz)