3GPP TSG-RAN WG4 Meeting #103-e	R4-2209294

# Electronic Meeting, 9 - 20 May, 2022

Source:	Huawei, HiSilicon, CBN

Title:	Updated TP for 37.717-00-00 to add SUL configuration SUL_n79C-n83A for CA_n28A_SUL_n79C-n83A

Agenda item:	8.19.2

Document for:	Approval

1 Background

This contribution provides updated text proposal on the NR SUL band combination CA_n28A_SUL_n79C-n83A with SUL_n79C-n83A.

2 Text Proposal

---Start of changes---

## 5.4	CA_n28A_SUL_n79A-n83A

### 5.4.1	Operating bands

Table 5.4.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n28_SUL_n79-n83 | n28, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.4.2	Channel bandwidths per operating band

Table 5.4.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A_SUL_n79C-n83A | SUL_n79A-n83A SUL_n79C-n83A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |

### 5.4.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.4.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

Referring to clause 7.3.2.2 from TR 38.862, we can get the IMD order for contiguous UL CA as below.

WGap = lowest band edge of n79 - highest DL band edge of n28 = 4400 – 803 = 3597 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 600) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 37.

Since IMD order is > 11, MSD can be ignored.

### 5.4.5	REFSENS requirements

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.4.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n83 | 15 | 100 | 100 | 100 | 100 |  | 100 |  |  |  |  |  |  |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 |  |  |  |  |  |  |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 | 100 |  | 100 |
|  |  | 30 |  |  |  |  |  |  | 50 | 50 | 50 | 50 |  | 50 |

The reference sensitivity exception between band n83 and n28 are specified below. The reference sensitivity exception between band n83 and n28 are specified below. The operation is limited to using equal or wider separation from n83 Tx to n28 Rx compared to n28 Tx-Rx separation. The reference sensitivity exception does not take into account the condition where band n83 channel bandwidths is wider than band n28. Otherwise, UE REFSENS performance will be degraded.

Table 5.4.5-2: Reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n83 | n28 | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |

Table 5.4.5-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n83 | n28 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.4.6	∆TIB and ∆RIB values

For CA_n28_SUL_n79-n83, the TIB,c and RIB,c values are given in the tables below.

Table 5.4.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28_SUL_n79-n83 | n28 | 0.5 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |

Table 5.4.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28_SUL_n79-n83 | n28 | 0.2 |
|  | n79 | 0.5 |

---End of changes---

Reference

[1]	RP-220695, “Revised WID: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, Huawei, HiSilicon