3GPP TSG-RAN WG4 Meeting # 103-e                	R4-2209390

Electronic Meeting, February 21 – March 3, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUSCH simulation results for the extension to 71 GHz

Agenda item: 	9.15.10.1

Document for: 	Discussion

Introduction

This paper presents PUSCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

| Parameter | value |
| --- | --- |
| Transform precoding | disabled |
| CP | Normal CP |
| Carrier frequency | 70 GHz |
| DMRS symbols | [1 9] |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| SCS | 120, 480, and 960 kHz |
| TDD pattern | 120 kHz: 3D1S1U, S=10D:2G:2U 480 kHz: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U 960 kHz: 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U |
| DMRS type | Type 1 with single-symbol DM-RS (l0=2) |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups with data | 2 |
| Symbol length | 10 |
| Start symbol index | 1 |
| PRTS | On |
| HARQ | ON |
| Time domain resource allocation type | type B |
| Frequency domain resource | For 120 kHz SCS and - CHW= 100 MHz: 66 - CHW= 400 MHz: 264 For 480 kHz SCS and - CHW= 400 MHz: 66 - CHW= 800 MHz: 132 - CHW= 1600 MHz: 264 For 960 kHz SCS (aligned with RF agreements R4-2206536) and - CHW= 400 MHz: 33 - CHW= 800 MHz: 66 - CHW= 1600 MHz: 132 - CHW= 2000 MHz: 156 |
| MCS index | MCS 4, 16, 20 |
| Propagation condition | TDL-A5, TDLA10 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Phase noise model | 38808 Set2 |
| Number of HARQ transmissions | 4 |
| RV sequence | {0.2.3.1} |
| Testing metric | SNR at 70% of the maximum throughput |

Simulation results

| SCS | MCS | CHBW MHz | N PRBs | Channel model | SNR @ 70% TPUT |
| --- | --- | --- | --- | --- | --- |
| 120 | 4 | 100 | 66 | TDL-A 10 | -3.62 |
|  |  | 400 | 264 | TDL-A 10 | -4.04 |
|  | 16 | 100 | 66 | TDL-A 10 | 7.55 |
|  |  | 400 | 264 | TDL-A 10 | 7.09 |
|  | 20 | 100 | 66 | TDL-A 10 | 11.01 |
|  |  | 400 | 264 | TDL-A 10 | 10.45 |
| 480 | 4 | 400 | 66 | TDL-A 5 | -3.62 |
|  |  | 800 | 132 | TDL-A 5 | -4.01 |
|  |  | 1600 | 264 | TDL-A 5 | -3.67 |
|  | 16 | 400 | 66 | TDL-A 5 | 8.31 |
|  |  | 800 | 132 | TDL-A 5 | 7.07 |
|  | 20 | 400 | 66 | TDL-A 5 | 11.44 |
|  |  | 800 | 132 | TDL-A 5 | 10.46 |
| 960 | 4 | 400 | 33 | TDL-A 5 | -3.29 |
|  |  | 800 | 66 | TDL-A 5 | -3.80 |
|  |  | 1600 | 132 | TDL-A 5 | -3.42 |
|  |  | 2000 | 156 | TDL-A 5 | -3.47 |
|  | 16 | 400 | 33 | TDL-A 5 | 8.08 |
|  | 20 | 400 | 33 | TDL-A 5 | 11.95 |
|  |  |  |  | TDL-A 10 | 10.99 |
|  |  | 800 | 66 | TDL-A 5 | 10.81 |
|  |  |  |  | TDL-A 10 | 12.30 |

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation