3GPP TSG-RAN WG4 Meeting # 103-e		R4-2209416

Electronic Meeting, May 09 – May 20, 2022

Source:	China Telecom

Title:	Simulation results on UE MMSE-IRC receiver for intra-cell inter-user interference suppression

Agenda Item:	9.11.2.2

Document for:	Discussion

- Introduction

In the RAN4#102-e meeting, all the test parameters for MMSE-IRC receiver for suppressing intra-cell inter-user interference have been agreed. In this paper, our simulation result based on the agreed parameters in the approved WF [1] are given.

- Simulation results

In this section, our simulation result based on the agreed parameters in the approved WF[1] are given in the below table.

Table 1. Ideal and impairment SNR @ 70% max TP of MMSE-IRC receiver suppressing intra-cell inter-user interference

| Duplex Mode | Channel model | Rank for target UE | Rank for co-schedule UE | Antenna Configuration | Alignment Result | Impairment Result |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | TDLC300-100 | 1 | 1 | 2T2R Low | 15.1 | 16.6 |
|  |  |  |  | 2T4R Low | 8.7 | 10.2 |
|  | TDLA30-10 | 2 | 2 | 4T4R Low | 13.5 | 15.0 |
| TDD | TDLC300-100 | 1 | 1 | 2T2R Low | 15.9 | 17.4 |
|  |  |  |  | 2T4R Low | 9.1 | 10.6 |
|  | TDLA30-10 | 2 | 2 | 4T4R Low | 13.5 | 15.0 |

- References

R4-2207244, Way Forward on MMSE-IRC for intra-cell inter-user interference, Huawei, RAN4 #102-e, March 2022.