3GPP TSG-RAN WG4 Meeting #103-e		R4-2209432
Electronic Meeting, 09th May. – 20th May., 2022

Agenda item:	9.2.3.1

Source:	OPPO

Title:	Lab alignment criteria for FR1 TRP TRS campaign

Document for:	Approval

1	Introduction

The FR1 TRP TRS lab alignment campaign is kicked off after RAN4 #102-e meeting. And more than 3 labs have finished LADs measurements. However, the lab alignment criteria are not clearly finalized so far. The latest working procedure for performance activity [1] is approved in RAN4 #102-e. The conclusions on lab alignment criteria are captured below.

This paper presents our views and proposals on the leaving issues of lab alignment criteria.

2	Discussion

Two issues are discussed in the paper. The first one is how to identify apparent outliers. The second one is how to perform averaging approach to derive the reference value. Finally, Pass/Fail limit for lab alignment is proposed.

2.1 Identify apparent outliers

It is normal that there will be some deviations between different labs even if the same test equipment used. The expanded measurement uncertainties in TR38.834 [2] Annex B quantify the possible deviation with confidence interval of 95%.

With this understanding, the measurement results within the gap of the measurement uncertainty is reasonable, and should be not treated as apparent outliers. Therefore, it is proposed to identify the apparent outliers as follow.

Proposal 1: It is proposed to identify the apparent outliers based on the above table.

2.2 Averaging approach

For TRP, there are two approaches to perform the averaging of collected data, i.e. averaging in dBm and averaging in mW. Considering the apparent outliers are already removed, averaging in dBm is the preferable approach.

Proposal 2: For TRP, the reference value of LAD is derived with averaging approach in dBm.

For TRS, there are four approaches to perform the averaging of collected data, i.e. linear averaging in dBm, inverse averaging in dBm, linear averaging in mW and inverse averaging in mW. The same consideration with TRP, two averaging methods in dBm are down-selected. With the understanding that inverse averaging approach is effectively applied on power in mW, it is proposed to derive the TRS reference value of LAD with linear averaging approach in dBm.

Proposal 3: For TRS, the reference value of LAD is derived with linear averaging approach in dBm.

2.3 Pass/Fail limit for lab alignment

As we have analysed in previous document [3], labs with some measurement deviation may not necessarily affect the performance requirement derivation. To encourage more labs have opportunity to provide commercial devices measurement data, 1 MU is proposed as Pass/Fail limit for lab alignment.

Proposal 4: 1 MU is proposed as Pass/Fail limit for lab alignment.

3	Conclusion

Proposal 1: It is proposed to identify the apparent outliers based on the below table.

Proposal 2: For TRP, the reference value of LAD is derived with averaging approach in dBm.

Proposal 3: For TRS, the reference value of LAD is derived with linear averaging approach in dBm.

Proposal 4: 1 MU is proposed as Pass/Fail limit for lab alignment.

References

[1] R4-2207317 Update of Working procedure for performance activity

[2] 3GPP TR 38.834 v17.0.0, Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology (Release 17)

[3] R4-2204987 Views on Pass/Fail limit for lab alignment