3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2209491

# Electronic Meeting, May 09 – May 20, 2022

Source: 	vivo

Title: 	Discussion on L1-SINR measurements in R17 FR1 HST

Agenda Item:	10.8.2

Document for:	Discussion

Introduction

In RAN4 100e, the WF R4-2115439 on FR1 HST RRM enhancements was agreed in [1]. In Issue 3-1 and Issue 3-2, the issue about L1-SINR was discussed and some evaluation assumptions were agreed in [1].

In RAN4 101e, 101-bis-e and 102-e meeting, the following on L1-SINR were captured in [2][3][4].

Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario

Agreements in 1st round

- For L1-SINR for FR1 HST

  - The existing measurement delay requirements for L1-SINR is reused

  - The existing accuracy requirements for L1-SINR is reused

    - Whether and how to define applicability of side condition for the L1-SINR accuracy requirements can be further discussed and concluded in the perf. phase

      - Option 1: the upper bound of the side condition for L1-SINR accuracy is 5dB

      - Option 2: no upper bound of side condition for L1-SINR accuracy

In this paper our views on the issue of L1-SINR measurements in FR1 HST are provided.

Discussion on L1-SINR accuracy requirements

Scenarios of L1-SINR measurements

For the L1-SINR requirements discussion in R17 FR1 HST, the same scenarios as in R16 should be considered. In R16 FR1 HST, from UE perspective, typical deployment scenarios were discussed, and the corresponding demod requirements for HST-SFN and HST-DPS were defined clause 5.2.2.1.9 and 5.2.2.1.10 of TS 38.101-4 [5]. The issue about the number of TCI states that UE can track was extensively discussed in R16 HST in demod session. It was concluded that UE is mandatory to track only 1 TCI state, i.e. supporting scenario of DPS 1a. For UE who supports to track more than 1 TCI state, tests for DPS 1b or HST-SFN may apply. However, support of HST-SFN is an optional UE feature as described in the R16 UE feature list [6].

Observation 1 According to demod discussion in R16 HST, since UE is mandatory to support tracking only 1 TCI state, the baseline scenario for defining requirements should be DPS 1a scenario.

Observation 2 Support of frequency tracking and demodulation under R16 HST-SFN scenario is an optional UE capability according to R16 UE feature list.

The figure for DPS is provided in figure 1 for convenience. According to the DPS 1a test case setup in 5.2.2.1.10 and the Doppler trajectory defined in B3.3 of TS 38.101-4[5], the TCI state here is based on TRS tracking, and the main information conveyed to UE in the TCI is QCL-A, i.e. delay and doppler information.

Figure 1  DPS scenario defined in TS 38.101-4 B3.3 for R16 HST

Regarding the max Doppler shift considered in R16 HST, in demod session, there were various values defined for different cases, summarized in Table 1. The doppler trajectory for HST-DPS, for example, is also shown in Figure 2. There was extensive discussion on the relation between HST DPS and HST single tap, and the reason to consider only 870Hz for DPS is to deal with the max Doppler jump of 1740Hz as shown in the right part of Figure 2, although for some other UE it is also OK to consider less Doppler jump as shown based on the left part. Note that according to TRS pattern, the frequency track ability of TRS is +/-1750Hz for SCS15kHz, and +/-3500Hz for SCS30kHz.

On the other hand, for HST single tap, there was discussion on whether to reuse such test methodology in NR, and whether to adopt the value larger than 875Hz for such test in 15kHz FDD case. Based on operator’s input on the carrier frequency and the assumed UE speed, higher Doppler is possible, which is beyond UE capability for Doppler tracking especially considering UE capable of tracking only 1 TCI. Therefore, companies finally achieved the compromise of 972Hz for 15kHz FDD in HST single-tap with a note captured under Table B3.1-1 that 972 Hz is only applied for performance verification.

Note that for DPS and SFN, the actual doppler shift considered for test cases is 870Hz but not 972Hz. It is within TRS tracking ability. In R17 network-based pre-compensation is extensively discussed in R17 feMIMO WI in RAN1. Our understanding is that network may also be able to perform pre-compensation based on R15/R16 specifications and hence from UE requirement perspective, it is feasible not to consider such high doppler shift in DPS and SFN.

| [Hz] | HST single-Tap | HST-DPS | HST-SFN | HST multi-path |
| --- | --- | --- | --- | --- |
| 15kHz FDD | 972 | 870 | 870 | 600 |
| 30kHz TDD | 1667 | 1667 | 1667 | 1200 |
| Note: These values are for 500km/h HST. For HST single Tap, 750Hz for 15kHz FDD and 1000Hz for 30kHz TDD were also defined for 300km/h HST in R15. |  |  |  |  |

Table 1 Summary of max Doppler shift considered in various scenarios in R16

Figure 2 Doppler shift trajectory considered for DPS scenario (870 Hz for 15kHz-SCS)

Observation 3 According to TS 38.101-4, 972Hz Doppler shift for SCS15kHz HST single tap is only for performance verification, and it is not meant to indicate the max Doppler shift UE needs to dealt with in real SCS15kHz HST deployment.

Observation 4 According to TRS pattern, the frequency track ability of TRS is +/-1750Hz for SCS15kHz, and +/-3500Hz for SCS30kHz.

Analysis of L1-SINR measurement accuracy

In R16 discussion on L3 SS-SINR under HST scenario, the issue about the in-accurate measurement in high SINR scenario was discussed. The calculation of SINR was clarified in [7].

RSRPNoise+interference+ICI

It seems the reason for relaxation is that ICI will degrade the accuracy of SINR. However, at that time, according to evaluation results provided in [7], companies see the issue that when L3 SS-SINR measurement is performed on the neighbour cell or neighbour RRH, the accuracy can be degraded due to the ICI. However, if it is performed on the serving cell, as shown in Table 2 of [7], there is no performance degradation even though interference from the neighbour cell can still be 1944Hz. In other word, our understanding is that there will be no ICI for the serving cell reception, i.e. the tracked RRH, if one-tap model is considered.

Note that in last meeting, some companies raised the issue that there could be residual frequency error even for the DPS 1a scenario, which is significant if multi-shot TRS-based tracking is considered and when UE passes-by the RRH. We observed that the residual frequency error issue was also discussed in R16, but the impact of it is trivial according to some of the contributions. [8] Normally in the higher Es/iot region, UE needs not to perform multi-shot TRS based tracking, and the time duration of unlocked frequency error is no more than 40ms.

Observation 5  In R16 SS-SINR accuracy evaluations for HST, no performance degradation is shown on the serving cell SS-SINR measurements if the one-tap scenario is considered, even assuming 1944Hz Doppler shift, and the impact of residual frequency error when UE passes-by RRH is trivial.

Note that L3 SS-SINR measurement is based on SSB, which is a cell-specific RS and is also used for cell detection, therefore it is not able to perform any frequency tracking before L3 measurements are performed on SSB. On the other hand, according to observation 1, L1-SINR measurements are based on CSI-RSs, which can be UE-specific reference signals. TCI states can also be configured to CSI-RSs that used for L1-SINR measurements. Hence it is feasible for some pre-compensation and there is no need to consider higher Doppler the same as SSB-based scenario, especially if the TCI is active and UE performs time-frequency tracking of the TCI based on TRS. Moreover, in R16 L1-SINR measurement requirements, the requirements were specified without considering that the Doppler shift can exceed TRS tracking ability.

Observation 6  The considered scenario in R17 is different from R16, since CSI-RSs for L1 measurements are UE-specific RSS on which UE may perform time-frequency tracking according to TRS, but SSBs for L3 measurements are cell-specific RSs for cell detection, which are measured without time-frequency finer tracking.

According to above analysis and performance evaluation result in annex, we observe that for DPS 1a scenario, if the max doppler offset is beyond max TRS tracking ability, performance degradation can be expected. But if not, no performance degradation is expected, while L1-SINR measurement is performed on the RSs that associated to the active TCI. As shown in the evaluation results of [9], for L1-SINR measurements performed on the neighbour RRH, performance degradation can be expected.

Proposal 1  For DPS 1a scenario, if max doppler shift does not beyond TRS tracking ability,

- No impact to L1-SINR measurement accuracy requirements if the measured RS is associated with active TCI of the UE in DPS 1a scenario, i.e. legacy performance requirements still apply to DPS 1a scenario.

- No accuracy requirements for L1-SINR measurements on RSs that are not associated with active TCI of the UE in DPS 1a scenario when side condition is above 5dB.

On the other hand, for DPS 1b or HST-SFN scenario, the frequency tracking trajectory was also discussed in R16. Since there could be some residual frequency error, the DMRS would be important for residual frequency error compensation. However, for L1-SINR measurements, it would be impossible to compensate the residual frequency error. Therefore,

Proposal 2  For DPS 1b or HST-SFN scenario, no accuracy requirements for L1-SINR measurements when side condition is above 5dB.

Conclusion

Based on above analysis, we have following observations and proposals.

Observation 1 According to demod discussion in R16 HST, since UE is mandatory to support tracking only 1 TCI state, the baseline scenario for defining requirements should be DPS 1a scenario.

Observation 2 Support of frequency tracking and demodulation under R16 HST-SFN scenario is an optional UE capability according to R16 UE feature list.

Observation 3 According to TS 38.101-4, 972Hz Doppler shift for SCS15kHz HST single tap is only for performance verification, and it is not meant to indicate the max Doppler shift UE needs to dealt with in real SCS15kHz HST deployment.

Observation 4 According to TRS pattern, the frequency track ability of TRS is +/-1750Hz for SCS15kHz, and +/-3500Hz for SCS30kHz.

Observation 5  In R16 SS-SINR accuracy evaluations for HST, no performance degradation is shown on the serving cell SS-SINR measurements if the one-tap scenario is considered, even assuming 1944Hz Doppler shift, and the impact of residual frequency error when UE passes-by RRH is trivial.

Observation 6  The considered scenario in R17 is different from R16, since CSI-RSs for L1 measurements are UE-specific RSS on which UE may perform time-frequency tracking according to TRS, but SSBs for L3 measurements are cell-specific RSs for cell detection, which are measured without time-frequency finer tracking.

Proposal 1  For DPS 1a scenario, if max doppler shift does not beyond TRS tracking ability,

- No impact to L1-SINR measurement accuracy requirements if the measured RS is associated with active TCI of the UE in DPS 1a scenario, i.e. legacy performance requirements still apply to DPS 1a scenario.

- No accuracy requirements for L1-SINR measurements on RSs that are not associated with active TCI of the UE in DPS 1a scenario when side condition is above 5dB.

Proposal 2  For DPS 1b or HST-SFN scenario, no accuracy requirements for L1-SINR measurements when side condition is above 5dB.

- References

[1]  R4-2115439  WF on RRM for FR1 HST, CMCC.  RAN4 #100-e

[2]  R4-2120285  WF on RRM for FR1 HST, CMCC.  RAN4 #101-e

[3]  R4-2202590  WF on RRM for FR1 HST, CMCC.  RAN4 #101-bis-e

[4]  R4-2206845  WF on RRM for FR1 HST, CMCC.  RAN4 #102-e

[5]  TS 38.101-4  v16.6.0.

[6]  TR 38.822  v16.2.0

[7]  R4-1910824  SS-SINR measurement accuracy in HST, MediaTek.  RAN4 #92bis

[8]  R4-2001457  Discussion on the requirement definition for NR UE HST single tap channel, Huawei, HiSilicon.  RAN4 #94e

[9]  R4-2204204  Discussion on Rel-17 HST in FR1, MediaTek.  RAN4 #102e

- Annex  L1-SINR evaluation results

On the agreed evaluation assumptions in [1], according to the scenario discussed above, it makes no sense to us why HST condition in G.3 of TS 38.104 is considered here. In our view it should be removed. Considering DPS 1a, carrier frequency offset is already enough to describe the issue. Therefore, we have removed the assumptions on propagation condition or revise it to Table B3.3-1 in TS 38.101-4.

The updated evaluation assumptions are provided in Table 2 below.

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 | 3.6 GHz |  |
| channel | AWGN |  |
| BS transmit antennas for CSI-RS | 1 tx |  |
| UE receive antennas | 2 rx |  |
| Data channel subcarrier spacing | The same as CSI-RS subcarrier spacing |  |
| Measurement period (in number of measurement samples) | 1 sample |  |
| Subcarrier spacing | 15 kHz; 30 kHz |  |
| CSI-RS periodicity | 5 ms |  |
| Carrier frequency offset | Option 1: 972Hz for 15kHz; 1667Hz for 30kHz | Option 2: 1944Hz for 15kHz; 3334Hz for 30kHz |
| Number of PRBs | 48 |  |
| Density | 3 |  |
| Side condition (SNR) on CMR | -3 dB |  |
| Frequency tracking | TRS |  |

Table 2 Updated evaluation assumption for investigation on L1-SINR accuracy in FR1 HST

The evaluation results are provided as below, i.e. Table 3 and Table 4. ‘No-TRS’ is the for the cases without TRS-based frequency tracking, i.e. ideal frequency tracking is assumed, while ‘TRS’ is for the cases where TRS-based frequency tracking is done.

| CSI-RS Ês/Iot (dB) | Delta SINR [dB] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 972Hz |  |  |  |  |  | Option 2 1944Hz |  |  |  |  |  |
|  | No-TRS |  |  | TRS |  |  | No-TRS |  |  | TRS |  |  |
|  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| -5 | -1.25 | 0.66 | 2.36 | -1.25 | 0.69 | 2.37 | -1.52 | 0.51 | 2.24 | -2.31 | 0.00 | 1.81 |
| 0 | -0.67 | 0.37 | 1.43 | -0.62 | 0.43 | 1.48 | -0.92 | 0.20 | 1.28 | -1.55 | -0.37 | 0.77 |
| 5 | -0.49 | 0.25 | 1.05 | -0.42 | 0.32 | 1.11 | -0.74 | 0.04 | 0.86 | -1.36 | -0.58 | 0.27 |
| 10 | -0.45 | 0.20 | 0.90 | -0.36 | 0.30 | 1.00 | -0.76 | -0.08 | 0.61 | -1.54 | -0.87 | -0.14 |
| 15 | -0.53 | 0.12 | 0.82 | -0.36 | 0.30 | 0.99 | -1.04 | -0.38 | 0.30 | -2.23 | -1.59 | -0.92 |
| 20 | -0.82 | -0.11 | 0.66 | -0.40 | 0.33 | 1.16 | -1.83 | -1.16 | -0.45 | -3.88 | -3.30 | -2.67 |
| 25 | -1.58 | -0.77 | 0.14 | -0.48 | 0.43 | 1.60 | -3.64 | -3.01 | -2.30 | -6.83 | -6.39 | -5.89 |
| 30 | -3.28 | -2.45 | -1.48 | -0.68 | 0.67 | 2.69 | -6.76 | -6.26 | -5.72 | -10.87 | -10.59 | -10.27 |

Table 3 Evaluation of L1-SINR accuracy under high doppler when SCS=15kHz

| CSI-RS Ês/Iot (dB) | Delta SINR [dB] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Option 1 1667Hz |  |  |  |  |  | Option 2 3334Hz |  |  |  |  |  |
|  | No-TRS |  |  | TRS |  |  | No-TRS |  |  | TRS |  |  |
|  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| -5 | -1.24 | 0.74 | 2.43 | -1.24 | 0.76 | 2.43 | -1.46 | 0.61 | 2.36 | -1.32 | 0.77 | 2.42 |
| 0 | -0.68 | 0.42 | 1.49 | -0.65 | 0.47 | 1.53 | -0.84 | 0.29 | 1.39 | -0.61 | 0.45 | 1.55 |
| 5 | -0.50 | 0.29 | 1.13 | -0.42 | 0.34 | 1.19 | -0.66 | 0.13 | 0.93 | -0.42 | 0.34 | 1.13 |
| 10 | -0.44 | 0.24 | 0.96 | -0.36 | 0.31 | 1.04 | -0.65 | 0.02 | 0.72 | -0.35 | 0.30 | 1.01 |
| 15 | -0.50 | 0.17 | 0.90 | -0.35 | 0.31 | 1.05 | -0.85 | -0.21 | 0.49 | -0.34 | 0.29 | 1.02 |
| 20 | -0.73 | 0.00 | 0.82 | -0.39 | 0.34 | 1.18 | -1.50 | -0.82 | -0.06 | -0.37 | 0.32 | 1.15 |
| 25 | -1.37 | -0.51 | 0.48 | -0.51 | 0.44 | 1.57 | -3.02 | -2.35 | -1.58 | -0.48 | 0.41 | 1.60 |
| 30 | -2.80 | -1.86 | -0.72 | -0.69 | 0.66 | 2.63 | -5.82 | -5.27 | -4.63 | -0.70 | 0.61 | 2.77 |

Table 4 Evaluation of L1-SINR accuracy under high doppler when SCS=30kHz

Based on the above results, we see in most cases the SINR accuracy can be ensured when SNR is high, e.g. 25dB. The exception is 1944Hz Doppler. Normally TRS-based frequency tracking would be beneficial, but for the case when Doppler is 1944Hz for 15kHz-SCS, TRS-based frequency tracking may not work properly. Significant performance degradation is observed.