3GPP TSG-RAN WG4 Meeting # 103-e 	R4-22xxxxx

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2381 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on unified TCI in R17 feMIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. No known condition defined for common TCI state switch. 2. For DCI-based TCI switch, it should be applicable only if more than one TCI codepoint is activated by MAC CE. 3. For TCI state list update, currently the requirements are only defined for the case all TCIs are known. Need to consider the case some of them are unknown. 4. The applicability of PL-RS, i.e. beam alignment case, should be in the introduction part so that the DCI based and TCI list based may also be clarified. 5. Requrements for PL-RS switched by DCI are missing, and so does the applicablility of DCI based PL-RS switching. 6. UL TCI list update should only be considered for UL related transmission, but not for receiving the DCI. 7. PL-RS list update delay when SSBs from different cells overlaps is not right. 8. Wording on applicability of requirements needs to be improved. 9. Some typos are found. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Defined known condition for common TCI state switch 2. Clarified the applicablity of DCI-based TCI switch 3. Add requirements for TCI state list updates, when some of TCIs are unknown. 4. Moved applicability of PL-RS requirements to 8.16.1 5. Add requirements and applicablity for DCI based TCI switch. 6. Clarified the end point of UL TCI state list updates. 7. Corrected PL-RS list update requirements 8. Clarification on the applicability of requirements 9. Editorial change to correct typos 10. Add latest RRC parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreements are correctly implemented, or the spec is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1, 8.15.2, 8.15.3,8.15.4, 8.15.5, 8.16.1, 8.16.2, 8.16.3, 8.16.4, 8.16.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

8.15.1	Introduction

The requirements in this clause apply for a UE configured with more than one DLorJointTCIState or UL-TCIState configurations for both DL and UL channels/signals on a serving cell or a cell with PCI different from a serving cell on a CC, or serving cells on all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR. UE shall complete the switch of active downlink TCI state within the delay defined in this clause.

The requirements in this clause for a cell with different PCI from serving cell are applicable for such cell is known for a UE. A cell with different PCI from serving cell is known if the the following conditions are met

Active BWP of the serving cell and a cell with different PCI is same

Center frequency, SCS and SFN offset of a cell with different PCI from serving cell are as the same as serving cell

During the last 5s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with different PCI

Timing offset between serving cell and the cell with different PCI from serving cell is within CP of the corresponding SCS

The SSB from the cell with different PCI remains detectable according to the cell identification requirements specified in clause 9.2

Otherwise, the cell with different PCI from serving cell is unknown.

### 8.15.2	Known conditions for downlink TCI state

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command

-	The target downlink TCI state remains detectable during the downlink TCI state switching period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

-	The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

Otherwise, the downlink TCI state is unknown.

For intra-band CA, the common TCI state for all CC in the same list configured by simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 is known if the associated RS in the reference CC is known.

### 8.15.3	MAC-CE based downlink TCI state switch delay

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in DL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.15.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch.

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk = 1 when TCI state switching involves other QCL types only

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

8.15.4	DCI based downlink TCI state switch delay

When a UE is configured with the higher layer parameter with DLorJointTCIState or UL-TCIState, activated with more than one TCI state codepoints for downlink and/or uplink transmission, and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State or TCI state pair in the active TCI list for a CC or all CCs with a common indicated TCI-State in the same CC list configured by simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4., the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication.

If the target TCI state is known, the downlink TCI switching to the indicated DL TCI state or joint TCI state in the DCI format shall be completed starting from the first slot that is at least BeamAppTime-r17 symbols after the last symbol of the PUCCH carrying HARQ-ACK in response to the DCI triggering TCI state activation. The first slot and the BeamAppTime-r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The value of BeamAppTime-r17 is defined in TS 38.331 [2]. The known condition for TCI state defined in clause 8.15.2 is applied.

8.15.5	Active Downlink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘SeparateULDL’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘JointULDL’, and a MAC CE activates more than one target joint TCI.

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH or PDSCH with the new target TCI states at the first slot that is after

n +3Nslotsubframe,µ + (THARQ + TO*(Tfirst-SSB_List + TSSB-proc)) / NR slot length.

Where

-	If any DL TCI is unknown, KD = 1 and TO is TOuk,

-	if all unknown TCI states involve QCL-TypeD, and the source RSs for QCL-TypeD are SSBs, TOuk = 0,

-	otherwise, TOuk = 1

-	If all TCIs are known, TO is TOk,

-	if any target TCI state is not in the active TCI state list, TOk = 1,

-	otherwise, TOk = 0

-	If the number of cells associated with the target TCIs that are not in the active TCI list is larger than 1, and SSBs associated to the TCIs are overlapped,

-	Tfirst-SSB_List = Ncell * Tfirst-SSB, where Ncell is the number of cells associated with the target TCIs that are not in the active TCI list, whose SSBs are overlapped. Ncell ≤ [Nmax] + 1, where [Nmax] is the number of cells with PCI different from serving cell, and [Nmax] = 1.

-	Otherwise,

-	Tfirst-SSB_List = Tfirst-SSB.

-	THARQ, Tfirst-SSB, TSSB-proc are defined in clause 8.15.3.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘JointULDL’, or

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘SeparateULDL’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to receive on DL before UE completes the DL and UL TCI state list update.

<End of Change #1>

<Start of Change #2>

8.16.1	Introduction

The requirements in this clause apply for a UE configured with more than one DLorJointTCIState or UL-TCIState configurations for for both DL and UL channels/signals on a serving cell or a cell with PCI different from a serving cell on a CC, or serving cells on all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR. There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter UL-TCIState associated to SRS. UE shall complete the switch of active uplink TCI state within the delay defined in this clause when the UE is requested to switch to a TCI state with the higher layer parameter DLorJointTCIState or UL-TCIState associated to a DL RS.

PL-RS may be associated with or included in UL TCI state or joint TCI state. The requirements in this clause shall apply if the following conditions are met:

- PL-RS is identical to source RS in UL TCI state or joint TCI state

- PL-RS and source RS in UL TCI state or joint TCI state are QCL-Type D

The requirements in this clause for a cell with different PCI from serving cell are applicable for such cell is known for a UE. A cell with different PCI from serving cell is known if the the following conditions are met

Active BWP of the serving cell and a cell with different PCI is same

Center frequency, SCS and SFN offset of a cell with different PCI from serving cell are as the same as serving cell

During the last 5s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with different PCI

Timing offset between serving cell and the cell with different PCI from serving cell is within CP of the corresponding SCS

The SSB from the cell with different PCI remains detectable according to the cell identification requirements specified in clause 9.2

Otherwise, the cell with different PCI from serving cell is unknown.

### 8.16.2	Known conditions for uplink TCI state

The uplink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target uplink TCI state to the completion of active uplink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target uplink TCI state or QCLed to the target uplink TCI state

-	Uplink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target uplink TCI state before the uplink TCI state switch command

-	The target uplink TCI state remains detectable during the uplink TCI state switching period

-	The SSB associated with the uplink TCI state remain detectable during the uplink TCI switching period

-	SNR of the uplink TCI state ≥ -3dB

-	The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

Otherwise, the uplink TCI state is unknown.

For intra-band CA, the common TCI state for all CC in the same list configured by simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 is known if the associated RS in the reference CC is known.

### 8.16.3	MAC-CE based uplink TCI state switch delay

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in UL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.16.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

    - If target TCI state is known,

- The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms).

    - If target TCI state is unknown,

- The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms.

Where,

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

8.16.4	DCI based uplink TCI state switch delay

When a UE is configured with the higher layer parameter with DLorJointTCIState or UL-TCIState, activated with more than one TCI state codepoints for downlink and/or uplink transmission,  and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State or TCI state pair in the active TCI list  for a CC or all CCs with a common indicated TCI-State in the same CC list configured by simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4, the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication.

If the target TCI state is known, the uplink TCI switching to the indicated UL TCI state or joint TCI state in the DCI format shall be completed starting from the first slot that is at least BeamAppTime-r17 symbols after the last symbol of the PUCCH carrying HARQ-ACK in response to the DCI triggering TCI state activation. The first slot and the BeamAppTime-_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The value of BeamAppTime_r17 is defined in TS 38.331 [2]. The known condition for TCI state defined in clause 8.16.2 is applied.

If a PL-RS is associated with or included in UL TCI state or joint TCI state, the UL TCI switching and PL-RS switching shall be completed at the same time. The requirements in this clause are applicable only if the DCI format indicating UL TCI state or joint TCI state switch is received by UE after the PL-RS is maintained by the UE.

8.16.5	Active Uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘SeparateULDL’, and a MAC CE activatesmore than one target separate TCIs, and at least one UL TCI is included, or

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘JointULDL’, and a MAC CE activates more than one target joint TCI.

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n+ 3Nslotsubframe,µ + (THARQ + NM * (Tfirst_target-PL-RS_List + 4 * Ttarget_PL-RS_List + 2ms))) / NR slot length,

Where

-	If any UL TCI is unknown, KU = 1 and NM is 1.

-	If all TCIs are known,

-	if any target PL-RS associated with or included in any UL TCI is not maintained by the UE, NM = 1,

-	otherwise, NM = 0

-	If the number of cells associated with the target TCIs that are not in the active TCI list is larger than 1, and SSBs associated to the TCIs are overlapped,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS + (Ncell -1)* Ttarget-PL-RS, Ttarget-PL-RS_List = Ncell * Ttarget-PL-RS, where Ncell is the number of cells associated with the target TCIs that are not in the active TCI list, whose SSBs are overlapped. Ncell ≤ [Nmax] + 1, where [Nmax] is the number of cells with PCI different from serving cell, and [Nmax] = 1.

-	Otherwise,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS, Ttarget-PL-RS_List = Ttarget-PL-RS.

-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.16.3.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘JointULDL’, or

-	higher layer configuration ‘unifiedtci-StateType’ is set to ‘SeparateULDL’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL before UE completes the DL and UL TCI list update.

<End of Change #2>