3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2209504

# Electronic Meeting, May 09 – May 20, 2022

Source: 	vivo

Title: 	Discussion on remaining issues in other RRM impacts in R17 feMIMO

Agenda Item:	9.18.2.3

Document for:	Discussion

Introduction

In RAN4 102-e meeting, the RRM impacts from TRP-specific BFR and QCL train definition update achieved conclusions in R4-2206935[1], and related CR is approved in [2].

Based on above information, we provide our view on the remaining issues in R17 feMIMO

Discussion

QCL train update

In RAN4 102-e, the following was agreed.

Agreements in RAN4 #102-e

- Issue: QCL definition

- Agreements

  - In R17, UL TCIs are only applicable to UL signals/channels, and UL RSs cannot be used as source RSs of DL TCIs or joint TCIs.

  - In Rel-17, DL TCI chain and UL TCI chain need to be defined for deriving the QCL information of PDCCH/PDSCH and PUCCH/PUSCH respectively.

In last meeting, whether to include SRS in the UL TCI train definition may need further discussion. As in latest TS 38.331, in uplink TCI state, the configuration of QCL is not applied.

UL-TCIState-r17 ::=                   SEQUENCE {

ul-TCIState-Id-r17                    UL-TCIState-Id-r17,

servingCellId-r17                         ServCellIndex                                         OPTIONAL,   -- Need S

referenceSignal-r17                       CHOICE {

ssb-Index-r17                             SSB-Index,

csi-RS-Index-r17                          NZP-CSI-RS-ResourceId,

srs-r17                                   PUCCH-SRS

},

additionalPCI-r17                     AdditionalPCIIndex-r17                                    OPTIONAL,   -- Need R

ul-powerControl-r17                   Uplink-powerControlId-r17                                 OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17            PUSCH-PathlossReferenceRS-Id                              OPTIONAL    -- Need S

-- Editor's Note: Check if new id -r17 is needed to cover full ID range

}

Therefore, we see no need to update QCL train definition for SRS.

Proposal 1  According to TS 38.331 , SRS should not be included in the QCL train.

TRP specific BFR

In RAN4 102-e, the following was agreed.

Agreements in RAN4 #102-e

- Issue: Requirement for TRP specific Beam Failure Recovery

- Agreements

  - RRM requirements for TRP-specific BFR should be specified for FR1 in R17 and do not introduce sharing factor PTRP in FR1;

  - Introduce PTRP = 2 in FR2 for overlapping resources for equal sharing between BFD/ CBD resources (SSB and CSI-RS) between the 2 TRPs.

- FFS the conditions on applying PTRP = 2 in FR2Agreements

  - In R17, UL TCIs are only applicable to UL signals/channels, and UL RSs cannot be used as source RSs of DL TCIs or joint TCIs.

  - In Rel-17, DL TCI chain and UL TCI chain need to be defined for deriving the QCL information of PDCCH/PDSCH and PUCCH/PUSCH respectively.

Based on agreements achieved in recent meetings, we do not see too many differences between cell specific BFR as in R15/R16, and TRP specific BFR. The main difference is the number of RS sets configured for BFD and CBD, and the sharing factor introduced in FR2. Therefore, we do not see too strong motivation to create a separate section for it. Moreover, if a new section is created, maintenance of it to include some other R17 features, such as FR2 HST, Gap enh., Power saving enh., etc, needs to be considered. Moreover, it seems only BFD requirements has been captured in [2], while CBD, scheduling availability, SCell BFR requirements, etc., are missing. In general, an incomplete requirements defined in separate section would leads to heavy maintenance work to an important feature with very limited impact to legacy requirements.

Therefore, it is proposed to remove current standalone section for TRP-specific BFR, and add corresponding revisions in 8.6 of TS 38.133.

Proposal 2  In TRP-specific BFR, some basic requirements, such as CBD, are needed, and R17 BFD relaxation for UE power saving should be applicable, therefore, TRP-specific BFR requirements are NOT captured in standalone section.

Conclusion

Based on above analysis, we have following observations and proposals.

Proposal 1  According to TS 38.331 , SRS should not be included in the QCL train.

Proposal 2  In TRP-specific BFR, some basic requirements, such as CBD, are needed, and R17 BFD relaxation for UE power saving should be applicable. Therefore, TRP-specific BFR requirements are NOT captured in standalone section.

- References

[1]  R4-2206935	WF on other RRM requirements for FeMIMO, Huawei, HiSilicon.  RAN4 #102-e

[2]  R4-2207123	Big CR: RRM requirements for Rel-17 NR feMIMO, Samsung.  RAN4 #102-e