3GPP TSG-RAN WG4 Meeting #103-e 	       R4-2209539

# E-meeting, 9th – 20th May 2022

Source: 		Nokia, Nokia Shanghai Bell

Title: 	BS requirements for LTE TDD band in 1670-1675 MHz

Agenda Item: 	12.1.3

Document for:	Approval

# Introduction

This document provides the list of expected changes (excluding system parameters) to 36.104 due to introduction of LTE TDD band in 1670-1675 MHz.

# Discussion

Below are (highlighted in yellow) expected changes (excluding system parameters) to 36.104 due to introduction of LTE TDD band in 1670-1675 MHz:

#### Clause 6.6.3.1	Minimum requirements for Wide Area BS (Category A):

For E-UTRA BS operating in Bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 22, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45, 48, 50, 52, 65, 66, 69, 70, 74, 75, 105 emissions shall not exceed the maximum levels specified in Tables 6.6.3.1-4 to 6.6.3.1-6:

#### Clause 6.6.3.3	Additional requirements

The following requirements may also apply to E-UTRA BS operating in Band 105 in certain regions. The level of emissions in the 1541 – 1650 MHz band, measured in measurement bandwidth according to Table 6.6.3.3-13 shall not exceed the maximum emission levels PEM,B105,a, PEM,B105,b, PEM,B105,c, PEM,B105,d, PEM,B105,e and PEM,B105,f declared by the manufacturer.

Table 6.6.3.3-13: Declared Band 105 emissions levels for protection of the 1541-1650 MHz band

| Operating Band | Frequency range | Declared emission level [dBW] (Measurement bandwidth = 1 MHz) | Declared emission level [dBW] of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz) | Declared emission level (dBW) of discrete emissions of less than 2 kHz bandwidth (Measurement bandwidth = 1 kHz) |
| --- | --- | --- | --- | --- |
| 105 | 1541 - 1559 MHz | PEM,B105,a |  | PEM,B105,f |
|  | 1559 - 1610 MHz | PEM,B105,b | PEM,B105,d |  |
|  | 1610 - 1650 MHz | PEM,B105,c | PEM,B105,e |  |

Note:	The regional requirements is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the BS unwanted emission level at the antenna connector, Gant equals the BS antenna gain minus feeder loss. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement.

#### Clause 6.6.4.3	Additional spurious emissions requirements

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 105 | 1670 - 1675 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 105. |

#### Clause 6.6.4.4	Co-location with other base stations

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band 105 | 1670 – 1675 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 105 |
| LA E-UTRA Band 105 | 1670 – 1675 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 105 |
| MR E-UTRA Band 105 | 1670 – 1675 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 105 |

### Clause 7.6.1	General blocking requirement

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52, 65, 66, 68, 70, 105 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 1-7, 9-11, 13-14, 18,19, 21-23, 24, 27, 30, 33-45, 48-53, 65, 66, 68, 70, 105 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52-53, 65, 66, 68, 70, 105 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |

### Clause 7.6.2	Co-location with other base stations

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band 105 | 1670 - 1675 | +16** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 105 | 1670 - 1675 | -6** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 105 | 1670 - 1675 | +8** | PREFSENS + 6dB* | CW carrier |

# Conclusion

It is proposed to agree on proposed changes in this document, draft CR to 36.104 will be provided in the coming meeting.