3GPP TSG-RAN WG4 Meeting #103-e 	       R4-2209541

# E-meeting, 9th – 20th May 2022

Source: 		Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.844: On UE receiver selectivity requirements for combined UE CBW (one cell)

Agenda Item: 	11.2.3.2.2

Document for:	Approval

# Introduction

During the discussion on the Status Report for Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, the following remaining open issue has been included:

Further studies of UE receiver selectivity requirements for proposed methods

This document addresses further details of these requirements for combined UE CBW (one cell).

# Discussion

It should be noted the following bands and irregular channel bandwidths are included in the TR 38.844:

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

Taking into account details above, impacted UE receiver selectivity requirements in TS 38.101-1 are analyzed below for these bands and channel BWs.

Reference sensitivity power level

For the frequency bands of interest for this SI, the following requirements (channel BWs dependent) are defined in TS 38.101-1 tables 7.3.2-1a and 7.3.2-1b:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |
| n85 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

It should be noted for all FDD bands, the reference sensitivity power level requirement is defined for channel BWs which are smaller and larger than the considered irregular channel BWs. Therefore, the reference sensitivity level for these irregular channel BWs can be defined with minimum effort e.g. by using linear interpolation between the signal levels in mW (not in dBm). For band n29, since there is REFSENS formula agreed already, only new channel bandwidths would need to be added to the table.

Maximum input level

The following requirements (channel BWs dependent) are defined in TS 38.101-1 table 7.4-1:

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Power in transmission bandwidth configuration value is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

Minor changes are needed to add new channel BWs to the table above.

Adjacent channel selectivity

The following requirements (channel BWs dependent) are defined in TS 38.101-1 tables 7.5-1, 7.5-3 and 7.5-4:

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |
| Pinterferer4 | dBm | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 5 |  |  |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |

Case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -56.5 | -53.5 | -50.5 + 10log10(BWChannel /20) |
| Pinterferer | dBm | -25 |  |  |
| BWinterferer | MHz | 5 |  |  |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to ￼MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

While for irregular channel BWs below 10MHz, 33dB ACS and Pinterferer = REFSENS + 45.5 dB (case 1) can be re-used; for irregular channel BWs larger than 10MHz, Pinterferer in case 1 can be defined with minimum effort e.g. by using linear interpolation between the interferer powers in mW, by including new channel BWs larger than 10MHz to already agreed 5/10MHz requirement or (although not preferred) by including new channel BWs larger than 10MHz to already agreed 15MHz requirement. Similarly, the requirement in case 2 can be re-used for irregular channel BWs below 10MHz and defined with minimum effort for irregular channel BWs larger than 10MHz e.g. by using linear interpolation between the powers in transmission bandwidth configuration in mW, by including new channel BWs larger than 10MHz to already agreed 5/10MHz requirement or (although not preferred) by including new channel BWs larger than 10MHz to already agreed 15MHz requirement.

In-band blocking

The following requirements (channel BWs dependent) are defined in TS 38.101-1 table 7.6.2-1 (it should be noted there is no impact to table 7.6.2-2 due to irregular channel BWs):

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

While for irregular channel BWs below 10MHz the in-band blocking requirement can be re-used; for irregular channel BWs larger than 10MHz, the in-band blocking requirement can be defined with minimum effort e.g. by using linear interpolation between the powers in transmission bandwidth configuration in mW, by including new channel BWs larger than 10MHz to already agreed 5/10MHz requirement or (although not preferred) by including new channel BWs larger than 10MHz to already agreed 15MHz requirement.

Out-of-band blocking

The following requirements (channel BWs dependent) are defined in TS 38.101-1 table 7.6.3-1 (it should be noted there is no impact to table 7.6.3-2 due to irregular channel BWs):

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

While for irregular channel BWs below 10MHz the out-of-band blocking requirement can be re-used; for irregular channel BWs larger than 10MHz, the out-of-band blocking requirement can be defined with minimum effort e.g. by using linear interpolation between the powers in transmission bandwidth configuration in mW, by including new channel BWs larger than 10MHz to already agreed 5/10MHz requirement or (although not preferred) by including new channel BWs larger than 10MHz to already agreed 15MHz requirement.

Narrow band blocking

For the frequency bands of interest for this SI, the following requirements (channel BWs dependent) are defined in TS 38.101-1 table 7.6.4-1:

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n74, n75, n76, n85, n101 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |  |

The Pw for narrow band blocking requirement can be defined with minimum effort e.g. by using linear interpolation between the wanted signal powers in mW or consider one fixed offset to PREFSENS (e.g. PW = PREFSENS + 16 dB) for all new channel BWs.

Spurious response

The following requirements (channel BWs dependent) are defined in TS 38.101-1 table 7.7-1:

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration value is rounded to the next higher 0.5dB value. |  |  |  |  |

While for irregular channel BWs below 10MHz the spurious response requirement can be re-used; for irregular channel BWs larger than 10MHz, the spurious response requirement can be defined with minimum effort e.g. by using linear interpolation between the powers in transmission bandwidth configuration in mW, by including new channel BWs larger than 10MHz to already agreed 5/10MHz requirement or (although not preferred) by including new channel BWs larger than 10MHz to already agreed 15MHz requirement.

Wide band intermodulation

The following requirements (channel BWs dependent) are defined in TS 38.101-1 table 7.8.2-1:

| Rx parameter |  | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Pw in Transmission Bandwidth Configuration, per CC5 |  | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| PInterferer 1 (CW) |  | dBm |  | -46 |  |  |
| PInterferer 2 (Modulated) |  | dBm |  | -46 |  |  |
| BWInterferer 2 |  | MHz |  | 5 |  |  |
| FInterferer 1 (Offset) |  | MHz |  | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |
| FInterferer 2 (Offset) |  | MHz |  | 2*FInterferer 1 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. NOTE 5: Pw in Transmission Bandwidth Configuration per CC shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |  |

While for irregular channel BWs below 10MHz the wide band intermodulation requirement can be re-used; for irregular channel BWs larger than 10MHz, the wide band intermodulation requirement can be defined with minimum effort e.g. by using linear interpolation between the wanted signal powers in transmission bandwidth configuration in mW, by including new channel BWs larger than 10MHz to already agreed 5/10MHz requirement or (although not preferred) by including new channel BWs larger than 10MHz to already agreed 15MHz requirement.

# TP to TR 38.844

#### 6.7.2	Combined UE CBW (one cell)

Overall impact is limited since both the “main RF carrier” and the “additional RF carrier” would conform to existing 3GPP requirements, to guarantee co-existence. For UE receiver selectivity requirements, the impact is also limited to

- reusing the existing requirement where the requirement for the next narrower and next wider CBW is the same,

- reusing the existing requirement where the requirement for the CBW range around the irregular BW is given by a formula or

- otherwise using linear interpolation between the CBW dependent power levels on a power (i.e. mW rather than dBm) scale or by including new channel BWs to already agreed requirements for 5/10MHz or (although not preferred) 15MHz.