3GPP TSG-RAN WG4 Meeting#103-e                               R4-2209604

E-meeting,9th May – 20th May,2022

Agenda item:	9.5.5.1.3

Source: 	ZTE Corporation

Title: 	Discussions on NR repeater test cases

Document for:	Approval

Introduction

Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we want to provide some initial analysis how to select the test case for NR repeater similar as NR BS.

Discussion

For the legacy LTE repeater, only one RF channel is specified for the conformance testing and its details could be found as following

Most tests in the present document are only performed for a subset of the possible combinations of test conditions. For instance: - Only one RF channel may be specified to be tested; - Not all channel bandwidths may be specified to be tested.

From our understanding, only single RF channel might be not sufficient compared with legacy NR BS conformance testing. Similar as the legacy NR BS conformance testing, we propose the following RF channels for NR repeater conformance testing where similar principle as NR BS is used for NR repeater.

|  | RF channel |  |  |
| --- | --- | --- | --- |
|  | Single carrier | Multi-carrier | Multi-carrier, Multi-band |
| Repeater output power | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Frequency stability Error Vector Magnitude | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Out of band gain | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Unwanted emissions, ACLR, spurious emission | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Input intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Output intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Adjacent Channel Rejection Ratio (ACRR) | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Transmit ON/OFF power | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Note: since there is no carrier related definition in repeater, carrier should be placed by nominal channel bandwidth of pass band. |  |  |  |

Conclusions

In this contribution, we want to provide some initial analysis how to select the test case for NR repeater similar as NR BS and proposals are made as following:

Proposal 1: the same principle is applied for radiated and and conducted.

Proposal 2: propose to adopt the test case in above table for NR repeater conformance testing.

References

- TS 38.106 17.0.0

- TS 36.143 15.0.0

- TS 38.176-1 16.3.0

- TS 38.176-2 16.3.0

- R4-1813878, WF on RF channel for OTA testing, Nokia, approved.