3GPP TSG-RAN WG4 Meeting#103-e                                R4-2209605

E-meeting, May 9 – May 20,2022

Agenda item:	9.5.5.2

Source: 	ZTE Corporation

Title: 	Discussion on FR1 NR repeater test conformance testing

Document for:	Approval

Introduction

In last RAN4 meeting, Core requirement of NR repeater has been completed. Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we provide some initial views on conformance testing requirement for FR1 NR repeater.

Discussion

- 2.1. Test setup for FR1 NR repeater

For the conformance testing for NR repeater, the 1st issue need to be addressed is how to establish the measurement setup for NR repeater conformance testing. Since for FR1 NR repeater type 1-C is similar as LTE repeater or UTRA repeater, therefore we propose the use the measurement setup in TS 36.143 as baseline for FR1 NR repeater as following:

C.1	Maximum output power

Power Meter or equivalent

Repeater
under
test

NR
Signal 
Generator

Figure C.1-1: Measuring system set-up for maximum output power

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.2	Frequency stability

Frequency counter

Repeater
under
test

NR
Signal 
Generator

Figure C.2-1: Measurement system set-up for frequency stability

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.3	Out of band gain

30 dB precision attenuator

Spectrum
 Analyser

Repeater
under 
test

CW 
Signal 
Generator

30 dB precision attenuator

Figure C.3-1: Measuring system set-up for out of band gain

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.4	Unwanted emission: Operating band unwanted emission

Spectrum analyser

Repeater
under
test

NR
Signal 
Generator

Figure C.4-1: Measuring system set-up for unwanted emission: ACLR and Operating band unwanted emission

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.5	Unwanted emission: Spurious emission

Spectrum analyser

Repeater
under
test

NR 
Signal 
Generator

Figure C.5-1: Measuring system set-up for unwanted emission: Spurious emission.

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.6	Modulation Accuracy: Error Vector Magnitude

Signal
 analyser

Repeater
under
test

NR 
Signal 
Generator

Figure C.6-1: Measuring system set-up for error vector magnitude.

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.7	Input intermodulation

CW Generator

Repeater
under
test

Spectrum Analyser

Coupler

CW Generator

Figure C.7-1: Measuring system set-up for input intermodulation.

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.8	Output Intermodulation

Repeater
under
test

Spectrum 
Analyser

NR
Signal Generator
F1

Circulator

NR
Signal Generator
F2

Figure C.8-1: Measuring system set-up for output intermodulation.

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.9	Adjacent Channel Rejection Ratio

Spectrum Analyser

Repeater
under
test

NR 
Signal 
Generator

Figure C.9-1: Measuring system set-up for Adjacent Channel Rejection Ratio

Note that a repeater is a bi-directional device. The signal generator may need protection.

C.10	 Transmit ON/OFF power

Spectrum analyser

Repeater
under
test

NR
Signal 
Generator

Figure C.10-1: Measuring system set-up for transmitter ON/OFF power and transition period

Note that a repeater is a bi-directional device. The signal generator may need protection.

- 2.2. Measurement uncertainty

For the measurement uncertainty for FR1 repeater,we proposed to use the MU of LTE repeater as baseline for further discussion. In addition, it should be noted that the upper frequency range is also increased up to 5GHz. This upper frequency range of repeater should also depend on the discussion on the supported band for repeater. From our understanding, in the existing TS 38.106 spec, NR-U related band n46, n96 and n102 is not supported, therefore the upper frequency limit of FR1 NR repeater should be up to 5GHz (e.g. band n79).

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies | Derivation of Test System Uncertainty |
| --- | --- | --- |
| Output power | ±0,7 dB, f ≤ 3.0 GHz ±1,0 dB, 3.0 GHz < f ≤ [5 GHz] |  |
| Frequency stability | ±12 Hz Measurement results of 500 Hz |  |
| Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ [5 GHz] Calibration of test set-up shall be made without DUT in order to achieve the accuracy |  |
| Operating band unwanted emission (except 9.1.5.4) | 1,5 dB, f ≤ 3.0 GHz ±1,8 dB, 3.0 GHz < f ≤ [5 GHz] The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS38.141-1 [xx] |  |
| Protection of the BS receiver in the operating band | for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB |  |
| Spurious emissions | In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power 9kHz < f 4 GHz ±2,0 dB; 4GHz < f [19 GHz] ±4,0 dB. The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according to TS38.141-1 [xx] |  |
| Error vector magnitude | 1% signal analyser, 2% stimulus signal | Requirement limit shifted by RSS requirement and stimulus signal EVM. Analyser error added to requirement limit. |
| Input intermodulation | ±1,2 dB | Formula: RSS CW1 level error, 2 x CW2 level error, and measurement error (using all errors = ±0,5 dB) |
| Output intermodulation | 2,1 dB operating band unwanted emission The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station | Formula: RSS 2x Interference signal level error and operating band unwanted emission measurement level error. (1 dB interference signal level error is assumed.) |
| Adjacent channel rejection ratio | 0,7 dB |  |
| Transmit OFF power | ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ [5 GHz] (Note) |  |
| Transmitter transient period | NA |  |

- 2.3. Test procedure

Regarding the test procedures for FR1 NR repeater, the existing test procedures for LTE repeater could be used as baseline. In addition, some additional requirements for FR1 NR based repeater should also been taken into account e.g. non-contiguous spectrum or multi-band operation, transmitter ON/OFF power etc. Stimulus signals should be updated as NR based signal e.g. NR-FR1-TM1.1 and ? for IAB-MT.

Conclusions

In this contribution, we provided some initial views on conformance testing requirement for FR1 NR repeater. and proposals are made as following:

Proposal 1: propose to use the measurement setup in TS 36.143 as baseline for FR1 NR repeater;

Proposal 2: propose to use the measurement uncertainty in TS 36.143 as baseline for FR1 NR repeater with upper frequency limit restricted to 5GHz;

Proposal 3: propose to use the existing test procedure of LTE repeater as baseline and take the additional requirements defined for FR1 NR based repeater into account. e.g.non-contiguous spectrum or multi-band operation, transmitter ON/OFF power etc

References

- TS 38.106 17.0.0

- TS 36.143 17.0.0