# 3GPP TSG-RAN WG4 Meeting #103-e   	R4-2209685

# Electronic Meeting, 9th – 20th May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2386 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for applicability of RLM measurement relaxation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-16 IDLE mode power saving, TS38.304 specifies that low mobility criterion must be filfilled for a period of TSearchDeltaP; however, that measurement rule in Rel-17 is missed - The section number of relaxed criteria specified in TS38.331 should be specified - The UE capability and parameter name of relaxed criterion should be specified |  |  |  |  |  |  |  |  |
|  |  | . |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UE should fulfill low mobility criterion defined in 5.7.13.1 of TS 38.331 [2] for a period of TSearchDeltaP-Connected - low mobility criterion defined in 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] are specified - UE capability rlm-Relaxation-r17 and bfd-Relaxation-r17 are specified. In the meanwhile, parameter name of relaxed criterion goodServingCellEvaluationRLM/BFD, SSearchDeltaP-Connected and TSearchDeltaP-Connected are also added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The Rel-17 low mobility criterion and Rel-16 low mobility criterion are not aligned - The section number of relaxed criteria specified in TS38.331 is missed - UE capability and parameter name of relaxed criterion are missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------- Beginning of Change 1 -----------------------------------------------------------

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports rlm-Relaxation-r17 and configured with explicit signaling goodServingCellEvaluationRLM:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion SSearchDeltaP-Connected and TSearchDeltaP-Connected, and UE has fulfilled both low mobility criterion defined in 5.7.13.1 of TS 38.331 [2] for a period of TSearchDeltaP-Connected and good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2].

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion SSearchDeltaP-Connected and TSearchDeltaP-Connected, and UE has fulfilled both low mobility criterion defined in 5.7.13.1 of TS 38.331 [2] for a period of TSearchDeltaP-Connected and good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell.

The UE is allowed to apply the relaxed requirements defined in clause 8.1.2.[4] for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.[4] for CSI-RS based radio link monitoring; otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

The UE is not allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The UE has triggered T310 timer

-------------------------------------------------------- Beginning of Change 2 -----------------------------------------------------------

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

The relaxed beam failure detection procedures specified in this clause apply provided for the UE supports bfd-Relaxation-r17 and configured with dedicated signaling goodServingCellEvaluationBFD:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion SSearchDeltaP-Connected and TSearchDeltaP-Connected, and UE has fulfilled both low mobility criterion defined in 5.7.13.1 of TS 38.331 [2] for a period of TSearchDeltaP-Connected and good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2].

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in SSearchDeltaP-Connected and TSearchDeltaP-Connected, and UE has fulfilled both low mobility criterion defined in 5.7.13.1 of TS 38.331 [2] for a period of TSearchDeltaP-Connected and good serving cell quality criterion defined in 5.7.13.2 of TS 38.331 [2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell.

Note: 	the explicit signalling indicates that the low mobility state of the UE, if the low mobility criteria is not configured

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is allowed to apply the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection; otherwise, UE shall apply the requirements defined in clause 8.5.5 for SSB based beam failure detection and the requirements defined in clause 8.5.3 for CSI-RS based beam failure detection.

The UE is not allowed to apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

The UE sends a beam failure instance indication to the higher layers,

The UE has triggered T310 timer.

------------------------------------------------------------ End of Change ----------------------------------------------------------------