3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2209705

# Electronic Meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	On RedCap general UE demodulation and CSI reporting requirements

Agenda item:	9.19.5.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the UE demodulation and CSI requirements for RedCap topic are captured in the summary [1] and WF [2].

In this contribution we will express our views on the open issues related to configurations for UE demodulation and CSI reporting requirements for RedCap.

Discussion

Configurations for UE demodulation and CSI reporting 	requirements

In WF ‎[2], two options were discussed in regard to support of FDD duplex modes:

UL/DL pattern used for FDD tests for 1Rx UE - Assume Full-duplex FDD to derive the requirements, e.g., SNR to achieve 70% of maximum throughput. - For the requirement specification, - Option 1: Consider Full-duplex FDD only - Option 2: Consider Full-duplex FDD and half-duplex FDD with applicability rule

At RAN4 #102-e, companies agreed to define demodulation and CSI reporting requirements for FD-FDD and for TDD. There was no consensus, whether to also define requirements for HD-FDD and corresponding test cases (with corresponding applicability rule).

Generally, companies argued no impact on demodulation and CSI reporting requirements is expected, as only scheduling of DL and UL transmissions is affected in case of HD-FDD versus FD-FDD. We share this view and hence propose that FD-FDD and TDD requirements, as well as corresponding test cases, are sufficient to be specified.

Moreover HD-FDD is an optional feature, whilst FD-FDD is mandatory for RedCap UE. It is noted the WID [3] contains the objective: “HD-FDD type A with the minimum specification impact.”

Hence, we do not see a need for separate applicability requirements or test cases for HD-FDD and support option 1.

Select Option 1: Consider Full-duplex FDD only.

Conclusion

In this contribution we have provided our views on open issues related to configurations for UE demodulation and CSI reporting requirements for RedCap.

We have made the following proposal:

- Select Option 1: Consider Full-duplex FDD only.

- References

- R4-2207451 Email discussion summary for [102-e][331] NR_RedCap_Demod, Ericsson

- R4-2207206 WF on RedCap demodulation and CQI reporting requirements, Ericsson

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- 3GPP TS 38.101-4 “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 16)”.

- 3GPP TR 38.875, “Study on support of reduced capability NR devices”, V17.0.0 (2021-03).