3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2209709

# Electronic Meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for Redcap PDCCH and PBCH

Agenda item:	9.19.5.2.2

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the UE demodulation and CSI requirements for RedCap topic are captured in the summary [1] and WF [2].

In this contribution, we present a summary of first simulation results for PDCCH and PBCH. Please note that these are first results and might be subject to change.

PDCCH simulation results summary

Simulation results are reported in this section for simulation assumptions contained in the WF [2]. In addition, simulation results for AL16 for FR1 FDD 15 kHz and FR1 TDD 30 kHz are recorded.

Furthermore, the following configurations are used, where appropriate:

| Parameter | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern | FR1.30-1 |  |  |  |
| CCE to REG mapping type | interleaved |  | interleaved |  |
| Interleaver size | 3 |  |  |  |
| REG bundle size | 2 | 2 |  |  |
| Shift Index | 0 |  |  |  |

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | New | 14.6 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | New | 4 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse | 5.5 |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 | Reuse | -0.9 |
| 1Rx UE | 10 | 48 | 2 | 16 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 5 | New | 4.7 |
| 2Rx UE | 10 | 48 | 2 | 16 | TDLA30-10 | 1x2 Low | 1% | 5.3.2.1.1-1 Test 5 | Reuse | -1.6 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | New | 6.3 |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | New | 1.4 |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | New | 1.5 |
| 2Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | New | -2.2 |
| 1Rx UE | 20 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 3 | New | -0.9 |
| 2Rx UE | 20 | 48 | 2 | 16 | TDLC300- 100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 3 | Reuse | -4.3 |

TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 | Reuse | 1.4 |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 | Reuse | -1.3 |

PBCH simulation results summary

Simulation results are reported in this section for simulation assumptions contained in the WF [2].

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New | -5.8 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New | -4.7 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse | -8.4 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse | -9.6 |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) | -8.8 |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) | -10.7 |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse | -8.2 |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse | -9.2 |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse | -8.0 |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse | -8.9 |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |  |

Conclusion

In this contribution we have provided a summary of first PDCCH and PBCH simulation results for agreed scenarios in WF [2].

We have made no proposals or observations.

- References

- R4-2207451 Email discussion summary for [102-e][331] NR_RedCap_Demod, Ericsson

- R4-2207206 WF on RedCap demodulation and CQI reporting requirements, Ericsson