3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2209710

# Electronic Meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RI requirements

Agenda item:	9.19.5.3.2

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the UE demodulation and CSI requirements for RedCap topic are captured in the summary [1] and WF [2].

In this contribution we will express our views on the open issues related to RI reporting requirements.

Discussion

Scope of RI reporting requirements

In WF ‎[2], two options were discussed:

Whether to define RI reporting requirements for RedCap 2Rx UEs - Option 1: Define RI reporting tests for RedCap 2Rx UE. - Option 2: Not to define RI reporting tests for RedCap 2Rx UE.

As commented at RAN4 #102-e, not supporting MIMO operation is not one of the reduced capabilities of the RedCap UE.

MIMO operation is not one of the reduced capabilities of the RedCap UE.

Like PMI reporting, for which performance requirements and corresponding test cases have been agreed at RAN4 #102-e [2], RI is still part of Redcap UE mandatory capabilities, hence RI reporting needs to be tested. We potentially see a strong negative TPUT performance impact if no requirements for RI reporting are specified, or rather such functionality in not implemented to increase spectral efficiency.

We can’t follow the argumentation from the last meeting that 2Rx should not be tested because 1Rx is not tested, as RI reporting requirements for 1Rx UEs do not make sense in the first place.

Thus, option 1 should be selected.

Select Option 1: Define RI reporting tests for RedCap 2Rx UE.

Conclusion

In this contribution we have provided our views on RedCap UE RI reporting requirements.

We have made the following observation and proposal:

- MIMO operation is not one of the reduced capabilities of the RedCap UE.

- Select Option 1: Define RI reporting tests for RedCap 2Rx UE.

- References

- R4-2207451 Email discussion summary for [102-e][331] NR_RedCap_Demod, Ericsson

- R4-2207206 WF on RedCap demodulation and CQI reporting requirements, Ericsson

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- 3GPP TS 38.101-4 “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 16)”.

- 3GPP TR 38.875, “Study on support of reduced capability NR devices”, V17.0.0 (2021-03).