3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2209712

# Electronic Meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on measurement in RRC_INACTIVE state

Agenda item:	9.20.2.3

Document for:	Discussion

# Introduction

RAN4 has progressed the discussion on NR positioning enhancements for the Rel-17 WID [1] at RAN4 #102-e [2] and made following agreements and listed following issues for further study related to measurement in RRC_INACTIVE, as summarized below and depicted in WF [3].

- PRS collision - The value of X regarding collision of other functions - If PRS is outside initial DL BWP. - X=0.5ms if one or both of the serving cell and PFL is in FR1 - X=0.25ms if both the serving cell and PFL are in FR2 - If PRS is within initial DL BWP. - X=0 - FFS: the definition of “PRS resource” for defining the collision between PRS resource and other DL signals/channels. - PRS collision with PDSCH Wait for RAN1 agreement. - PRS measurement requirements applicability in RRC_INACTIVE state - The PRS measurement requirements applicability in RRC_INACTIVE state regarding state transition - If there is state transition from RRC_INACTIVE to RRC_CONNECTED (or vice-versa) during the measurement period: - For RSTD and PRS-RSRP(P) measurement, UE continue the measurement and measurement period can be longer without defining exact requirements. - For UE Rx-Tx measurement, UE shall re-start the measurement after it obtains SRS configuration and TA from the serving cell. - UE behavior for UE Rx-Tx time difference measurement under cell reselection transition - UE shall restart the UE Rx-Tx measurement after the cell reselection and after it obtains SRS configuration and TA from the new serving cell. - PRS measurement requirements applicability under cell reselection - Do not define exact measurement period for RSTD, PRS-RSRP and PRS-RSRPP if the cell reselection occurs during the measurement period. Add a general sentence that the measurement period requirements can be longer. - PRS measurement requirements applicability under cell selection - RAN4 not to define requirements for the case when cell selection is triggered during PRS measurement period. - Include applicability condition in TS 38.133: PRS measurement requirements apply provided that the cell selection procedure for the selected PLMN is not triggered during PRS measurement period. - PRS measurement requirements applicability under DRX change - If during the PRS measurement period the DRX cycle is reconfigured then the PRS measurement period can be longer. - SRS measurement requirements applicability in RRC_INACTIVE state - SRS measurement period requirements in RRC_INACTIVE state - No gNB measurement period requirements are defined in RRC_INACTIVE state. - SRS measurement accuracy requirements in RRC_INACTIVE state - The existing accuracy requirements of gNB Rx-Tx measurement and SRS-RSRP measurement in Rel-16 can be reused in RRC_INACTIVE state (i.e. no update is needed in the sections on gNB measurement accuracy requirements). - Prioritization between SRS transmission for positioning and other UL signals/channels - RAN4 not to discuss prioritization between SRS transmission for positioning and other UL signals/channels (which is RAN1 scope). - Measurement period requirements for positioning measurement in RRC_INACTIVE state - Whether to support the reduced number of samples in RRC_INACTIVE state - Support of the reduced number of samples in RRC_INACTIVE state is UE capability - The UE capability is under discussion in RAN1, and wait for RAN1 agreement. - PRS measurement requirements with reduced number of samples in RRC_INACTIVE state are applicable only for UE which supports PRS measurements with reduced number of samples. - The requirements with reduced sample number are applicable when UE is requested by LMF to perform measurement with reduced sample number. - FFS: PRS measurement requirements with reduced number of samples in RRC_INACTIVE are defined under the same side conditions as agreed for RRC CONNECTED state. - Tavailable_PRS,i calculation for PRS measurement requirements in RRC_INACTIVE state Tavailable_PRS,i is the least common multiple between TPRS and DRX cycle. - Teffct,i calculation for PRS measurement requirements in RRC_INACTIVE state The same approach as R16 can be used. - How to define carrier specific scaling factor (Kcarrier_PRS) for PRS measurement requirements in RRC_INACTIVE state - UE capabilities for PRS measurements in RRC_INACTIVE state - Capability #1: UE not performing parallel PRS measurements (note: this is the default capability for UE supporting PRS measurements in RRC_INACTIVE state) - Capability #2: UE performing parallel PRS measurements - For Capability #1 UEs: - Update the definition of Kcarrier in 4.2.2.4 and Nlayer in 4.2.2.7 by adding one positioning frequency layer. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to Nlayer + 1. - If Srxlev ? SnonIntraSearchP or Squal ? SnonIntraSearchQ, Kcarrier_PRS equals to Kcarrier +1. - For Capability #2 UEs: - Do not update the definition of Kcarrier in 4.2.2.4 and Nlayer in 4.2.2.7. - Kcarrier_PRS equals to 1. - Working assumption for the PRS measurement requirements in RRC_INACTIVE state • Keep the existing R16 framework and the working assumption is not needed. - PRS processing procedure for the PRS measurement requirements in RRC_INACTIVE state Keep the existing R16 framework for PRS processing procedure.

This contribution treats the above depicted open RAN4 issues. Furthermore, RAN4 has also received a LS from RAN1 [4] in reply to our LS [5] on applicability of PRS processing window in RRC_INACTIVE.

# Discussion

Above open issues from RAN4#102-e are discussed in this section based on [2] and [3].

## PRS measurement requirements

### 2.1.1 	Definiton of PRS resource for the collision case

- RAN4 #102-e discussed how to define the term “PRS resource” for identifying the collision between PRS resource and other DL signals/channels [2]. The following dependencies were mentioned to determine the PRS resource length for determining the overlap with other DL signals/channels according to the agreement for PRS collision in section 1:

- the length of the PRS resource, i.e. between start and end of the PRS resocurce, includes measurement time and processing time of the PRS symbols of the same PRS occasion,

- the length of the PRS resource takes into account the PRS processing type (slot level, symbol level),

- the length of the PRS resource includes expectedRSTD-uncertainty.

- The following proposal is made.

The length of the PRS resource for determining the overlap with other DL signals/channels, includes measurement time and processing time of the PRS symbols of the same PRS occasion, takes into account the PRS processing type (slot level, symbol level) and includes expectedRSTD-uncertainty.

### 2.1.2 		PRS collision with PDSCH

- RAN4 #102-e discussed this matter based on following options:

Candidate options: - Option 1: - UE is not expected to receive PDSCH on PRS resources or on symbols for RF-retuning within Y symbols after a DCI. - Option 2: - PDSCH receiving is high priority than PRS in RRC_INACTIVE state and the PRS measurement period can be extended when colliding with PDSCH. - Option 3: - Wait for RAN1 agreement.

- RAN4 then concluded to wait for RAN1 agreement.

- It is noted RAN1 #108-e concluded the issue and endorsed a TP to TS 38.214 in R1-2202523:

- Thus, TS 38.133 should specify the priority of PDSCH reception over PRS according to option 2.

- The following proposal is made.

PDSCH reception has higher priority than PRS reception in RRC_INACTIVE state and the PRS measurement period can be extended in case PRS is collides with PDSCH.

### 2.1.3 	PRS measurement period requirements

- RAN4 #102-e discussed whether PRS measurement requirements with reduced number of samples in RRC_INACTIVE are defined under the same side conditions as agreed for RRC CONNECTED state.

- In our view, the work on reduced number of PRS samples should follow the work for RRC_CONNECTED state. The support of reduced PRS samples should be bound to UE capability. In our view, a UE that supports reduced number of PRS samples in RRC_CONNECTED state should also support the feature in RRC_INACTIVE state, if it supports positioning measurement in RRC_INACTIVE state.

- At RAN4 #102-e it was discussed whether condition #1, used for latency reduction in RRC_CONNECTED to define the condition for reduced number of AGC samples for PRS measurements, can be reused for RRC_INACTIVE.

  - Condition #1:

    - PRS bandwidth is within the active BWP and

    - Difference between the serving cell SS-RSRP and neighbor cell/TRP PRS-RSRP is within [6] dB

- While it is true that active BWP for RRC_CONNECTED needs to be mapped to initial BWP for RRC_INACTIVE, as raised during RAN4 #102-e, accuracy requirements in RRC_INACTIVE can be based on the same side conditions, i.e. PRS is covered by the initial BWP and difference between serving and neighbour cells SS-RSRP is within 6 dB, the latter depending on deployments rather than UE’s RRC state. In our understanding performance for the same PRS bandwidths will be specified as for RRC_CONNECTED.

- Thus, to ensure balanced performance, the side conditions should be aligned to RRC_CONNECTED state as the conditions should be same and not depend on UE’s RRC state.

- The following proposal is made.

A UE that supports reduced number of PRS samples in RRC_CONNECTED state should also support the feature in RRC_INACTIVE state, if it supports positioning measurement in RRC_INACTIVE state. To ensure balanced performance, side conditions in RRC_INACTIVE should be aligned to RRC_CONNECTED state.

### 2.1.4 	Applicability of PRS processing window in RRC_INACTIVE

- An LS from RAN1 [4] is received at RAN4 #103-e. RAN4 had asked if the PRS processing window defined for PRS measurement outside of measurement gap can be applied for PRS measurement for RRC_INACTIVE state. RAN1 decided the PRS processing window is not supported in RRC_INACTIVE state in Rel-17.

- RAN4 #101bis-e discussed the open issue how to calculate Tavailable_PRS,i capturing following options in [6].

Candidate options: - Option 1: (CATT, CMCC, OPPO, Ericsson) - Tavailable_PRS,i could be the least common multiple between TPRS and DRX cycle. - Option 2: (Huawei) - Tavailable_PRS,i for Inactive state measurement is defined as the LCM of Tprs, measurement window periodicity and DRX cycle.

As RAN1 confirmed in their reply [4] that the PRS processing window is not supported in RRC_INACTIVE, option 1 selected at RAN4 #102-e is confirmed.

- The following proposal is made.

RAN4 to keep the definition for Tavailable_PRS,i as the least common multiple between TPRS and DRX cycle for RRC_INACTIVE.

# Conclusion

In this contribution, we provide our view on open issues related to positioning measurement support for RRC_INACTIVE state. Following proposals are made:

- The length of the PRS resource for determining the overlap with other DL signals/channels, includes measurement time and processing time of the PRS symbols of the same PRS occasion, takes into account the PRS processing type (slot level, symbol level) and includes expectedRSTD-uncertainty.

- PDSCH reception has higher priority than PRS reception in RRC_INACTIVE state and the PRS measurement period can be extended in case PRS is collides with PDSCH.

- A UE that supports reduced number of PRS samples in RRC_CONNECTED state should also support the feature in RRC_INACTIVE state, if it supports positioning measurement in RRC_INACTIVE state. To ensure balanced performance, side conditions in RRC_INACTIVE should be aligned to RRC_CONNECTED state.

- RAN4 to keep the definition for Tavailable_PRS,i as the least common multiple between TPRS and DRX cycle for RRC_INACTIVE.

# References

[1]	RP-210903, “Revised WID on NR Positioning Enhancements”, CATT, Intel Corporation, Ericsson

[2]	R4-2207072, “Email discussion summary for [102-e][223] NR_pos_enh_2”, Moderator (CATT)

[3]	R4-2206997, “WF on NR Positioning Enhancements (Part 2)”, CATT, RAN4#102-e

[4]	R1-2202619, “Reply LS on the applicability of PRS processing window in RRC_INACTIVE state”, RAN1

[5] 	R4-2202687, “LS on the applicability of PRS processing window in RRC_INACTIVE state”, RAN4

[6]	R4-2202684, “WF on NR Positioning Enhancements (Part 2)”, CATT, RAN4#101bis-e