3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209776

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.19.3.2

Source:	Apple

Title:	Discussion on RRM requirement with eDRX for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement with eDRX for RedCap has been discussed and the conclusions were captured in the WF[1].

We further discuss the RRM requirement with eDRX for RedCap in this contribution.

# eDRX requirements for inactive state

The remaining issues from last meeting are as below,

Issue 1-4-1: Inactive state requirements when idle eDRX is longer than 10.24s - Option 1: The inactive UE requirements are based on inactive DRX or inactive eDRX when inactive eDRX is configured (Huawei vivo MTK) - Option 2: Based on the paging monitoring cycle of T agreed in RAN2 (Apple Ericsson xiaomi Nokia) IDLE eDRX[s]; Inactive eDRX[s]; Outside CN PTW or during CN PTW; T. >10.24; Not configured; During CN PTW; Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. >10.24; Not configured; Outside CN PTW; RAN paging cycle.. >10.24; ≤10.24; During CN PTW; Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. >10.24; ≤10.24; Outside CN PTW; INACTIVE eDRX cycle. Agreement: no consensus Issue 1-4-2: Inactive state requirements when idle eDRX is no longer than 10.24s - Option 1: The inactive UE requirements are based on inactive eDRX or inactive DRX when inactive eDRX is not configured (Huawei vivo MTK) - Option 2: Based on the paging monitoring cycle of T agreed in RAN2 (Apple Ericsson xiaomi vivo Nokia) IDLE eDRX[s]; Inactive eDRX[s]; Outside CN PTW or during CN PTW; T. ≤10.24; Not configured; NA; Shortest of RAN paging cycle and IDLE eDRX cycle. ≤10.24; ≤10.24; NA; The shortest of IDLE eDRX cycle and INACTIVE eDRX cycle.. Agreement: No consensus, FFS at maintenance stage

Based on conclusion in RAN2 and SA2, in Inactive mode UE can only use eDRX up to 10.24s and no PTW shall be considered in this case.

RAN2 had new agreements in #117e meeting:

- The invalid configuration where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured, is captured in the field description of the parameter ran-ExtendedPagingCycle.

- The invalid configuration where INACTIVE eDRX cycle is longer than IDLE eDRX cycle, is captured in the field description of the parameter ran-ExtendedPagingCycle.

- In system information broadcast introduce 1 bit to indicate whether eDRX is allowed for UEs in RRC_IDLE and RRC_Inactive. Come back to this (and potentially introduce 2 bits) if a separate UE capability will be available

- UE considers RRC_IDLE eDRX cycle for comparing with the modification period for both RRC_IDLE and RRC_INACTIVE to decide if eDRX acquisition period is used. Capture this in TS 38.331 as:

1> if the UE is in RRC_IDLE, configured with an eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set:

- Rel-17 eDRX for RRC_IDLE UEs is captured in TS38.306 as optional feature without capability signalling

- inactiveStatePO-Determination-r17 introduced in R2-2111586 covers eDRX scenario, and no new UE capability is needed. A UE supports eDRX shall also support inactiveStatePO-Determination-r17.

RAN2 had following agreement in #116e meeting:

- When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX.

- When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX.

- During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.

- During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.

Agreements online:

- For the eDRX PTW start calculation, agree to N=8. No signalling needed to CN.

- The eDRX acquisition period is the same for IDLE and INACTIVE.

  - A)	For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

  - B)	For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

RAN2 had following agreement in #115e meeting:

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is no longer than 10.24s, outside CN PTW, T is determined by INACTIVE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle.

Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle.

Option 2: T is determined by RAN paging cycle.

We still think UE could follow PO monitoring periodicity T to perform the RRM measurement in Inactive mode, and at the same time UE anyway needs to perform T/F tracking before each PO. That’s the simplest way for UE to follow for both PO monitoring and RRM measurement.

Table 1. Summary of RAN2 agreement for paging monitoring cycle T

| IDLE eDRX[s] | Inactive eDRX[s] | Outside CN PTW or during CN PTW | T |
| --- | --- | --- | --- |
| >10.24 | Not configured | During CN PTW | Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer |
| >10.24 | Not configured | Outside CN PTW | RAN paging cycle. |
| >10.24 | ≤10.24 | During CN PTW | Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer |
| >10.24 | ≤10.24 | Outside CN PTW | INACTIVE eDRX cycle |
| ≤10.24 | Not configured | NA | Shortest of RAN paging cycle and IDLE eDRX cycle |
| ≤10.24 | ≤10.24 | NA | The shortest of IDLE eDRX cycle and INACTIVE eDRX cycle. |

The measurement cycle for inactive mode requirement with eDRX shall be specified based on the RAN2 agreements summarized in table 1.

Proposal 1: the measurement cycle for inactive mode requirement with eDRX shall be specified based on the paging monitoring cycle of T from RAN2 agreements summarized in table 1.

# Conclusion

We discuss the RRM requirement with eDRX for RedCap in this contribution.

Proposal 1: the measurement cycle for inactive mode requirement with eDRX shall be specified based on the paging monitoring cycle of T from RAN2 agreements summarized in table 1.

Table 1. Summary of RAN2 agreement for paging monitoring cycle T

| IDLE eDRX[s] | Inactive eDRX[s] | Outside CN PTW or during CN PTW | T |
| --- | --- | --- | --- |
| >10.24 | Not configured | During CN PTW | Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer |
| >10.24 | Not configured | Outside CN PTW | RAN paging cycle. |
| >10.24 | ≤10.24 | During CN PTW | Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer |
| >10.24 | ≤10.24 | Outside CN PTW | INACTIVE eDRX cycle |
| ≤10.24 | Not configured | NA | Shortest of RAN paging cycle and IDLE eDRX cycle |
| ≤10.24 | ≤10.24 | NA | The shortest of IDLE eDRX cycle and INACTIVE eDRX cycle. |

# References

[1] R4-2207105, WF on eDRX and RRM measurement relaxations requirements for Redcap UE, vivo, RAN4 #102-e