3GPP TSG-RAN WG4 # 103-e	R4-2209793

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.11.2.2

Source:	MediaTek Inc.

Title:	Simulation results for MMSE-IRC receiver with intra-cell interference

Document for:	Discussion

- Introduction

In RAN4#102-e, the detail for the simulation assumptions for MMSE-IRC receiver under the scenario of intra-cell inter-user interference was agreed [1]. We provide simulation results in this contribution.

- Simulation Results

Based on the agreed assumptions in Table 1 [1] and same scrambling ID for both target UE and co-scheduled UE, we show our simulation results in Table 2.

Table 1: Simulation assumptions for MU-MIMO

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD and 40/30 for TDD |  |
| Duplex mode |  |  | FDD and TDD |  |
| TDD pattern |  |  | 7D1S2U S=6D+4G+4U |  |
| MCS |  |  | 13 | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R ULA Low and 2T4R ULA Low for case with rank1+1 and 4T4R ULA Low for case with rank 2+2 |  |
| Number of CDM groups without data |  |  | 1 for case with rank 1+1 and 2 for case with rank 2+2 |  |
| HARQ process number |  |  | 4 for FDD and 8 for TDD |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of Co-scheduled UE is not equal to any column of precoding matrix of Target UE - For case with rank 2+2: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | For rank 1+1: TDLC300-100 For rank 2+2: TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

Table 2: Summary of simulation results

|  | Parameters |  |  |  |  |  | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | #CDM | #Tx | #Rx | MCS | Propagation channel | PMI | MMSE-IRC |
| FDD | 1 | 2 | 2 | 13 | TDLC300-100 | Rand | 15.6 |
|  |  | 2 | 4 | 13 | TDLC300-100 | Rand | 9.1 |
|  | 2 | 4 | 4 | 13 | TDLA30-10 | Orthog | 13.0 |
| TDD | 1 | 2 | 2 | 13 | TDLC300-100 | Rand | 16.0 |
|  |  | 2 | 4 | 13 | TDLC300-100 | Rand | 9.4 |
|  | 2 | 4 | 4 | 13 | TDLA30-10 | Orthog | 13.2 |

- Reference

- R4-2207244, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon