3GPP TSG-RAN WG4 # 103-e	R4-2209799

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.19.5.3.1

Source:	MediaTek Inc.

Title:	Simulation results and discussion on the CQI requirements for RedCap

Document for:	Discussion

- Introduction

In RAN4#102-e, a way forward on RedCap was agreed [1]. Based on the agreed simulation assumptions, we provide simulation results on CQI requirements for RedCap UE to determine SNR test points.

- Discussion

2.1 CQI reporting in static channel

We provide simulation results for CQI reporting in static channel in Table 2 based on the assumptions in Table 1.

Table 1. Simulation assumptions for static channel in FDD

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 |

Table 2. Simulation results for CQI reporting in static channel

|  | 1Rx | 2Rx |
| --- | --- | --- |
| SNR [dB] | Median CQI | Median CQI |
| -2 | 2 | 3 |
| -1 | 4 | 4 |
| 0 | 4 | 4 |
| 1 | 5 | 5 |
| 2 | 5 | 5 |
| 3 | 6 | 6 |
| 4 | 6 | 6 |
| 5 | 7 | 7 |
| 6 | 7 | 7 |
| 7 | 8 | 8 |
| 8 | 8 | 8 |
| 9 | 9 | 9 |
| 10 | 10 | 9 |
| 11 | 10 | 10 |
| 12 | 10 | 10 |
| 13 | 11 | 11 |
| 14 | 12 | 11 |
| 15 | 12 | 12 |
| 16 | 12 | 12 |
| 17 | 13 | 13 |
| 18 | 13 | 13 |
| 19 | 14 | 14 |
| 20 | 14 | 14 |

In Rel-15 CQI requirements, RAN4 sets SNR test points as SNR = 8/9 and SNR=14/15dB for 2Rx, 2 layers case. We suggest reusing the same SNR test points for 2Rx RedCap UE. In Rel-15, there is a 3dB SNR test point shift when the number of Rx doubled but the same number of layers.  However, unlike Re-15, there is no such relationship for the cases in Table 1. Hence, for 1Rx RedCap UE, we suggest setting the same SNR test points as 2Rx RedCap UE, i.e., SNR = 8/9 and SNR=14/15dB.

Proposal 1: For static channel CQI reporting test with CQI table 1, we suggest setting SNR=8/9dB and SNR=14/15dB for both 1Rx and 2Rx cases.

2.1 CQI reporting in fading channel

We provide simulation results for CQI reporting in fading channel in Table 4 based on the assumptions in Table 3.

Table 3. Simulation assumptions for fading channel in FDD

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high |

Table 4. Simulation results for CQI reporting in fading channel

|  | 1Rx UE |  |  | 2Rx UE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | TP ratio with following CQI and Median CQI | BLER with following CQI | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | TP ratio with following CQI and Median CQI | BLER with following CQI |
| 0 | 53 | 1.42 | 0.09 | 54 | 1.40 | 0.03 |
| 2 | 53 | 1.27 | 0.1 | 47 | 1.30 | 0.03 |
| 4 | 44 | 1.20 | 0.12 | 46 | 1.09 | 0.09 |
| 6 | 44 | 1.22 | 0.13 | 50 | 1.08 | 0.09 |
| 8 | 44 | 1.21 | 0.11 | 47 | 1.09 | 0.1 |
| 10 | 44 | 1.19 | 0.11 | 45 | 1.18 | 0.09 |
| 12 | 42 | 1.21 | 0.13 | 43 | 1.15 | 0.09 |
| 14 | 37 | 1.27 | 0.11 | 40 | 1.10 | 0.1 |
| 16 | 38 | 1.22 | 0.12 | 64 | 1.11 | 0.24 |
| 18 | 32 | 1.12 | 0.23 | 67 | 1.24 | 0.33 |

For 2Rx case, we suggest reusing the same SNR test points in Rel-15, e.g., SNR=6/7dB and SNR=12/13dB. As for 1 Rx case, we can set 3dB higher SNR test points, which are SNR=9/10dB and 15/16dB.

Proposal 2: For wideband CQI reporting test in fading channel with CQI table 1, we suggest setting SNR=9/10dB and SNR=15/16dB for 1Rx case and SNR=6/7dB and SNR=12/13dB for 2Rx case.

- Conclusion

In this contribution, we provide simulation results on CQI requirements for RedCap based on the agreed simulation assumption in [1]. The proposals are summarized as below:

Proposal 1: For static channel CQI reporting test with CQI table 1, we suggest setting SNR=8/9dB and SNR=14/15dB for both 1Rx and 2Rx cases.

Proposal 2: For wideband CQI reporting test in fading channel with CQI table 1, we suggest setting SNR=9/10dB and SNR=15/16dB for 1Rx case and SNR=6/7dB and SNR=12/13dB for 2Rx case.

- Reference

- R4-2207206, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson