3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2209816

# Electronic Meeting, May.9th – 20th, 2022

Title: 	Simulation results for CRS-IM with 15kHz SCS

Source: 	Huawei, HiSilicon

Agenda item:	9.11.2.3.2

Document for:	Discussion

Background

As per [1], RAN 4 agreed to introduced CRS-IM receiver requirements for both scenario 1 and scenario2. In this paper, we provide our simulation results and views on issues about test setup on scenario 1.

Discussions

The simulation results for both scenario 1 and scenario 2 are captured in clause 2.1 and clause 2.2.

Scenario 1:

Figure 2-1:  BLER- SNR curve for scenario 1

Table 2-1: Summary of simulation results for scenario 1

| CRS port | Duplex mode | Antenna configuration | SNR (dB)@ 70% of max TP |  | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LLR weighting | Baseline |  |
| 2 CRS pots | FDD | 2T2R | 7.2 | 9.3 | 2.1 |
|  |  | 2T4R | 4.3 | 6.4 | 2.1 |
|  | TDD | 2T2R | 7.0 | 9.2 | 2.2 |
|  |  | 2T4R | 4.1 | 6.2 | 2.1 |
| 4 CRS ports | FDD | 4T2R | 7.5 | 9.6 | 2.1 |
|  |  | 4T4R | 4.3 | 6.4 | 2.1 |
|  | TDD | 4T2R | 9.1 | 10.6 | 1.5 |
|  |  | 4T4R | 5.6 | 7.4 | 1.8 |

In the last meeting, we agreed to configure CRS port for scenario based on performance gain of CRS-IM. Based on the simulation summary, both case with 2 CRS ports and 4 CRS ports have similar performance gain for FDD. However, gain of case with 4 CRS ports is smaller than case with 2 CRS ports for TDD. Hence, we prefer to configure 2 CRS ports for scenario 1.

Proposal 1: Only consider 2 CRS ports for scenario 1.

  - Scenario 2

Figure 2-2: Simulation results for scenario 2

Table 2-2: Summary of simulation results for scenario 2

| CRS port | Duplex mode | Antenna configuration | SNR (dB)@ 70% of max TP (By using LLR weighting) |
| --- | --- | --- | --- |
| 2 CRS pots | FDD | 2T2R | 7.7 |
|  |  | 2T4R | 4.3 |
|  | TDD | 2T2R | 8.4 |
|  |  | 2T4R | 5.1 |

Reference

[1]   R4-2207239  WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver.