3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209819

# Electronic Meeting,  May.9th - 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: Introduction of CRS-IM receiver for 30kHz SCS TDD scenario 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN 4 agreed to introduce CRS-IM receiver for 30kHz SCS TDD scenario if obvious gain can be observed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requirements for CRS-IM receiver for 30kHz SCS TDD scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.2.2.1.19 and 5.2.3.1.19 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4. |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.2.2.1.19	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.2.1.19-4 and Table 5.2.2.1.19-5, with the addition of test parameters in Table 5.2.2.1.19-2 and 5.2.2.1.19-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.19-1.

Table 5.2.2.1.19-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1,2-1 |

Table 5.2.2.1.19-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.19-3: Tests parameter for interference cells

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| INR |  | dB | 10.45 | 6.2 |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 20 | 20 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 10% probability of occurrence of LTE data transmission | 10% probability of occurrence of LTE data transmission |
| Time offset |  | us | 1.5 | -0.5 |
| Frequency offset |  | Hz | 300 | -100 |
| TDD UL-DL pattern |  |  | DDDSU S=10D2S2U | DDDSU S=10D2S2U |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Note 1: The channel for interference cells and serving cell are independent. |  |  |  |  |

The requirements for UE supporting CRS information assistance information on LTE bandwidth are specified in Table 5.2.2.1.19-4. In this requirements, tested UE has knowledge about Interference cell#1 and Interference cell#2

Table 5.2.2.1.19-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.6 TDD | 20 / 30 | 16QAM 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

The requirements for UE not supporting CRS information assistance information on LTE bandwidth are specified in Table 5.2.2.1.19-5 with following test procedure:

TBD

Table 5.2.2.1.19-5: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.6 TDD | 40 / 30 | 16QAM 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

5.2.3.1.19	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.3.1.19-4 and Table 5.2.3.1.19-5, with the addition of test parameters in Table 5.2.3.1.19-2 and 5.2.3.1.19-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.19-1.

Table 5.2.2.1.19-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1,2-1 |

Table 5.2.2.1.19-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.19-3: Tests parameter for interference cells

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| INR |  | dB | 10.45 | 6.2 |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 | 10 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 20% probability of occurrence of LTE data transmission | 20% probability of occurrence of LTE data transmission |
| Time offset |  | us | 1.5 | -0.5 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Note 1: The channel for interference cells and serving cell are independent. |  |  |  |  |

The requirements for UE supporting CRS information assistance information on LTE bandwidth are specified in Table 5.2.3.1.19-4:

Table 5.2.3.1.19-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.6 TDD | 40 / 30 | 16QAM 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |

The requirements for UE not supporting CRS information assistance information on LTE bandwidth are specified in Table 5.2.3.1.19-5 with following test procedure:

TBD

Table 5.2.3.1.19-5: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.6 TDD | 40 / 30 | 16QAM 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |

<End of Change 1>