3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209822

# Electronic Meeting, May. 9th  – 20th, 2022

Title: 	Discussion on CQI requirements with MMSE-IRC receiver for inter-cell interference

Source: 	Huawei, HiSilicon

Agenda item:	9.12.2.1.2

Document for:	Discussion

Background

As per [1], we provide our views on inter-cell IRC CQI open issues

Discussions

Test setup methodology for signal power

- Option 1: Define test based on SINR - Option 2: Define test based on SNR

Considering we have two cases that case 1 has interference and noise and case 2 has only noise with the condition that power of interference plus noise of case 1 equals to noise of case 2. SNR methodology will make a confusion that only power of noise is same.

Proposal 1: Define test based on SINR

SINR for 2 Rx

- Option 1: -2 dB

- Option 2: 0dB or -1 dB

Based on our simulation results, the TP gain for IRC reporting is obvious when SNR is -2dB, hence we prefer option 1 to keep align with LTE.

Proposal 2: Use SINR=-2dB for 2RX

T-put gain for 2 Rx

- Option 1: 2

- Option 2: 1.5

Based on our simulation all options can feasible, we prefer option 1 to keep align with LTE.

Proposal 3: Use 2 for T-put gain for 2RX

SINR for 4 Rx

- Option 1: -2 dB

- Option 2: SINR requirement for 4Rx should be lower than that for 2Rx

We prefer option 1 to keep align with option 1

Proposal 4: Use SINR=-2dB for 4RX

T-put gain for 4 Rx

- Option 1: 2.5

- Option 2: 2

- Option 3: 3

- Option 4: 1.5

The gain depends on SNR point. Our simulation results show option 2 and 4 are feasible if SNR=-2dB is used. Hence, we prefer option 2 or option 4.

Proposal 5: Use 2 or 1.5 as T-put gain for 4RX

Conclusion

In this paper, we provide our simulation results for inter cell IRC CQI requirements. The proposals are:

Proposal 1: Define test based on SINR

Proposal 2: Use SINR=-2dB for 2RX

Proposal 3: Use 2 for T-put gain for 2RX

Proposal 4: Use SINR=-2dB for 4RX

Proposal 5: Use 2 or 1.5 as T-put gain for 4RX

Reference

- R4-2207243, WF on CSI requirements for inter-cell interference MMSE-IRC. Ericsson