3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209834

# Electronic Meeting, May.9th –20th, 2022

Title: 	Discussion and simulation results for RedCap PDCCH and PBCH

Source: 	Huawei, HiSilicon

Agenda item:       9.19.5.2.2

Document for:	Discussion

Background

As per [1], in this paper, we provide our simulation results for PDCCH and PBCH requirements. Additionally, we share our views on PDCCH aggregation levels for test.

Simulation results

PDCCH requirements

The PDCCH BLER-SNR curve are shown in Figure 2-1. Summary of simulation results are shown in Table 2-1~Table 2-3. Simulation assumptions are shown in Table 5-1 and 5-2 in Appendix.

Figure 2-1: BLER-SNR curve for PDCCH

Table 2-1: Summary of simulation results for FDD 15kHz SCS FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | Ideal SNR@1% of BLER | Impairment SNR@1% of BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | 12.8 | 14.8 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | 2.5 | 4.5 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | Reuse | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | Reuse | Reuse |

Table 2-2: Summary of simulation results for TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | Ideal SNR@1% of BLER | Impairment SNR@1% of BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 5.8 | 7.8 |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | -0.1 | 1.9 |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 0.3 | 2.3 |
| 2Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | -0.1 | 1.9 |

Table 2-3: Summary of simulation results for TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | Ideal SNR@1% of BLER | Impairment SNR@1% of BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 7.3.2.2.1-1 Test 1-2 (R.PDCCH. 5-1.2 TDD) | Reuse | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 7.3.2.2.2-1 Test 2-1 (R.PDCCH.5-1.3 TDD) | Reuse | Reuse |

PBCH requirements

The PBCH BLER-SNR curve are shown in Figure 2-2. Summary of simulation results are shown in Table 2-4.

Figure 2-2: BLER-SNR curve for PBCH

Table 2-4: Summary of simulation results for PBCH

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | Ideal SNR@1% of BLER | Impairment SNR@1% of BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | -4.1 | -2.1 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | -4.1 | -2.1 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse | Reuse |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) | Reuse (*1) |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) | Reuse (*1) |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse | Reuse |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |  |

Discussions

Only one open issue on aggregation level is left:

Aggregation level(s) - Option 1: Specify PDCCH demodulation requirements with AL4 and AL8. - Option 2: Specify PDCCH demodulation requirements with AL4 or AL8.

Based on our understanding, to balance the test number and test coverage, we prefer option 1 and use AL4 for case with 1TX and use AL8 for case with 2TX

Proposal 1: Use AL4 for case with 1TX and use AL8 for case with 2TX.

Conclusion

In this contribution, we provide simulation results and share our view on open issues for PDCCH AL. The proposal is:

Proposal 1: Use AL4 for case with 1TX and use AL8 for case with 2TX.

Appendix

Table 5-1: Simulation assumptions for FDD PDCCH

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |

Table 5-2: Simulation assumptions for TDD PDCCH

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 2 |  |  |
| Shift Index |  | 0 |  |  |  |

Reference

[1]   REV_ R4-2207206 WF RedCap UE demod. Ericsson