3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209848

# Electronic Meeting, May.9th –20th, 2022

Title: 	Discussions on PRACH requirements for FR2-2

Source: 	Huawei, HiSilicon

Agenda item:       9.15.10.3.3

Document for:	Discussion

Background

As per [1], RAN 4 agreed to define PRACH performance requirements for FR2-2 part. In this paper, we provide our views on test setup.

Issue 2-5-1: Scope of PRACH requirements Define the following PRACH performance requirements: - False alarm probability requirements - PRACH miss detection requirements Issue 2-5-2: SCS PRACH SCS: - 120, 480 kHz - FFS 960 kHz Issue 2-5-3: Sequence length PRACH sequence length: - 120 kHz: - 1151, 571, 139 - 480 kHz: - 571, 139 - 960 kHz: - 139 Issue 2-5-4: PRACH formats PRACH formats: A2, B4, C2 Issue 2-5-6: Frequency offset - 7100 Hz Issue 2-5-7: Time error tolerance Time error tolerance: - 0.07us for AWGN - FFS for multi-path fading Issue 2-5-8: (Ncs, logical sequence index, v) - Consider the following PRACH parameters as starting point: Ncs = 69, logical sequence index = 0, v =0 as starting point - Other options are not precluded

Discussions

SCS

As discussed in PUSCH part, we don’t think 960 kHz is practical in the real network. Hence, we propose not to define PRACH requirements with 960 kHz SCS.

Proposal 1: Only define PRACH performance requirements with 960kHz SCS.

Time error tolerance

Test metric from Rel-15 can be reused. As for time tolerance, procedure of Rel-15 can be reused. i.e. Time error tolerance for TDLA30-300=Time error tolerance for AWGN + maximum delay for TDLA30-300. It is noted that the last path was ignored in Rel-15 PRACH requirements definition and we propose to follow this way. For Time error tolerance for AWGN, values in Table 11.4.2.2-1 can be used as baseline and scaling can be used for 480 kHz SCS. However, the value depends on RMS time delay. We propose to use TDLA5-200. Therefore, we propose to use assumptions in Table 2:

Table 2-1: Time error tolerance for AWGN and TDLA30-300

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-300 |
| A2, B4, C2 | 120 | 0.07 us | 0.10 us |
|  | 480 | 0.02 us | 0.05 us |

Proposal 2: Use TDLA5-200 as propagation conditions and use parameters in Table 2-1 as time error tolerance

Conclusion

In this paper we provide our views on PRACH requirements. The proposals are:

Proposal 1: Only define PRACH performance requirements with 960kHz SCS.

Proposal 2: Use TDLA5-200 as propagation conditions and use parameters in Table 2-1 as time error tolerance

Reference

[1]   R4-2207223 WF on demodulation performance requirements definition for 52.6 - 71 GHz. Intel