3GPP TSG-RAN WG4 Meeting #103-e	R4-2209867

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on PRACH demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	9.8.4.2.3

Document for:	Information

Background

During RAN#102-e meeting, WF [2] on FR2 HST BS demodulation was approved. In this contribution, we provide our updated simulation results for the PRACH demodulation requirements for FR2 HST.

Simulations

Here we provide updated simulation results for FR2 HST PRACH for alignment, as shown in Table 2-2, based on simulation assumption as shown in Table 2-1.

Table 2-1 Simulation assumption for PRACH requirement

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Time error tolerance | 0.07us |
| Timing offset | 4.8us with TO range 0:0.48:4.8 |

Table 2-2 Simulation results for FR2 HST PRACH

| Case number | Timing offset | SNR @ 1% BLER [dB] |  |
| --- | --- | --- | --- |
|  |  | Ideal | Impairment |
| 1 | [0,0.48...4.8] | -14.07 | -11.57 |

Conclusion

In this contribution, we provide our updated simulation results for PRACH performance requirements for NR HST FR2.

Reference

R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101-e, Samsung

R4-2207235, WF on BS demodulation requirement for FR2 HST, RAN4#102-e, Nokia, Nokia Shanghai Bell