3GPP TSG-RAN WG4 Meeting #103-e	R4-2209871

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on 1024QAM PDSCH

Source: 	Huawei, HiSilicon

Agenda item:	9.6.4.1

Document for:	Information

Background

During RAN4#102-e meeting, Way forward [1] for NR DL1024QAM demodulation and CQI reporting requirements was approved. In this contribution, we provide our updated simulation results about the 1024QAM PDSCH demodulation requirements.

Simulation

The simulation assumption is derived as following Table 2-1 and the updated simulation results is shown in Figure 2-1 and Table 2-2.

Table 2-1 Simulation assumption for 1024QAM PDSCH

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

Figure 2-1 Ideal simulation results for 1024QAM PDSCH

Table 2-2 Ideal simulation results for 1024QAM PDSCH

| Case number | Duplex | Rx number | SNR@70%maximum throughput [dB] |
| --- | --- | --- | --- |
| 1 | FDD | 2 | 25.56 |
| 2 | FDD | 4 | 22.05 |
| 3 | TDD | 2 | 25.46 |
| 4 | TDD | 4 | 21.98 |

Conclusion

In this contribution, we provide updated simulation results on UE PDSCH demodulation requirements for 1024QAM.

Reference

R4-2207254, Way forward for NR DL1024QAM demodulation and CQI reporting requirements, RAN4#102-e, Ericsson