3GPP TSG-RAN WG4 Meeting #103-e	R4-2209872

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Discussion and simulation results on 1024QAM CQI

Source: 	Huawei, HiSilicon

Agenda item:	9.6.4.3

Document for:	Discussion

Background

During RAN4#102-e meeting, Way forward [1] for NR DL1024QAM demodulation and CQI reporting requirements was approved. In this contribution, we share our views about the 1024QAM CQI reporting requirements.

Discussion

CQI reporting tests

- Discuss the final CQI definition requirements with CQI table 4 considering the impairment margin and SNR levels provided by TE with the assumption of Tx EVM 2.5%. - 2Rx UE: - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 28-30dB for 2Rx UE without impairment margin. - Option 1: SNR = 28/29dB - Option 2: SNR = 29/30dB - 4Rx UE: - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 26-28dB for 4Rx UE without impairment margin. - Option 1: SNR = 25/26dB - Option 2: SNR = 26/27dB

The CQI reporting evaluation results is shown in following Table 2.1-1.

Table 2.1-1 Ideal simulation results for CQI reporting for AWGN channel

|  | 2Rx |  |  |  |  | 4Rx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 |
| 23 | 13 | 100.00% | 0.00% | 0.04% | 100.00% | 14 | 100.00% | 0.00% | 0.90% | 100.00% |
| 24 | 13 | 100.00% | 0.00% | 0.00% | 100.00% | 14 | 100.00% | 0.00% | 0.00% | 100.00% |
| 25 | 13 | 100.00% | 0.00% | 0.00% | 100.00% | 14 | 100.00% | 0.00% | 0.00% | 100.00% |
| 26 | 14 | 100.00% | 0.00% | 0.61% | 100.00% | 15 | 100.00% | 0.00% | 4.31% | - |
| 27 | 14 | 100.00% | 0.00% | 0.00% | 100.00% | 15 | 100.00% | 0.00% | 0.00% | - |
| 28 | 14 | 100.00% | 0.00% | 0.00% | 100.00% | 15 | 100.00% | 0.00% | 0.00% | - |
| 29 | 15 | 100.00% | 0.00% | 4.41% | - | 15 | 100.00% | 0.00% | 0.00% | - |
| 30 | 15 | 100.00% | 0.00% | 0.00 | - | 15 | 100.00% | 0.00% | 0.00% | - |

In our view, the CQI reporting case should be defined only when at least 1024QAM can be reported during the test. From the above evaluation results, we propose to select rank1 with 28/29dB for 2Rx and 25/26dB for 4Rx for CQI reporting case. However, impairment margin is not considered in the simulation results, so we cannot ensure 1024QAM can be reported for the real test at the SNR point that 1024QAM can be reported for the ideal simulation. So the impairment margin should be considered for UE implementation similar as Rel-16 DL FR2 256QAM WI did. The specific value can be selected based on method in our contribution [2] for 1024QAM PDSCH, e.g. 6dB for 2Rx and 4dB for 4Rx.

Select rank1 with 28/29dB for 2Rx and 25/26dB for 4Rx for CQI reporting case. In addition, the extra 6dB margin for 2Rx and 4dB margin for 4Rx should be considered for UE implementation similar as Rel-16 DL FR2 256QAM WI did.

In addition, to reduce the test effort, applicability rule can be considered. That means if UE has pass the 1024QAM CQI cases, UE can skip the Test 2 of the Rel-15 defined cases (14/15 dB). With this applicability rule, we are also OK to add only 3dB margin for all cases since the duplicated testing can be avoided, even though 1024QAM may not be tested.

Define applicability rule that if UE has pass the 1024QAM CQI cases, UE can skip the Test 2 of the Rel-15 defined cases (14/15 dB).

Proposals

In this contribution, we discuss on UE CQI reporting requirements for 1024QAM. Our observations and proposals are:

- Select rank1 with 28/29dB for 2Rx and 25/26dB for 4Rx for CQI reporting case. In addition, the extra 6dB margin for 2Rx and 4dB margin for 4Rx should be considered for UE implementation similar as Rel-16 DL FR2 256QAM WI did.

Define applicability rule that if UE has pass the 1024QAM CQI cases, UE can skip the Test 2 of the Rel-15 defined cases (14/15 dB).

Reference

R4-2207254, Way forward for NR DL1024QAM demodulation and CQI reporting requirements, RAN4#102-e, Ericsson

R4-2205748, Discussion and simulation results on 1024QAM PDSCH, RAN4#102-e, Huawei, HiSilicon