3GPP TSG-RAN WG4 Meeting #103-e	R4-2209884

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on BS coverage enhancement demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	9.17.2.1

Document for:	Information

Background

During RAN4#102-e meeting, WF [1] on PUSCH demodulation performance of Rel-17 NR coverage enhancement was approved. In this contribution, we provide our initial simulation results about the BS coverage enhancement PUSCH demodulation requirements.

Simulation

PUSCH TB over Multi Slots (TBoMS)

Here we provide the initial simulation results for PUSCH TBoMS for alignment, as shown in Table 2.1-3 and Figure 2.1-1, based on simulation assumption as shown in Table 2.1-1 and Table 2.1-2.

Table 2.1-1 Simulation assumption for PUSCH TBoMS (part 1)

| Parameter | Value |
| --- | --- |
| Physical/available slots | 2/4/8 for FDD, 2 for TDD |
| Repetition number | N/A |
| PRB number | 5 |
| Inter-slot frequency hopping | Disabled |
| Duplex/SCS | FDD 15kHz and TDD 30kHz |
| TDD UL-DL pattern | 7D1S2U, S=6D:4G:4U for TDD 30kHz |
| Transform precoding | CP-OFDM |
| MCS | 2/4 |
| RV sequence | [0 2 3 1] |
| PUSCH mapping type | B |
| Antenna configuration | 1T2R |
| Propagation condition | TDLB100-400 Low |
| Test metric | 70% of maximum throughput |

Table 2.1-2 Simulation assumption for PUSCH TBoMS (part 2)

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | Start symbol | 0 |
|  | Allocation length | 14 |
| Code block group based PUSCH transmission |  | Disabled |

Table 2.1-3 Simulation results for PUSCH TBoMS

| Case Number | Duplex | Repetition number | MCS | SNR @70% maximum throughput |
| --- | --- | --- | --- | --- |
| 1 | FDD | 2 | 2 | -7.18 |
| 2 | FDD | 2 | 4 | -5.29 |
| 3 | FDD | 4 | 2 | -9.64 |
| 4 | FDD | 4 | 4 | -7.93 |
| 5 | FDD | 8 | 2 | -12.40 |
| 6 | FDD | 8 | 4 | -11.00 |
| 7 | TDD | 2 | 2 | -7.19 |
| 8 | TDD | 2 | 4 | -5.26 |

Figure 2.1-1 Simulation results for PUSCH TBoMS

PUSCH demodulation with Joint Channel Estimation (JCE)

Here we provide the initial simulation results for PUSCH JCE for alignment, as shown in Table 2.2-3 and Figure 2.2-1, based on simulation assumption as shown in Table 2.2-1 and Table 2.2-2.

Table 2.2-1 Simulation assumption for PUSCH JCE (part 1)

| Parameter | Value |
| --- | --- |
| Actual TDW length | 2/4/8 for FDD, 2 for TDD |
| Configured TDW number | same as the aTDW length |
| PUSCH repetition type | A |
| PUSCH repetition number | Same as aTDW length |
| PRB number | Full applicable test bandwidth |
| Inter-slot frequency hopping | Disabled |
| TDD UL-DL pattern | 7D1S2U, S=6D:4G:4U for TDD 30kHz |
| Transform precoding | CP-OFDM |
| MCS | 4 |
| Propagation condition | TDLA30-10Low |
| Antenna configuration | 1T2R |
| PUSCH mapping type | B |
| Additional DM-RS position | 1+0, 1+1 |
| Phase and power offset modelling | N/A |
| Test metric | 70% of maximum throughput |

Table 2.2-2 Simulation assumption for PUSCH JCE (part 2)

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 for TDD 0, 0, 0, 0 for FDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | Start symbol | 0 |
| Resource allocation | Allocation length | 14 |
| Code block group based PUSCH transmission |  | Disabled |

Table 2.2-3 Simulation results for PUSCH JCE

| Case Number | Duplex | Repetition number | DMRS | SNR@ 70% maximum throughput |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | JCE on | JCE off | Diff |
| 1 | FDD | 2 | 1+0 | -6.08 | -5.59 | 0.49 |
| 2 | FDD | 2 | 1+1 | -6.50 | -6.21 | 0.29 |
| 3 | FDD | 4 | 1+0 | -8.71 | -7.58 | 1.13 |
| 4 | FDD | 4 | 1+1 | -9.11 | -8.36 | 0.75 |
| 5 | FDD | 8 | 1+0 | -11.20 | -9.45 | 1.75 |
| 6 | FDD | 8 | 1+1 | -11.60 | -10.40 | 1.20 |
| 7 | TDD | 2 | 1+0 | -5.66 | -5.21 | 0.45 |
| 8 | TDD | 2 | 1+1 | -5.96 | -5.68 | 0.28 |

Figure 2.2-1 Simulation results for PUSCH JCE

Conclusion

In this contribution, we provide our initial simulation results on BS PUSCH demodulation requirements for coverage enhancement.

Reference

R4-2207210, WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, RAN4#102-e, China Telecom