3GPP TSG-RAN WG4 Meeting #103-e	R4-2209892

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Discussion on UE FeMIMO CSI FeTypeII PS codebook

Source: 	Huawei, HiSilicon

Agenda item:	9.18.4.3.2

Document for:	Discussion

Background

During RAN4#102-e meeting, WF [1] on general and CSI requirement for Rel-17 FeMIMO was approved. In this contribution, we share our views about the UE FeMIMO CSI FeTypeII PS codebook requirements.

Discussion

Whether to define PMI requirement for Rel-17 FeType II PS codebook

- Option 1 : Yes - Option 1a: Define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook. - Option 1b: Consider defining PMI requirement for Rel-17 eType II port selection only if RAN4 can reach an agreement on a simplified way of testing with SU-MIMO test set-up, otherwise not to define requirement. - Option 2 : No - Companies are encouraged to provide the detail test setup and test metric , also including gNB implementation model in the next meeting

For enhancements on CSI, new codebook is designed based on FDD angle and delay reciprocity. The codebook structure is derived from Rel-16 eType II port selection codebook, i.e. W=W1W2WfH, in which W1∈NP×K1 is port selection matrix, W2∈CK1×Mv is a linear combination coefficient matrix, Wf∈CN3×Mv is DFT based compression matrix. The gNB estimate angle and delay information from the UL reference signal based on angle and delay reciprocity, and then beamforming different CSI-RS port with corresponding angle and delay information respectively to make UE to obtain coefficients according to the specific delay position. The new port selection codebook can reduce the measurement complexity and feedback overhead, at the same time some limitations such as number of selected ports, minimum frequency granularity can be relaxed. In our view, the beamforming operation is related to gNB implementation and greatly affect the accuracy of the precoding matrix. However, as companies proposed in last meeting, by explicitly modelling beam-formed CSI-RS port, the UE behaviour as the rest part of the FeType II port selection codebook scheduling can also be verified. Therefore, we are also OK to define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook.

Define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook.

General Test setup of PMI reporting requirement

- Option 1 : Both SU-MIMO and MU-MIMO - Option 2 : SU-MIMO

In Rel-16 eMIMO WI, this issue has been discussed. From our understanding, MU-MIMO is mainly for gNB related behavior. From demodulation point of view, we don’t see any UE behaviour difference for SU-MIMO and MU-MIMO with the default receiver assumption. Therefore, to align with Rel-16 eMIMO performance requirements, we propose to only consider SU-MIMO setup if PMI reporting requirement for eType II port selection is introduced.

Only consider SU-MIMO setup if PMI reporting requirement for FeTypeII port selection is introduced.

Modelling BF CSI-RS Port

- Option 1 - Option 1a: MIMO fading channel as Rel-13 LTE Class B K=1 PMI test cases - Option 1b: Power scaling method similar as Rel-13 LTE Class B K>1 CRI test case - Other options are not precluded - Apply option 1 as a starting point for initial evaluation in next meeting

From our understanding, the test purpose is to verify whether UE can correctly report the proper codebook. Either one beamforming model can be selected as long as the gNB implementation impact to UE is fixed. To make the test setup simpler, we prefer to select Option 1a if PMI reporting requirement for FeTypeII port selection is introduced.

Select Option 1a if PMI reporting requirement for FeTypeII port selection is introduced.

Test configuration

Here we discuss the test configuration if FeTypeII PS codebook PMI reporting case is introduced.

Codebook parameter configurations (paramCombination)

For Rel-17 FeTypeII codebook, there are different combinations of M, α and β, corresponding to the number of frequency domain basis, the number of selected CSI-RS ports and the number of nonzero coefficients. For M=1 that is the basic feature of FeTypeII, the Wf is assumed with all-one vector and not required to reported. To ensure UE correctly reporting Wf, also considering the test coverage, we propose to define two cases, {M=1, α=1, β=1} (i.e. configuration 4 in TS 38.214 Table 5.2.2.2.7-1) and {M=2, α=1/2, β=1/2} (i.e. configuration 5 in TS 38.214 Table 5.2.2.2.7-1).

Number of CSI-RS ports

For the number of CSI-RS ports, 16Tx can be starting point that is same as Rel-16 eTypeII PMI cases.

Rank & MCS

For the rank and MCS, same configuration can be reused from the Rel-16 eTypeII PMI cases, i.e. MCS20 and rank2 can be selected for Rel-17 FeTypeII PS codebook PMI reporting case.

Overview

For other parameters such as CSI-RS resource Type, reportConfigType, test metric, it is reasonable to reuse from Rel-16 eTypeII PMI cases. Over all, the test configuration is proposed as following Table 2.4-1.

Table 2.4-1 Test configuration for FeTypeII PMI reporting case

| Parameter | Value |
| --- | --- |
| Codebook parameter configurations (paramCombination) | {M=1, α=1, β=1} (i.e. configuration 4 in TS 38.214 Table 5.2.2.2.7-1) and {M=2, α=1/2, β=1/2} (i.e. configuration 5 in TS 38.214 Table 5.2.2.2.7-1) |
| R (numberOfPMISubbandsPerCQISubband) | 1 |
| Number of CSI-RS ports | 16 |
| Rank & MCS | MCS20 and rank2 |
| CSI-RS resource Type | Aperiodic |
| ReportConfigType | Aperiodic |
| Test metric | “FeTypeII follow” vs “TypeI single panel random” |
| Others | Same as Rel-16 eTypeII codebook PMI reporting case |

Use the test configuration in Table 2.4.1 for FeTypeII PMI reporting case.

Simulation

Here we provide the initial simulation results for UE FeMIMO CSI FeTypeII PS codebook.

Table 3.1 Ideal simulation results for UE FeMIMO CSI FeTypeII PS codebook

(TBD)

Proposals

In this contribution, we discuss on UE FeMIMO CSI FeTypeII PS codebook. Our observations and proposals are:

- Define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook.

Only consider SU-MIMO setup if PMI reporting requirement for FeTypeII port selection is introduced.

Select Option 1a if PMI reporting requirement for FeTypeII port selection is introduced.

Use the test configuration in Table 2.4.1 for FeTypeII PMI reporting case.

| Parameter | Value |
| --- | --- |
| Codebook parameter configurations (paramCombination) | {M=1, α=1, β=1} (i.e. configuration 4 in TS 38.214 Table 5.2.2.2.7-1) and {M=2, α=1/2, β=1/2} (i.e. configuration 5 in TS 38.214 Table 5.2.2.2.7-1) |
| R (numberOfPMISubbandsPerCQISubband) | 1 |
| Number of CSI-RS ports | 16 |
| Rank & MCS | MCS20 and rank2 |
| CSI-RS resource Type | Aperiodic |
| ReportConfigType | Aperiodic |
| Test metric | “FeTypeII follow” vs “TypeI single panel random” |
| Others | Same as Rel-16 eTypeII codebook PMI reporting case |

Reference

R4-2207209, WF on CSI requirement for Rel-17 FeMIMO, RAN4#102-e, Samsung