3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2209950

Electronic Meeting, May 9th – 20th, 2022

Title:	TP to TR 38.717-04-01: Addition of CA_n1-n8-n40-n78

Source:	Nokia, OPTUS

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-04-01 to add CA_n1A-n8A-n40A-n78A

************************************* Start of TP*****************************************

## 5.x	CA_n1-n8-n40-n78

### 5.x.1	Operating bands for CA

Table 5.x.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1A-n8A-n40A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

## 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported bandwidths per CA band combination for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n8A-n40A-n78A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.x.3	∆TIB,c and ∆RIB,c values

For CA_n1-n8-n40-n78 the DTIB,c and DRIB,c  values are shown in table 5.x.3-1 and  table 5.x.3-2, respectively. Values are re-used from DC_1-8_n40-n78 in 38.101-3.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1A-n8A-n40A-n78A | n1 | 0.5 |
|  | n8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1A-n8A-n40A-n78A | n1 | 0.2 |
|  | n8 | 0 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 5.x.4	REFSENS requirements

MSD requirements are captured in lower order combinations.

************************************* End of TP*****************************************