3GPP TSG-RAN4 Meeting # 103-e	R4-2210017

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR OBUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters_core |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When referencing the repeater power for the medium range OBUE requirements the equation Prated,x = Prated,c,AC – 10*log10(NTXU,countedpercell) is used carrier power is still referenced and scaling which is not used in repeater as requirements are type 1-C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The term Prated,c,AC is equivalent to the power limit term used in table 6.2.1-1 which is modified by the term X = 10*log (ceil (passband bandwidth/[20MHz])) The correct equation is hence: Prated,x = Prated,out_AC - 10*log (ceil (passband bandwidth/[20MHz])) Note. By subtracting the BW scaling from the rated power is the same as adding it to the limits as is done in table 6.2.1-1 but the way the table heading keep the same power limit numbers are used to e.g. 31dBm and 38dBm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of change>

##### 6.5.3.2.3	Minimum requirements for Medium Range repeater type 1-C (Category A and B) for DL

For Medium Range repeater type 1-C for DL, minimum requirements are specified in table 6.5.3.2.3-1 and table 6.5.3.2.3-2.

For the tables in this clause for repeater type 1-C, Prated,x = Prated,out_AC - 10*log (ceil (passband bandwidth/20MHz))

Table 6.5.3.2.3-1: Medium Range repeater type 1-C operating band unwanted emission minimum requirements, 31< Prated,x  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,x-53dB-75f_offsetMHz-0.05dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,x - 60dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) (Note 3) | 100 kHz |
| NOTE 1: For a repeater type 1-C DL supporting non-contiguous spectrum operation within any operating band the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be Min(Prated,x -60dB, -25dBm)/100kHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.5.3.2.3-2: Medium Range repeater type 1-C operating band unwanted emission minimum requirements, Prated,x  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 3) | 100 kHz |
| NOTE 1: For a repeater type 1-C DL supporting non-contiguous spectrum operation within any operating band the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -29dBm/100kHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

< end of change >