# 3GPP TSG-RAN WG4 Meeting # 103-e						R4-2210036

# Electronic Meeting, 9th  May –20th May, 2022

Source:		Ericsson

Title:			Initial considerations on SAN conformance testing - radiated requirements

Agenda item:		9.12.4.3

Document for:		Discussion

# Introduction

The core RF requirements for NTN SAN are in TS 38.108 and it is time to reflect on corresponding conformance testing specifications. In this document we discuss a few aspects related to radiated requirements .

# Discussion

## Test setup

Under the assumptions of considering NTN RU, feederlink, GW and non-NTN gNB a single entity, as proposed in [1], there is a concern about the number of equipment to be present inside the test chamber, especially if we envision measurement of TRP in reverberation chamber. For this reason we might want to agree on a test setup involving only NTN RU presence in the test chamber, while the other components are outside and connected by cable. This aspect is even more important in case of testing under extreme conditions, where there will be a significant limitation in the size of DUT that is possible to insert in temperature controlled enclosures, eventually offering vacuum conditions as well.

## Measurement uncertainty

Existing measurement uncertainties in TS38.141-2 (see clause 4.1.2) are also acceptable for SAN Type 1-O.

# Conclusions

In this contribution, we present our view on the SAN Type 1-O conformance testing.

Proposal 1: Only NTN RU to be placed in the testing chamber.

Proposal 2: Adopt the same measurement uncertainties for SAN Type 1-O as for NR BS Type 1-O

# References

- R4-2111460 On the Rx Parameters and Rx Testing Setup for NTN gNB, Thales