3GPP TSG-RAN WG4 Meeting #103-e	R4-2210052

Electronic Meeting, May 9 – 20, 2022

Source	: Nokia, Nokia Shanghai Bell

Title	: Remaining issues on unified TCI switching delay requirement

Agenda item	: 9.18.2.1

Document for	: Discussion

Introduction

RAN4#102 has declared completion of Rel-17 feMIMO WI with RRM core requirements. The requirements include RRM core requirements of items below :

- TCI switching delay requirements under Rel-17 unified TCI framework

- L1 measurement requirements for inter-cell beam management

- Others : BFD/CBD/BFRQ  and QCL

It still remains significant issues; we see statements from incomplete discussions with brackets in the CRs regarding Rel-17 TCI switching and L1 measurements, and some issues may cause system mal-operations or significant latency. In this contribution, we discussed remaining issues on TCI switching delay requirements in this contribution. In RAN4#102-e, RAN4 achieved agreements as

PL-RS related agreement in RAN4#102 Issue 1-1-1 Beam alignment assumption if PL-RS is included in UL TCI or joint TCI - PL-RS is identical to the source RS in UL or joint TCI Issue 1-1-2 Beam alignment assumption if PL-RS is associated with UL TCI or joint TCI - PL-RS and source RS in UL or joint TCI are QCL-Type D Issue 1-2-1 Requirement applicability of DCI based DL and UL TCI state switching delay - target TCI state is known - Note: UL TCI known state will be further clarified - target TCI state is in active TCI state list for DL and joint TCI switch - FFS: target TCI state is maintained for UL and joint TCI state switch. Issue 1-2-3 MAC CE based UL TCI state switch when UL TCI switch and PL-RS switch are activated in different MAC CE - No separate PL-RS switching supported in Rel-17. PL-RS switching delay will be the same as UL TCI state switch delay. Issue 1-2-5 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 - Proposals - Option 1(Huawei): - When a SSB is indicated as PL-RS in a UL TCI state, the scaling factor for beam sweeping needs to be introduced for PL-RS measurement time in FR2. - In FR2, the MAC-CE based UL TCI state switching delay need to be separately defined for SSB based PL-RS. - In FR2, when a SSB is indicated as PL-RS in a UL TCI state, the MAC-CE based UL TCI state switching delay for both known case and unknown case can be defined as: - THARQ + 3ms + NM*(5*TL1-RSRP_SSB + 2ms) with the assumption of M=1. - Where NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise. - Option 1a(Apple): The delay is - TFirstSSB + 39*TSSB - Option 2(Ericsson): - TL1-RSRP_SSB as (M*P)*TSSB - Option 3(Nokia, Samsung): Define generic requirement - Option 4: Further clarification is needed

Discussion

In RAN4#102, many companies assumes that path-loss maintenance is the condition of ‘active’ TCI state for UL. In fact, we cannot find any definition of ‘active’ TCI for UL or related behaviors in RAN1 and RAN2 spec yet, while active DL TCI definition and behaviors are found in TS38.133, TS38.214 and TS38.331. So, we assume ‘active’ TCI state is new concept for UL in Rel-17. Especially, we see still unclarity on how to maintain the target UL TCI state as FFS bullets in email discussions and multiple CRs, we further discuss on UL TCI switching in this contribution.

RAN4#102 WF R4-2206943 Issue 1-2-1 Requirement applicability of DCI based DL and UL TCI state switching delay - target TCI state is known - Note: UL TCI known state will be further clarified - target TCI state is in active TCI state list for DL and joint TCI switch - FFS: target TCI state is maintained for UL and joint TCI state switch.

Issue in ‘active’ TCI state for UL

For DL MAC-CE and DCI switching, active TCI state list has been used to trigger TCI switching with data scheduling. If a TCI state is active in DL, the UE is supposed to conduct time and frequency tracking on the source RS. Such DL MAC-CE active state has been defined in 133. We capture below to compare DL and UL switching statements as currently captured in latest Rel-17 version of 38.133.

CR R4-2206945 TS38.133 8.15.3 MAC-CE based downlink TCI state switch delay If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. … TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise. 8.16.3 MAC-CE based uplink TCI state switch delay For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell, - If target TCI state is known, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms). - If target TCI state is unknown, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms. NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise

A problem of the current spec is found related to ‘active’ TCI state for UL. The UE behavior is not clear regarding how a UE manages active UL TCI comparing to active DL TCI. For DL case, if target TCI state is in the active TCI state list for PDSCH/PDCCH, the UE should track time and frequency sync on the resource RS. When DL TCI is indicated, the UE can immediately demodulate the PDCCH/PDSCH based on the sync information as captured below. In RAN1/2 spec, DL case clearly specifies maxNumberActiveTCI-PerBWP in tci-statePDSCH, and a UE should track time and frequency sync.

tci-StatePDSCH TS38.331 - maxNumberActiveTCI-PerBWP indicates the maximum number of activated TCI-states per BWP per CC, including control and data. If a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. The UE shall include this field. Note the UE is required to track only the active TCI states.

For UL TCI transmission, RAN4 has mainly discussed path-loss measurement latency. However, we note firstly that fundamental condition for active UL TCI has been missing, a UE should keep time and frequency sync to the source RS in the UL TCI for UL transmission like the DL case. A UE cannot transmit PUCCH/PUSCH/SRS independently from DL sync. Putting TA adjustment discussion aside, maxNumberActiveTCI-PerBWP under tci-StatePDSCH must be about activated TCI-states per BWP per CC with UE synchronization for both DL and UL.

Observation 1 : Fundamental conditions for active TCI for UL are missing. There is no definition of active TCI for UL in RAN1/2 specs. Also, a UE should keep time and frequency sync tracking on the source RS in UL TCI to be capable of UL transmission.

Observation 2 : maxNumberActiveTCI-PerBWP under tci-StatePDSCH should be about activated TCI-states with UE synchronization for both DL and UL. The current spec addresses about DL only.

Proposal 1 : Rel-17 active UL TCI state should be under time and frequency tracking. This means that active UL TCI list belongs to active DL TCI state list.

- Add the time and frequency tracking condition to the active TCI state for UL.

If Proposal-1 is agreeable, there are two ways to correct it in the spec. One is to add the time and frequency tracking condition to the known UL TCI state (CR R4-2206945), and another way is to consider actual tracking delay in 8.16.3 MAC-CE based uplink TCI state switch delay (CR R4-2206945).

However, this tracking behavior does not resolve the problem in active UL-TCI list completely. As mentioned, active UL TCI state has an additional condition of PL-RS maintenance than active DL TCI state.  In fact, the active UL TCI state with PL-RS measurement maintenance is not written yet in any of RAN1/2 spec, now RAN4 has directly to discuss it in this meeting. RAN1/2 has only active TCI state for tci-statePDSCH, and it is up to 8 active TCI state can be configured up to UE capability.

Also, a UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions.

TS38.213 Chapter 7. Uplink Power control A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in subcaluses 7.1.1, 7.2.1, and 7.3.1.

In results, we assume that some of DL active TCI state will have UL active TCI state as the left diagram, although it is unclear whether RAN1/2 have assumed the right diagram is also possible.

The configuration and operation seem like RAN1/2 issues. RAN4 may want to send LS to clarify in Rel-17. Anyhow, if assuming maintained PL-RS measurement is the active UL TCI condition, we found the issues below in RAN4 perspective,

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

In Rel-16, PL-RS maintenance relies on DL and UL channel reciprocity. If DL reception is triggered using a beam selection, UL beam can be selected by the channel reciprocity. However, in Rel-17, the unified TCI framework is designed without DL and UL channel reciprocity to support non-collocated multiple cell scenarios. Therefore, the path loss measurement for active UL TCI in the UE side should be aligned separately by network scheduling intention.

In addition, the number of PL-RS samples are not clear to fix at 5. It is obvious that accurate PL-RS measurement can be a pre-condition to make UL transmission, but it is not clear if the 5 sample PL-RS measurement period is essential for ‘active’ UL TCI. RAN4 does not have accuracy requirement for PL-RS measurement, but the spec addresses unconditionally 5 samples are required, that is problematic. For an example of DCI-based UL TCI switching, this activation of UL TCI should be quicker than MAC-CE based switching. In DCI-based TCI switching requirements, we are not sure if the ‘active TCI list’ in the box is the same TCI list equal to ‘active TCI list’ for UL. The UE behaviors regarding the active UL TCI state should be clear to address in 8.16.4. At least the ‘active’ TCI list seems different between DL and UL cases because of PL-RS maintenance in UL.

### CR R4-2207097 DraftCR on DCI based DL and UL TCI switching delay requirements ### 8.15.4 DCI based downlink TCI state switch delay When a UE is configured with the higher layer parameter with [tci-StateId_r17] for unified TCI switching and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State with [tci-StateId_r17] in the active TCI list [for downlink] for a CC. ### 8.16.4 DCI based uplink TCI state switch delay When a UE is configured with the higher layer parameter with [tci-StateId_r17] for unified TCI switching and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State with [tci-StateId_r17] [in active TCI list for uplink] for a CC.

Observation 3 : We found main issues regarding ‘active UL TCI list’ in inter-cell or mTRP scenarios

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

Proposal 2 : Based on Observation-2,3 regarding ‘active TCI list’ for UL, we propose a LS draft in appendix.

- Send LS to RAN1/2 to share the issue statements on active TCI list for UL. Possible solution is up to RAN1/2 in Rel-17 or Rel-18 enhancement.

- Improve PL-RS measurement period for TCI switching using active UL TCI list by reducing strict 5 PL-RS sample measurement period.

Here is the insight to reduce the 5 PL-RS sample measurement period. In the spec, the sample usage boundary between L1-RSRP measurement and PL-RS measurement is not clearly written regarding UE behavior. Regarding the PL-RS measurement, TS38.213 spec specifies as

= referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by higher layer parameter QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

RAN4 just takes a baseline UL behavior assuming two different steps of pathloss measurement: step-(i) PL-RS measurement beam selection through L1-RSRP, and then step-(ii) derives ‘higher layer filtered RSRP’ from PL-RS (5 samples)’. However, if the two steps are performed on the same RS, that needs not to make two separate steps. The process measuring higher layer filtered RSRP can be combined to obtain the filtered received power.

Issue 1-2-5 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 - Proposals - Option 1(Huawei): - When a SSB is indicated as PL-RS in a UL TCI state, the scaling factor for beam sweeping needs to be introduced for PL-RS measurement time in FR2. - In FR2, the MAC-CE based UL TCI state switching delay need to be separately defined for SSB based PL-RS. - In FR2, when a SSB is indicated as PL-RS in a UL TCI state, the MAC-CE based UL TCI state switching delay for both known case and unknown case can be defined as: - THARQ + 3ms + NM*(5*TL1-RSRP_SSB + 2ms) with the assumption of M=1. - Where NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise. - Option 1a(Apple): The delay is - TFirstSSB + 39*TSSB - Option 2(Ericsson): - TL1-RSRP_SSB as (M*P)*TSSB - Option 3(Nokia, Samsung): Define generic requirement - Option 4: Further clarification is needed

M is used for defining L1-RSRP measurement period. As we state above, we find two problems. If a UE needs to maintain multiple PL-RSs in multiple UL TCI, which PL-RS will be maintained or not maintained and how network knows about UL-TCI with maintained PL-RS. There is no separate indication to make such activation of PL-RS measurement in the current specs.

Also, M won’t always be fixed as 5 samples. If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, then we think it can be less than 5. In Rel-17, assuming PL-RS with different QCL-D cannot be measured simultaneously, a scaling factor will be applied, but the number of PL-RS under pathloss measurement should be scheduled and managed.

This issue assumes when SSB is indicated as PL-RS in UL TCI state, so TSSB is referred in this issue discussion. However, the PL-RS configuration is possible with both SSB and CSI-RS in case, the notation in the spec should be ‘TPL_RS’ that makes more general and generic requirements.

Proposal 3 : when SSB is indicated as PL-RS in UL TCI state for FR2,

-  The number of sample M will not always be fixed as 5 samples. If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, the number of samples used for L1-RSRP is counted for pathloss measurement.

- Sharing factor P can be considerable like option-2 in Issue 1-2-5, however the number of maintained PL-RS should be scheduled and managed by network.

- We refer to make generic requirements with ‘TPL_RS’ notation. When SSB is indicated as PL-RS in UL TCI state, ‘TPL_RS’ = TSSB.

Issue 1-5-1 Whether to define MAC CE based PL-RS switching requirement when PL-RS is unknown - Proposals - Option 1(Intel, Ericsson, ZTE): - Re-use MAC CE based UL TCI state switch delay of unknown case. - Option 2(Nokia): - Not to define PL-RS switching delay requirement when PL-RS is identical to the source RS in UL/Joint-TCI AND when the target PL-RS is unknown. - Apply MAC-CE based UL TCI switching delay requirement of known UL target TCI state, when target PL-RS and source RS in UL/joint TCI are QCL-Type-D AND - when the UL target TCI state is known but when the target pathloss reference signal is unknown OR - when the UL target TCI state is unknown but when the target pathloss reference signal is known. - Option 3: No additional requirements are defined or needed

Regarding whether to define MAC CE based PL-RS switching requirement when PL-RS is unknown, the result of option-1 is to consider  TL1-RSRP

- For TCI unknow case, MAC CE based uplink TCI switching delay requirement is specified as

THARQ + 3ms + TL1-RSRP + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms)

Basically, we want to clarify when PL-RS is unknown. In other words, when additional TL1-RSRP   is required for the PL-RS requirement?  The option-1 just implies the result of PL-RS is unknow, but we want to ask about cases when PL-RS is unknown.

A baseline behavior will be that a UE will choose a measurement beam based on source RS in UL-TCI and actually measures path-loss on target pathloss reference signal. We want to clarify UE behavior when one of PL-RS and source RS in UL TCI is known.  The UE can practically use a beam measuring L1-RSRP on the source RS. In this case, additional TL1-RSRP is not required for PL-RS measurement.

Proposal 4 : When the source RS in UL-TCI is known and the source RS is associated with the configured PL-RS by QCL-D, the PL-RS is always treated as known.

    - The UE can practically use the beam measuring L1-RSRP for pathloss measurement. In this case, additional TL1-RSRP is not required for PL-RS measurement.

TCI state-pair indication requirement

Issue 1-2-4 TCI state-pair indication requirement - Proposals - Option 1(Apple, MTK, Intel, ZTE, Huawei): - The TCI state switching delay requirement can be defined for UL TCI and DL TCI switching independently. - Option 2 (vivo): - Remove the issue here without conclusions since the intention is not clear. - Option 3(Samsung, Nokia): - No requirements for TCI state-pair indication

We think that joint TCI and sperate TCI state switching delay requirements can be defined with UL TCI and DL TCI switching. But RAN4 can further discuss if UE implementation is impacted when DL and UL switching are indicated for switching at the same time. We assume joint TCI (DL+UL) and a pair of DL+UL separate TCI indications are identical w.r.t TCI switching delay. From option-1, the TCI state switching delay requirement can be defined for UL TCI and DL TCI switching independently, so it won’t impact switching delay each other, one delay requirement does not need to refer to another delay requirement, since it is independent. We don’t see good reason to define TCI state-pair indication requirement separately, but we can further discuss to understand UE behaviors if needed.

Proposal 5 : Rel-17 unified TCI switching delay requirement can be defined with UL-TCI and DL-TCI switching requirements respectively. We don’t see good reason to define TCI state-pair indication requirement.

MAC CE based TCI state list update delay for serving cell

Issue 1-6-1 MAC CE based TCI state list update delay for serving cell - Proposals - Option 1(vivo, Nokia, Ericsson): - For MAC CE based TCI state list update, specify requirements for the case when not all TCI states are known. - Option 2(Apple, MTK, Samsung): - Define MAC CE based TCI state list update requirement for known TCI state case

As we discussed, network can update the MAC CE based TCI state list with multiple TCI state elements, all of the TCI states cannot be known always. The list updates may include unknown TCI states. So, we consider option-1 is definitely common cases, accordingly RAN4 introduces TCI switching requirements based on option-1.

Proposal 6 : Regarding MAC-CE based TCI state list update for SC and NSC cells, consider option-1 (specifying requirements for the case when not all TCI states are known), that is definitely a common case.

Conclusion

In this contribution we provide our observations and proposals on remaining issues on unified TCI switching requirements.

Observation 1 : Fundamental conditions for active TCI for UL are missing. There is no definition of active TCI for UL in RAN1/2 specs. Also, a UE should keep time and frequency sync tracking on the source RS in UL TCI to be capable of UL transmission.

Observation 2 : maxNumberActiveTCI-PerBWP under tci-StatePDSCH should be about activated TCI-states with UE synchronization for both DL and UL. The current spec addresses about DL only.

Proposal 1 : Rel-17 active UL TCI state should be under time and frequency tracking. This means that active UL TCI list belongs to active DL TCI state list.

- Add the time and frequency tracking condition to the active TCI state for UL.

Observation 2 : We found main issues regarding ‘active UL TCI list’ in inter-cell or mTRP scenarios

Observation 3 : We found main issues regarding ‘active UL TCI list’ in inter-cell or mTRP scenarios

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

Proposal 2 : Based on Observation-2,3 regarding ‘active TCI list’ for UL, we propose a LS draft in appendix.

- Send LS to RAN1/2 to share the issue statements on active TCI list for UL. Possible solution is up to RAN1/2 in Rel-17 or Rel-18 enhancement.

- Improve PL-RS measurement period for TCI switching using active UL TCI list by reducing strict 5 PL-RS sample measurement period.

Proposal 3 : when SSB is indicated as PL-RS in UL TCI state for FR2,

-  The number of sample M will not always be fixed as 5 samples. If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, the number of samples used for L1-RSRP is counted for pathloss measurement.

- Sharing factor P can be applicable, however the number of UL-TCI under pathloss measurement maintenance should be scheduled and managed between network and a UE.

- We refer to make generic requirements with ‘TPL_RS’ notation. When SSB is indicated as PL-RS in UL TCI state, ‘TPL_RS’ = TSSB.

Proposal 4 : When the source RS in UL-TCI is known and the source RS is associated with the configured PL-RS by QCL-D, the PL-RS is always treated as known.

    - The UE can practically use the beam measuring L1-RSRP for pathloss measurement. In this case, additional TL1-RSRP is not required for PL-RS measurement.

Proposal 5 : Rel-17 unified TCI switching delay requirement can be defined with UL-TCI and DL-TCI switching requirements respectively. We don’t see good reason to define TCI state-pair indication requirement.

Proposal 6 : Regarding MAC-CE based TCI state list update for SC and NSC cells, consider option-1 (specifying requirements for the case when not all TCI states are known), that is definitely a common case.

- 4. References

[1] R4-2207108, WF on RRM requirements for inter-cell beam management, RAN4#102-e

[2] R4-2206935, WF on other RRM requirements for FeMIMO, RAN4#102-e

[3] R4-2206943, WF on FeMIMO RRM impact for unified TCI state, RAN4#102-e

[4] R4-2207068, Email discussion summary [102-e][240]  FeMIMO_ICBM, RAN4#102-e

[5] R4-2207081, Email discussion summary [102-e][240] NR_feMIMO_RRM_TCI, RAN4#102-e

- 5. Appendix

3GPP TSG-RAN WG4 Meeting #103e             	    			         R4-220XXXX

Electronic Meeting, May 9 – May 20, 2022

Title:	[DRAFT] LS on active TCI state list for UL TCI

Response to:

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN WG4

To:	RAN WG1/WG2

Cc:

Contact Person:

#### Name:	Daejung DJ  Yoon

#### E-mail Address:	 daejung.yoon@nokia-bell-labs.com

Attachments:

1. Overall Description:

In RAN4 #103-e meetings, RAN4 has discussed active TCI list with UL TCI switching with path loss measurements.

A problem is found related to ‘active’ TCI state for ‘UL’. For DL case, if target TCI state is in the active TCI state list for PDSCH/PDCCH, the UE should track time and frequency sync on the resource RS. However, there is no statement for UL TCI corresponding to the active TCI list. Legacy signaling associated with the active TCI list is stated only under DL behaviors.

RAN4 also considers path loss reference signal maintenance as an extra condition for active TCI list for UL.

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

RAN4 respectfully asks RAN1/2 to take RAN4 observations in consideration for UL TCI signaling and UE behaviors and define ‘active’ TCI state for ‘UL’.

2. To RAN WG1 group.

ACTION: RAN4 respectfully asks RAN1/2 to take RAN4 observations in consideration for UL TCI signaling and UE behaviors and define ‘active’ TCI state for ‘UL’.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #104e