3GPP TSG-RAN WG4 Meeting #103-e	R4-2210053

# Electronic meeting, May 9 – May 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on DCI based DL and UL TCI switching delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Active TCI state list for UL is new concept in Rel-17, while active TCI state list for DL exists. The switch behavior with acitive TCI state list in DL and UL are specfically specified. TCI switching with UL-CA is possible. The UL switching requirement is applicable to all CCs with an indicated UL TCI-State. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ‘active TCI list for downlink’ in DL and ‘active TCI list for uplink’ in UL Add requirements for all CCs with an indicated TCI-State in UL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no definition of active TCI state list for UL, so DCI-based TCI switching for UL cannot be triggered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.4 and 8.16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==========================Start of first change =============================

### 8.15.4   DCI based downlink TCI state switch delay

When a UE is configured with the higher layer parameter with [tci-StateId_r17] for unified TCI switching and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State with [tci-StateId_r17] in the active TCI list for downlink for a CC or [all CCs with an indicated TCI-State in the same CC list configured by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2], the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication.

If the target TCI state is known, the downlink TCI switching to the indicated TCI-State in the DCI format shall be completed starting from the first slot that is at least [BeamAppTime_r17] symbols after the last symbol of the PUCCH carrying HARQ-ACK in response to the DCI triggering TCI state activation. The first slot and the [BeamAppTime_r17 ] symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The value of [BeamAppTime_r17] is defined in TS 38.331 [2].  The known condition for TCI state defined in clause [8.15.2] is applied.

==========================End of first change =============================

==========================Start of second change =============================

### 8.16.4   DCI based uplink TCI state switch delay

When a UE is configured with the higher layer parameter with [tci-StateId_r17] for unified TCI switching and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State with [tci-StateId_r17] [in the active TCI list for uplink] for a CC or [all CCs with an indicated TCI-State in the same CC list configured by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2], the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication.

If the target TCI state is known, the uplink TCI switching to the indicated TCI-State in the DCI format shall be completed starting from the first slot that is at least [BeamAppTime_r17] symbols after the last symbol of the PUCCH carrying HARQ-ACK in response to the DCI triggering TCI state activation. The first slot and the [BeamAppTime_r17 ] symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The value of [BeamAppTime_r17] is defined in TS 38.331 [2].  The known condition for TCI state defined in clause [8.16.2] is applied. [Editor note : PL-RS is maintained for DCI based UL TCI state switch.]

Editor note : TBD how to capture that pathloss reference singal is switched as a part of uplink TCI state switching.

==========================End of second change =============================