# 3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210135

# Electronic Meeting, May 09- May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2394 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SCell activation/deactivation with PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Multiple SCells activation delay is agreed in this meeting and respective changes need to be captured in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multiple SCells activation delay when one of the SCell is PUCCH SCell is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for PUCCH SCell activation delay are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.12, 8.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### 8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell

The requirements in this clause shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

If the UE has a valid TA for transmitting on an SCell then the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+X+TCSI_ReportingNR slot length,

Where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running.

-	THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tactivation_time for FR1 is the SCell activation delay in millisecond as specified in section 8.3.2.

-	Tactivation_time for FR2 is the SCell activation delay in millisecond as specified in section 8.3.2 in which Tuncertainty_MAC is updated as below:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	TCSI_reporting is the delay (in ms) specified in clause 8.3.2

-	X sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known

-	X = 0 when PL-RS of PUCCH SCell is QCL-Type A with other active serving cell PL-RS or PL-RS of PUCCH SCell is same as other active serving cell PL-RS

-    X = 4 for unknown PUCCH SCell activation case

-    X=5 in all other cases

If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot lengthTHARQ+Tactivation_time+TCSI_ReportingNR slot length, and shall be capable to perform uplink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length n+THARQ+Tdelay_PUCCH_SCell+TCSI_Reporting_PUCCH_SCellNR slot lengthand shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length, where:

Tdelay_PUCCH_SCell = Tactivation_time + [X] + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3)) + TCSI_reporting_after

Where:

-	Tactivation_time is the SCell activation delay in millisecond as specified in section 8.3.2.

-	X sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known

-	X = 0 when PL-RS of PUCCH SCell is QCL-Type A with other active serving cell PL-RS or PL-RS of PUCCH SCell is same as other active serving cell PL-RS

-    X = 4 for unknown PUCCH SCell activation case

-    X=5 in all other cases

-	TFirst_available_CSI: the delay uncertainty in acquiring the first available downlink CSI reference resource.

-	TCSI_processing: the UE processing time for CSI reporting.

-	TCSI_reporting_after: the delay uncertainty in acquiring the first available CSI reporting resource after T3

-	T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the PUCCH SCell after Tactivation_time.

-	T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in the table 8.1-1 of TS 38.213

-	T2 is the delay from slot n + (THARQ + Tactivation_time +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH Scell being activated. Slot n is the slot where the UE receives PUCCH SCell activation command.

-	T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal:

The target pathloss reference signal determination is based on the latest L3 RSRP measurement reporting

The target pathloss reference signal remains detectable during the PUCCH SCell activation period

SNR of the target pathloss reference signal≥-3dB

The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

The target pathloss reference signal determination is based on the latest L1-RSRP measurement reporting

The target pathloss reference signal remains detectable during the PUCCH SCell activation period

SNR of the target pathloss reference signal≥-3dB

The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

The above delay requirement (Tdelay_PUCCH SCell) shall apply provided that:

-	The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and

-	No SRS carrier based switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay (Tdelay_PUCCH_SCell) can be extended. The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is the same as the interruption in single SCell activation requirement in clause 8.3.2.

-	For unknown PUCCH SCell activation, the requirements only apply when UE supports cross PUCCH group CSI reporting capability [TBD], and UE is configured with CSI reporting via SpCell. And the PDCCH order (when applicable) and the activation command for TCI, UL spatial relation, and PL-RS are based on latest valid L1-RSRP reporting via Primary PUCCH group.

### 8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells

#### 8.3.13.1	Introduction

The requirements in this clause shall apply for the UE configured with multiple deactivated downlink SCells and PUCCH is configured for a SCell, and when PUCCH SCell with downlink SCell(s) are activated by one MAC command.

In EN-DC, NE-DC, standalone NR, or in one CG of NR-DC, the requirements in this clause shall apply when the following conditions are met:

-	UE only receives one single MAC command for multiple SCell activation within the activation period defined in this clause

-	in each single CG, there are no other SCell activation, deactivation, addition or release before activation is completed for all the SCells activated by the single MAC CE in this clause, and

-	in EN-DC and NE-DC, there are no E-UTRAN SCell activation, deactivation, addition or release before multiple SCell activation is completed in this clause, and

-	any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band

In two CGs of NR-DC, the requirements in this clause shall apply when the following conditions are met:

-	UE receives one MAC command per CG for multiple SCell activation within the activation period defined in this clause, and

-	UE supports per-FR measurement gap capability, and

-	any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band

#### 8.3.13.2	Requirements

If the CSI report is configured to be sent on primary PUCCH group for all the SCells including PUCCH SCell, only valid TA case needs to be considered for multiple SCell activation.

Upon receiving SCell activation command in slot n for PUCCH SCell with multiple downlink SCell(s), if the UE has a valid TA for transmitting on a PUCCH SCell then the UE shall be able to transmit valid CSI report and apply actions related to the SCell activation command as specified in [7] for the PUCCH SCell being activated  no later than in slot n+ Tactivate_total.

Where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running.

-	Tactivate_total is THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

Tdelay_PUCCH_ multiple_SCells = Tactivation_time_multiple_scells + [X] + TCSI_Reporting

Where:

-	Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7.

-	X sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known

-	X = 0 when PL-RS of PUCCH SCell is QCL-Type A with other active serving cell PL-RS or PL-RS of PUCCH SCell is same as other active serving cell PL-RS

-    X = 4 for unknown PUCCH SCell activation case

-    X=5 in all other cases

-	TCSI_Reporting is same as specified in clause 8.3.7.

If the CSI report of primary PUCCH group is configured to be sent on SpCell and CSI report of secondary PUCCH group is configured to be sent on PUCCH SCell and if the UE has a valid TA for transmitting on a PUCCH SCell, the UE shall be able to transmit valid CSI report and apply actions related to the SCell activation command as specified in [7] for the PUCCH SCell activated, no later than in slot n+ Tactivate_total.

Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

Tdelay_PUCCH_ multiple_SCells = Tactivation_time_multiple_scells + [X] + max (TCSI_Reporting_SpCell, TCSI_Reporting_PUCCH_SCell)

Where:

Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7.

-	X sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known

-	X = 0 when PL-RS of PUCCH SCell is QCL-Type A with other active serving cell PL-RS or PL-RS of PUCCH SCell is same as other active serving cell PL-RS

-    X = 4 for unknown PUCCH SCell activation case

- X=5 in all other cases

- TCSI_Reporting_SpCell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on primary PUCCH group, as specified in TS 38.331 [2].

- TCSI_Reporting_PUCCH_SCell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on secondary PUCCH group, as specified in TS 38.331 [2].

If the CSI report of primary PUCCH group is configured to be sent on SpCell and CSI report of secondary PUCCH group is configured to be sent on PUCCH SCell and if the UE does not have a valid TA for transmitting on a PUCCH SCell, the UE shall be able to transmit valid CSI report and apply actions related to the SCell activation command as specified in [7] for the PUCCH SCell activated no later than in slot n+ Tactivate_total.

If the UE is not capable of capable of parallelTxPRACH-SRS-PUCCH-PUSCH, then the UE shall prioritize between PRACH and CSI reporting as below.

-	When the PRACH occasion on the PUCCH SCell at least partially overlap in time with CSI reporting resources on the SpCell in same CG or in different CGs, then the UE shall transmit PRACH and is allowed to postpone the CSI reporting

-	When the PRACH occasion on the PUCCH SCell at least partially overlap in time with CSI reporting resources on another PUCCH SCell in different CGs, then the UE shall transmit PRACH and is allowed to postpone the CSI reporting.

Otherwise, if UE is capable of capable of parallelTxPRACH-SRS-PUCCH-PUSCH, then the UE shall simultaneously transmit both PRACH and CSI reporting in the above scenario.

Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

Tdelay_multiple_PUCCH_SCell = Tactivation_time_multiple_scells + [X] + max (max ((TFirst_available_CSI_PUCCH_SCell + TCSI_processing_PUCCH_SCell), (T1+T2+T3)) + TCSI_reporting_after_PUCCH_SCell, TCSI_Reporting_SpCell + Tuncertainity)

Where:

- Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7.

- X sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known

-	X = 0 when PL-RS of PUCCH SCell is QCL-Type A with other active serving cell PL-RS or PL-RS of PUCCH SCell is same as other active serving cell PL-RS

-    X = 4 for unknown PUCCH SCell activation case

-    X=5 in all other cases

- Tfirst_available_CSI_ PUCCH_SCell: the delay uncertainty in acquiring the first available downlink CSI reference resources for secondary PUCCH group.

- TCSI_processing_ PUCCH_SCell: the UE processing time for CSI reporting of secondary group PUCCH SCells.

- TCSI_reporting_after_ PUCCH_SCell the delay uncertainty in acquiring the first available CSI reporting resource after T3

- T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the PUCCH SCell after Tactivation_time.

- T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in the table 8.1-1 of TS 38.213

- T2 is the delay from slot n + (THARQ + Tactivation_time +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. Slot n is the slot where the UE receives PUCCH SCell activation command.

- T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

- TCSI_Reporting_SpCell: is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on primary PUCCH group, as specified in TS 38.331 [2].

- Tuncertainity is uncertainty of first available CSI reporting resource, if CSI report on primary PUCCH group collides with PRACH on PUCCH SCell. In other cases, it is zero.

The interruption on the SpCell shall meet the PCell interruption requirements specified in section 8.2.2.2.7.

### <End of Change 1>