3GPP TSG-RAN WG4 Meeting#103-e	R4-2210151

Online, 9th May – 20th May, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on FeMIMO HST Performance Requirements

Agenda item:	9.18.4.2.1

Document for:	Discussion

- Introduction

In RAN4#102-e, simulation assumptions were agreed to define requirements HST-SFN Scheme A [1]. According to this scheme, TRS from different TRPs are to be transmitted in a non-SFN manner whereas DMRS and PDSCH would be transmitted in an SFN manner. There were also some open issues as discussed in [1]. In this paper, we share our views on whether to introduce performance requirements for Scheme B, assumption of maximum Doppler Shift as well as provide simulation results for Scheme A.

- Whether to define requirement with HST-SFN Scheme B

Issue 2-1-2: Whether to define PDSCH requireemnt with HST-SFN scheme B

Candidate options:

- Option 1: Yes

  - Option 1a: scheme A and scheme B with test applicability rule: If UE pass HST-SFN scheme A test cases, UE can skip HST-SFN scheme B test cases

- Option 2: No

- Option 3: do not introduce PDSCH requirements for SFN scheme B and define the following test applicability rule to guarantee performance with this scheme:

  - If UE passes the existing test cases (demodulation requirement for HST-SFN with high Doppler shift), the performance of SFN scheme B is guaranteed

According to implementation of scheme B, PDSCH will be Doppler pre-compensated from the network side, therefore it does not impact the UE demod processing in a meaningful way that requires defining requirements for this scheme. Even in the presence of residual Doppler (after pre-compensation), UE should be able to deal with it as the residual Doppler is expected to be small.

Proposal 1: Option 2 (Do not define PDSCH requireemnt with HST-SFN scheme B)

- Maximum Doppler Shift

Issue 2-2-3: Maximum Doppler shift

Candidate options:

- Define PDSCH requirement with HST-SFN scheme A with Maximum Doppler shift

  - 30KHz SCS: 1667Hz

  - 15 kHz SCS:

    - Option 1: 972 Hz

    - Option 2: 870 Hz

The difference between the estimated Dopplers for TRP#1 (i.e., estimated from TRS1) and TRP#2 (i.e., estimated from TRS2) should still be within the TRS pull-in range. Furthermore, UE has to come up with a mechanism to make use of the estimated Dopplers from two TRSs. Therefore, the assumption of single-tap model, for which a higher Doppler is assumed (option 2) may not be applicable here. Hence, we support a lower Doppler, i.e., option 2 for 15KHz SCS.

Proposal 2: (15KHz SCS) Option 2: 870 Hz

- Simulation Results for HST-SFN Scheme A

Based on the agreed simulation assumptions [1], we provide the simulation results as below for both FDD and TDD scenarios with 1667Hz and 870Hz Doppler, respectively.

|  | 2x2 | 2x4 |
| --- | --- | --- |
| Duplex Mode/SCS | Alignment SNR (dB) | Alignment SNR (dB) |
| FDD 15KHz |  |  |
| TDD 30KHz |  |  |

- Conclusions

In this paper, we share our views on the remaining open issues and provide simulation results for Scheme A. The following has been proposed.

Proposal 1: Option 2 (Do not define PDSCH requireemnt with HST-SFN scheme B)

Proposal 2: (15KHz SCS) Option 2: 870 Hz

# 6. References

[1] R4-2207208, “WF on demodulation requirement for Enhancement on HST-SFN deployment,” 3GPP TSG RAN WG4 Meeting #102-e, Online, Feb 21 – Mar 3, 2022.