3GPP TSG-RAN WG4 Meeting#103-e	R4-2210156

Online, 9th May – 20th May, 2022

Source: 	Qualcomm Incorporated

Title: 	Views and Simulation Results on Inter-cell Interference PDSCH Tests

Agenda item:	9.11.2.1.1

Document for:	Discussion

- Introduction

In RAN4#102-e, WF [1] was agreed and it had a few open issues. In this paper, we provide our views on those open issues for inter-cell interference scenarios.

- Test Parameters and Simulation Results

Interference modelling in PDCCH region

For RLM procedure, UE estimates the SNR based on SSB. Therefore, we prefer to align the interference seen by SSB and PDCCH and propose the following.

Proposal 1: Assume Option 3 (non-overlapping PDCCH) when SSB is non-colliding and Option 2 (PDCCH transmission from interference cells) when SSB is colliding.

Simulation Results

Based on simulation assumptions in [1], we provide the alignment SNR at 70% of peak throughput below for single cell interference assuming 100% loading on interfering cell.

Table 1: SNR in dB at 70% of peak throughput for single cell interference scenario with MCS13, Rank1, INR = 7.58dB

| Case | MMSE-MRC | MMSE-IRC |  |
| --- | --- | --- | --- |
|  |  | Alignment | Impairment |
| FDD: TDLA30-10 ULA Low, 2x2 | 11.72 | 9.41 | 11.41 |
| FDD: TDLA30-10 ULA Low, 2x4 | 8.05 | 4.72 | 6.72 |
| TDD: TDLA30-10 ULA Low, 2x2 | 11.71 | 9.44 | 11.44 |
| TDD: TDLA30-10 ULA Low, 2x4 | 8.10 | 4.75 | 6.75 |

Table 2: SNR in dB at 70% of peak throughput for 2 cell interference scenario with MCS13, Rank1, INR = (7.77,2.29) dB

| Case | MMSE-MRC | MMSE-IRC |  |
| --- | --- | --- | --- |
|  |  | Alignment | Impairment |
| FDD: TDLC300-100 ULA Low, 2x2 | 14.06 | 12.70 | 14.70 |
| FDD: TDLC300-100 ULA Low, 2x4 | 10.44 | 7.71 | 9.71 |
| TDD: TDLC300-100 ULA Low, 2x2 | 14.17 | 12.83 | 14.83 |
| TDD: TDLC300-100 ULA Low, 2x4 | 10.67 | 7.70 | 9.70 |

- Conclusions

This paper provides our views on open issues for inter-cell interference scenarios in RAN4 and also provides simulation results based on [1]. Following has been observed and proposed.

Proposal 1: Assume Option 3 (non-overlapping PDCCH) when SSB is non-colliding and Option 2 (PDCCH transmission from interference cells) when SSB is colliding.

# References

[1] R4-2207242, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, 3GPP TSG RAN WG4 Meeting #102-e, Online, 21 Feb – 3 Mar, 2022.