3GPP TSG-RAN WG4 Meeting#103-e	R4-2210160

Online, 9th May – 20th May, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation results for FR1 HST PDSCH CA Tests

Agenda item:	9.7.3

Document for:	Discussion

- Introduction

In RAN4#99-e, simulation assumptions were agreed in [1] for defining HST requirements for CA scenarios. It was agreed to define requirements for BW of {35,45} MHz for FDD 15kHz SCS in [2]. In this paper, we provide our simulation results for FR1 HST PDSCH CA test cases for 35 and 45MHz BWs.

- HST-SFN Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below for FDD 35/45MHz BWs.

Table 1: HST-SFN Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 35 | 10.69 | 13.19 | 8.8 | 11.3 |
| 45 | 10.56 | 13.06 | 8.72 | 11.22 |

- HST-DPS Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below for FDD 35/45MHz BWs.

Table 2: HST-DPS Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 35 | 10.26 | 12.26 | 7.45 | 9.45 |
| 45 | 10.26 | 12.26 | 7.47 | 9.47 |

- Conclusions

This paper provides our simulation results for FR1 HST PDSCH CA requirements for FDD 35/45MHz BWs.

# References

[1] R4-2108635, “WF on UE demodulation for FR1 HST”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 – May 27, 2021.

[2] R4-2207194, “WF on UE demodulation for FR1 HST”, 3GPP TSG RAN WG4 Meeting #102-e, Online, Feb 21 – Mar 3, 2022.