3GPP TSG RAN WG4 Meeting #103-e	R4-22010186

Electronic Meeting, 09-20 May 2022

Agenda Item:	5.3.3

Source: 	Ericsson

Title:	On timing advance (TADV) measurement report mapping

Document for:	Discussion

- Introduction

A new NR gNB positioning measurement, timing advance (TADV), has been introduced in TS 38.215 in Rel-17 as TEI17. RAN4 needs to define the measurement report mapping in TS 38.133 and inform RAN3 to develop the necessary NRPPa signaling between gNB and LMF.

In this paper we analyze the impact of the timing advance (TADV) in RAN4.

# Analysis of requirements for timing advance

The timing advance (TADV) has been specified as new gNB positioning measurement for NR positioning in clause 5.2.7 of TS 38.215 v17.1.0:

| Definition | Timing advance (TADV) is defined as the time difference TADV = (TgNB-RX – TgNB-TX), Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing PRACH transmitted from UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. The detected PRACH is used to determine the start of one subframe containing that PRACH. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. |
| --- | --- |

Furthermore, according to RAN1 LS sent to RAN2 and RAN3, the TADV reporting has been introduced as new NR UL E-CID positioning measurement. The gNB estimates the timing advance and report to the LMF over the NRPPa interface (between gNB and LMF).

In LTE a reporting mapping for similar measurement (timing advance type 1) for LTE E-CID is defined in TS 36.133, clause 10.3.1 as follows:

Table 10.3.1-1:  TADV measurement report mapping [TS 36.133]

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| TIME_ADVANCE_00 | TADV 2 | Ts |
| TIME_ADVANCE_01 | 2 TADV < 4 | Ts |
| TIME_ADVANCE_02 | 4 TADV < 6 | Ts |
| … | … | … |
| TIME_ADVANCE_2046 | 4092 TADV < 4094 | Ts |
| TIME_ADVANCE_2047 | 4094 TADV < 4096 | Ts |
| TIME_ADVANCE_2048 | 4096 TADV < 4104 | Ts |
| TIME_ADVANCE_2049 | 4104 TADV < 4112 | Ts |
| … | … | … |
| TIME_ADVANCE_7688 | 49216 TADV < 49224 | Ts |
| TIME_ADVANCE_7689 | 49224 TADV < 49232 | Ts |
| TIME_ADVANCE_7690 | 49232 TADV | Ts |

We therefore propose to define the timing advance measurement reporting mapping for NR UL E-CID in Rel-17 based on the mapping defined for timing advance type 1 for E-CID in TS 36.133, clause 10.3.1.

Furthermore, RAN4 sends an LS to RAN3 requesting to define the corresponding signaling between gNB and LMF based on the timing advance measurement reporting mapping introduced in TS 38.133.

# Draft of LS Response to RAN3

Title:	LS on timing advance (TADV) report mapping for NR UL E-CID positioning

Response to:	-

Release:	Rel-17

Work Item:	TEI17

Source:	TSG RAN WG3

To:	TSG RAN WG1, TSG RAN WG2

Cc:

Contact Person:

#### Name:	          Muhammad Kazmi

E-mail Address:	muhammad.kazmi@ericsson.com

1. Overall Description:

RAN4 would like to inform that RAN4 has introduced the following Timing advance (TADV) measurement report mapping in Rel-17, TS 38.133 for the timing advance (TADV) measurement defined by RAN1 in clause 5.2.7 of TS 38.215 v17.1.0:

The reporting range of Timing advance (TADV) is defined from 0 to 3150848 Tc with:

- 128 Tc resolution for timing advance less or equal to 262114 Tc and

- 512 Tc for timing advance greater than 262144 Tc.

The mapping of measured quantity is defined in Table 13.X.1-1:

Table 13.X.1-1: Timing advance (TADV) measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| TIME_ADVANCE_00 | TADV 128 | Tc |
| TIME_ADVANCE_01 | 128 TADV < 256 | Tc |
| TIME_ADVANCE_02 | 256 TADV < 384 | Tc |
| … | … | … |
| TIME_ADVANCE_2046 | 261888 TADV < 262016 | Tc |
| TIME_ADVANCE_2047 | 262016 TADV < 262144 | Tc |
| TIME_ADVANCE_2048 | 262144 TADV < 262656 | Tc |
| TIME_ADVANCE_2049 | 262656 TADV < 263168 | Tc |
| … | … | … |
| TIME_ADVANCE_7688 | 3149824 TADV < 3150336 | Tc |
| TIME_ADVANCE_7689 | 3150336 TADV < 3150848 | Tc |
| TIME_ADVANCE_7690 | 3150848 TADV | Tc |

2. Actions:

To RAN3

ACTION: 	RAN4 respectfully asks RAN3 to take into account the Timing advance (TADV) report mapping defined in TS 38.133 for introducing the necessary signaling in their specifications.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #104	    August 22-26, 2022, Toulouse, France

TSG-RAN WG4 Meeting #104-bis-e 	    October 10-19, 2022, Electronic Meeting

# Summary

The following are the observations and proposals related to timing advance (TADV) report mapping:

- Observation 1: The timing advance (TADV) has been specified as new gNB positioning measurement for NR positioning in clause 5.2.7 of TS 38.215 v17.1.0.

- Observation 2: TADV reporting is NR UL E-CID positioning measurement, which is reported by gNB to LMF.

- Observation 3: TADV for NR UL E-CID positioning measurement is analogous to timing advance for LTE E-CID defined in TS 36.133, clause 10.3.1.

- Proposal #1: Define Timing advance (TADV) report mapping in TS 38.133 as follows:

The reporting range of Timing advance (TADV) is defined from 0 to 3150848 Tc with:

- 128 Tc resolution for timing advance less or equal to 262114 Tc and

- 512 Tc for timing advance greater than 262144 Tc.

The mapping of measured quantity is defined in Table 13.X.1-1:

Table 13.X.1-1: Timing advance (TADV) measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| TIME_ADVANCE_00 | TADV 128 | Tc |
| TIME_ADVANCE_01 | 128 TADV < 256 | Tc |
| TIME_ADVANCE_02 | 256 TADV < 384 | Tc |
| … | … | … |
| TIME_ADVANCE_2046 | 261888 TADV < 262016 | Tc |
| TIME_ADVANCE_2047 | 262016 TADV < 262144 | Tc |
| TIME_ADVANCE_2048 | 262144 TADV < 262656 | Tc |
| TIME_ADVANCE_2049 | 262656 TADV < 263168 | Tc |
| … | … | … |
| TIME_ADVANCE_7688 | 3149824 TADV < 3150336 | Tc |
| TIME_ADVANCE_7689 | 3150336 TADV < 3150848 | Tc |
| TIME_ADVANCE_7690 | 3150848 TADV | Tc |

- Proposal #2: Send LS to RAN3 informing about the timing advance (TADV) report mapping agreed by RAN4 for defining relevant signalling (excerpt of LS out is given in section 3).

A Rel.17 CR to introduce the timing advance (TADV) report mapping is provided in [2].

- References

- R1-2110601, LS on NR Positioning support for TA measurement in NR UL E-CID, RAN LS to RAN2/RAN3

- R4-2210187, Timing advance (TADV) measurement report mapping in TS 38.133, Ericsson, NTT DOCOMO.