3GPP TSG-RAN WG4 Meeting #103-e	R4-2210215

May 2022

Title:	Handling of CA for DMRS bundling

Source:	Qualcomm Incorporated

Agenda item:	9.17.1.1

Release:	Rel-17

Work Item:	NR_cov_enh

Responsible WG:	RAN1

Document for:	Approval

# 1.   Introduction

CA with DMRS bundling has been left for RAN1 consideration and not discussed in RAN4 yet. In this paper we provide motivation, insight in to requirements and capabilities for DMRS bundling for DL and UL CA.

# 2. 	Discussion

## 2.1	Justification for DMRS bundling for CA

The premise of coverage enhancements assumes the UE close to or out of coverage and intuitively it might be questioned why CA and DMRS bundling need to be discussed together. However, for DL CA case, considering a case when UE is configured for n78 and n3 DL CA with UL on n3. This case itself makes sense when UE is out of UL coverage for n78. UE might also move out of n3 UL coverage and in order to send ACK/NACKs (PUCCH) about received PDSCH on n3 and n78, UE can use DMRS bundling.

Observation 1: UE may be out of UL coverage for two bands when it is in DL coverage for the same bands and using DMRS bundling one of the bands for PUCCH helps to extend the UL coverage

For UL CA case, while UE is configured for UL CA, it may only transmit on one band at the time. Then, the case in observation 1 becomes relevant for UL CA. The UE maybe configured for UL CA n3+n78 and drop out of UL coverage for both bands, or at least be UL coverage challenged for both bands and in this case DMRS bundling helps to extend the UL coverage that may be challenged only temporarily. For DL heavy applications, the user experience can have remarkable improvement if UL coverage is extended and most of the high BW data is transmitted using n78 DL.

Observation 2: When UE is configured for UL CA, DMRS bundling helps to extend the UL coverage and improve the link reliability

## 2.2	DMRS bundling requirements for Inter-band UL CA

The requirements were drafted in LS [2] in early phases of the WI. It was assumed that as long as there are no changes in power on other (or own) CCs, then phase continuity can be maintained. In precise form, it was said “when own CC is not impacted by other..” and this means that there is no change in MPR/A-MPR or maximum power due to dynamic power sharing to the own CC. With these, ran4 can derive requirements for inter-band CA.

Proposal 1: Define inter-band UL CA requirements for DMRS bundling.

Ran4 can further restrict or discuss cases as follows:

- Only one band can support DMRS bundling at a time

- In case two bands are supporting DMRS bundling, the bundles are not overlapping in time

- In case bundles are overlapping in time their time windows are aligned

In [33] we have provided our input of how to define the requirements according to A). Extending the coverage to B) and C) cases, would require changes to the PCMax sentences about handling of the cases when transmission starts or end on an other band where bundling is active since it may impact the power on the band with the bundling. Also frequency hopping may impact the PCMax.

Observation 3: Extending UL CA coverage to case B or C would need discussions on requirements in other sections than transmit signal quality.

It is also unclear if RAN1 needs to be involved if the TDW need to be restricted.

## 2.3	Impact on capability with CA

The agreement in RAN4 was to leave the CA aspect to ran1 with LS [27] since RAN4 has not discussed CA requirements. If the proposals in [36] or in this paper are agreeable, the RAN4 could further think if the DMRS bundling capability or maximum duration capability should be revisited.

The UE may be capable for bundling on two bands separately. UE may implement phase retention techniques to achieve good performance but since those circuits need real-estate, either in silicon or MIPS for software, UE may not be able to apply them on two bands simultaneously. These issues concern the case when UE may need to perform bundling on two bands but if RAN4 only covers the case A), the these problem don’t apply directly.

For the case A) and DL CA case, UE may still have problems with DMRS bundling that do not exist for the implementation for single band. For example, the frequency hopping on the other band may impact of the phase continuity for the band with the bundling sine the frequency generation circuitry may produce glitches. In some case frequency planning may also want to change reference frequencies to avoid harmonic overlaps and receiver desentization.

Observation 4: UE may have challenges in  maintaining phase continuity when it is configured for CA that do not exist in single band case

Proposal 2: Define the capabilities for DMRS bundling per band per band combination.

RAN4 should inform RAN2 and RAN1 about this issue. We provide a draft LS in the annex.

Proposal 3: Inform RAN1 that the DMRS bundling capabilities should be per band per band combination

# Conclusion

We made following observations:

Observation 1: UE may be out of UL coverage for two bands when it is in DL coverage for the same bands and using DMRS bundling one of the bands for PUCCH helps to extend the UL coverage

Observation 2: When UE is configured for UL CA, DMRS bundling helps to extend the UL coverage and improve the link reliability

Observation 3: Extending UL CA coverage to case B or C would need discussions on requirements in other sections than transmit signal quality.

Observation 4: UE may have challenges in  maintaining phase continuity when it is configured for CA that do not exist in single band case

And following proposals:

Proposal 1: Define inter-band UL CA requirements for DMRS bundling.

Proposal 2: Define the capability for DMRS bundling per band per band combination.

Proposal 3: Inform RAN1 that the DMRS bundling capabilities should be per band per band combination

# References

[1]	R4-2100014, ”LS on PUCCH and PUSCH repetition”, RAN1, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[2]	R4-2103393, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[3]	R4-2103363, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[4]	R4-2105417, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#98-Bis-e

[5]	R4-2105418, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition” Huawei, RAN4#98-Bis-e

[6]	R4-2107611, “Reply LS on PUCCH and PUSCH repetition”, RAN1, RAN4#99e

[7]	R4-2107880, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#99e

[8]	R4-2107881, “WF on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, RAN4#99e

[9]	R4-2111194, “Reply LS to RAN1 latest question on phase discontinuity”, Ericsson, RAN4#99e

[10]	R4-2111706, “LS on joint channel estimation for PUSCH and PUCCH”, RAN1, RAN4#100-e, Online, Aug 2021

[11] 	R4-2114991, “Reply LS on PUCCH and PUSCH transmissions”, Qualcomm, RAN4#100-e, Online, Aug 2021

[12]	R4-2114992, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, HiSilicon, RAN4#100-e, Online, Aug 2021

[13]	R4-2117006, “Reply LS on PUCCH and PUSCH repetition”. RAN1, RAN4#101e, Nov 2021

[14]	R4-2120002, ”Reply LS on PUCCH and PUSCH transmissions”, RAN4 (Qualcomm Incorporated), 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[15]	R4-2120003, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[16]	R4-2119494, “UE requirement for phase continuity”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[17] R4-2200339, “draft CR for EVM based requirements”, Qualcomm, RAN4#101Bis-e, Jan, 2022

[18]	R4-2119495, “Simulation results and criterias for setting requriements for UE”, Qualcomm Incorporated, RAN4#101e, Nov 2021

[19]	R4-2202418, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[20]	R4-2202368, “Reply LS on Maximum duration for DMRS bundling” RAN4(Qualcomm), 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[21]	R4-2200020, “Updated RAN4 RF work plan for NR coverage enhancements WI”, China Telecom, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[22]	R4-2200338, “Discussion on UE requirement for JCE”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #101Bis-e, Jan 2022

[23]	R4-2206537, “Reply LS on Maximum duration for DMRS bundling”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[24]	R4-2206538, “CR on UE RF requirements for DMRS bundling in TS 38.101-1”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[25]	R4-2206539, “CR on UE RF requirements for DMRS bundling in TS 38.101-2”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[26]	R4-2206593, “WF on issues for maintenance of NR coverage enhancements”, Ericsson. 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[27]	R4-2206580, “Reply LS on Length of Maximum duration”, China Telecom, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[28]	R4-2206541, “CR on measurment for DMRS bundling in TS 38.101-1”, Ericsson, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[29]	R4-2206542, “CR on measurment for DMRS bundling in TS 38.101-2”, Ericsson, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[30]	R4-2206538, “CR on UE RF requirements for DMRS bundling in TS 38.101-1”, Huawei, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[31]	R4-2206539, “CR on UE RF requirements for DMRS bundling in TS 38.101-2”, Huawei, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[32]	RP-220931, “Status report for WI NR coverage enhancements; rapporteur: China Telecom”, RAN1, RAN#95e, March 2022

[33]	R4-2207659, “CR 38.101-1 DMRS for CA”, Qualcomm, RAN4#103e, May 2022

[34]	R4-2207658, Discussion on additional requirements for DMRS bundling, Qualcomm, RAN4#103e, May 2022

# Annex: LS to Ran1 and Ran2 about the capabilities

3GPP TSG-RAN WG4 Meeting # 103-e 		R4-220xx

Online Meeting, May 2022

Title:	Reply LS on Maximum duration for DMRS bundling

Response to:	R1-2200773, (R4-2203504) “LS on DMRS bundling for PUSCH and PUCCH”, RAN1

Release:	Rel-17

Work Item:	NR_cov_enh

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact person:	Ville Vintola

vvintola@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

Ran4 has further discussed the CA with DMRS bundling and came to the conclusion that due to UE implementation issues, the DMRS bundling capability and maximum duration capability should per band and per band combination

# 2	Actions

To RAN1 and RAN2

ACTION: RAN4 respectfully asks RAN1 and RAN2 to take the above information in to consideration in their work.

# 3	Dates of next TSG RAN WG 4 meetings

3GPP RAN 4 RAN4#104 		2022-08-22	2022-08-26	Toulouse, France

3GPP RAN 4 RAN4#104-bis-e	2022-10-10	2022-10-19	Electronic meeting